3GPP TSG-RAN WG4 Meeting #105                                                      R4-2219485

# Toulouse, France, November 14 – November 18, 2022

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for NTN SAN PUSCH demodulation

Agenda item: 	6.2.6.2.1

Document for:	Information

# Introduction

The NTN WI is presented in [1], where the following RAN4 objectives are defined:

### 4.1.4 RAN4 Study the framework how NTN core requirements are defined. Specify the following requirements [RAN4] (Note 1) - UE RRM core requirements - Study and identify which bands may be potentially relevant to NTN including: - Analysis of regulations in the spectrum considered - Adjacent channel co-existence - Considering the potential bands to be used as example for the WID: - Specify needed generic RF core requirements for the network and the UE such that adjacent channel co-existence scenarios are met and performance of other RF parameters (RX performance, TX signal quality etc.) are subject to acceptable minimum requirements - Investigate and specify UE timing & frequency pre compensation accuracy requirements as needed [RAN4]. Note 1: It is assumed that this work item will be frequency agnostic and therefore we can consider that NTN can operate in FR1 or FR2 ranges. Defining NR bands for NTN should be included as part of dedicated Rel-17 RAN4 led work items including an analysis of regulations in spectrum considered, which bands 3GPP should specify, as well as potential co-existence between NR terrestrial and satellite Note 2: The spectrum usage on the service link for HAPS might be a different spectrum allocation than for Satellite.

In this contribution we present simulation results for PUSCH demodulation requirements for NTN.

# Discussion

We discuss open issues related to the SAN demodulation requirements in our accompanying paper [2].

# Simulation parameters

Simulation parameters, taken from agreed way forward [3], for demodulation requirements are shown in Table 1.

Table 1 Simulation parameters for PUSCH demodulation requirements

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled, Enabled |
| Channel bandwidth |  | 15kHz SCS: 5MHz 30kHz SCS: 10MHz |
| MCS |  | 4 |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 group hopping and sequence hopping are disabled for transform precoding enabled |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Number of TX antennas |  | 1 |
| Number of Rx antennas |  | 1, 2 |
| Propagation conditions and correlation matrix |  | NTN-TDLA100-200 Low, NTN-TDLC5-200 Low (K_factor = 8.05) |
| Test metric |  | SNR@70%maximum throughput |

# Simulation results

Simulation results for PUSCH demodulation requirements are shown in the following Figures 1-4.

Normalized throughput is provided for all considered configurations shown in Table 1. The results consider 0.1ppm (200Hz) residual frequency error, as well as both proposed SAN configuration (1 Rx and 2 Rx) and PUSCH mapping types in NTN-TDL-A and C.

Table 2.1-2 Ideal simulation results for PUSCH CP-OFDM (Transform precoding disabled)

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NTNTDL-A100 |  | NTNTDL-C5 |  |
|  |  |  | TypeA | TypeB | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | -0.97 | -0.95 | -1.3 | -1.28 |
| 2 | 15 / 5 | 1x2 | -2.72 | -2.72 | -5.08 | -5.07 |
| 3 | 30 / 10 | 1x1 | -0.04 | -0.04 | -1.53 | -1.54 |
| 4 | 30 / 10 | 1x2 | -3.75 | -3.77 | -5.28 | -5.29 |

Table 2.1-2 Ideal simulation results for PUSCH DFT-s-OFDM (Transform precoding enabled)

| Case Number | SCS (kHz) / CBW (MHz) | Antenna configuration | SNR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | NTNTDL-A100 |  | NTNTDL-A100 |  |
|  |  |  | TypeA | TypeB | TypeA | TypeB |
| 1 | 15 / 5 | 1x1 | -0.58 | -0.58 | -1.34 | -1.33 |
| 2 | 15 / 5 | 1x2 | -2.69 | -2.68 | -5.1 | -5.09 |
| 3 | 30 / 10 | 1x1 | 0.60 | 0.59 | -1.55 | -1.55 |
| 4 | 30 / 10 | 1x2 | -3.29 | -3.3 | -5.3 | -5.3 |

Observation 1: Both PUSCH mapping types A and B have almost same performance.

Observation 2: 70% of the peak throughput can be achieved with reasonable SNR especially with 2 SAN antennas.

# Conclusion

This contribution discusses aspects related to NTN SAN demodulation requirements and has the following observations:

Observation 1: Both PUSCH mapping types A and B have almost same performance.

Observation 2: 70% of the peak throughput can be achieved with reasonable SNR especially with 2 SAN antennas.

# References

- RP-222556, Revised WID: Solutions for NR to support non-terrestrial networks (NTN), Thales

- R4-2219484, Discussion on NTN demodulation requirements - general, Nokia, Nokia Shanghai Bell

- R4-2217348, WF on NTN SAN demodulation requirements, Huawei