3GPP TSG-RAN WG4 Meeting # 105-e	R4-2219504

# Toulouse, France, November 14 – November 18 ,2022

Title: 	Simulation results on PDCCH requirements

Source: 	Huawei, HiSilicon

Agenda item:       6.3.6.2.2

Document for:	Discussion

Background

As per [1], RAN4 updated simulation assumptions for PDCCH tests. In this paper we provide our updated PDCCH simulation results.

Simulation results

The summary of PDCCH simulation results are captured in Table 2-1:

Table 2-1: Summary of PDCCH simulation results

| Test number | CBW/SCS | CORESET duration | CORESET RB | Interleaver size | REG bundle size | Aggregation level | Propagation Condition | DCI format | Information Bit | Antenna configuration and correlation Matrix | SNR (dB)@70% 1%BLER |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 100/120 | 1 | 60 | 3 | 2 | 2 | TDLA30-200 | 1_0 | 40 | 1x2 Low | 2.8 |
| 2 | 100/120 | 1 | 60 | 2 | 6 | 4 | TDLA30-650 | 1_1 | 56 | 1x2 Low | -0.2 |
| 3 | 100/120 | 1 | 60 | 3 | 2 | 8 | TDLA30-200 | 1_1 | 56 | 2x2 Low | -3.5 |
| 4 | 100/120 | 2 | 60 | 3 | 2 | 16 | TDLA30-650 | 1_0 | 40 | 2x2 Low | -6.5 |
| 5 | 400/480 | 1 | 60 | 2 | 6 | 4 | TDLA10-200 | 1_1 | 56 | 1x2 Low | 0.0 |
| 6 | 400/480 | 1 | 60 | 2 | 6 | 8 | TDLA10-200 | 1_1 | 56 | 1x2 Low | -2.9 |
| 7 | 400/480 | 2 | 60 | 3 | 2 | 16 | TDLA10-200 | 1_0 | 40 | 2x2 Low | -6.6 |

Conclusion

In this paper, we updated the simulation results for PDCCH.

Reference

[1]   RAN4#104-bis-e BS_Demod_Testing Session meeting minutes