3GPP TSG-RAN WG4 Meeting # 105	R4-2219509

# Toulouse, France, November 14 – November 18, 2022

Title: 	Simulation results on FR2-2 PUCCH demodulation requirements

Source: 	Huawei, HiSilicon

Agenda item:       6.3.6.3.2

Document for:	Discussion

Background

As per [1], the discussions on open issues for FR2-2 PUCCH requirements are closed. In this paper we provide our simulation results based on agreed simulation assumptions.

Simulation results

Simulation results for PF0

Table 2-1: Simulation assumptions for FR2-2 PF0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |

Table 2-2: Simulation results for FR2-2 PF0

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | Ideal SNR(dB) | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 1 | 120 | 100 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | 5.4 | 7.4 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | -10.3 | -8.3 |
|  |  | TDLA10-650 Low | 1 | 480 | 400 | 1 | 1 | Disabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | 5.8 | 7.8 |
|  |  |  |  |  |  | 2 | 16 | Enabled | Prob(ACK miss)<1% Prob(DTX->ACK)<1% | -7.4 | -5.4 |

Simulation results for PF1

Table 2-3: Simulation assumptions for FR2-2 PF1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 and 16 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |

Table 2-4: Simulation results for FR2-2 PF1

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | Number of bits | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Hopping | Test metric | Ideal SNR(dB) | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 2 | 120 | 100 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -4.8 | -2.8 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -6.4 | -4.4 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -14.6 | -12.6 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<0.1% | -16.4 | -14.4 |
|  |  | TDLA10-650 Low | 2 | 480 | 400 | 14 | 1 | Enabled | Prob(NACK to ACK)<0.1% | -5.0 | -3.0 |
|  |  |  |  |  |  |  | 1 | Enabled | Prob(ACK miss)<1% | -6.6 | -4.6 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(NACK to ACK)<0.1% | -16.0 | -14.0 |
|  |  |  |  |  |  |  | 16 | Enabled | Prob(ACK miss)<1% | -16.2 | -14.2 |

Simulation results for PF2

Table 2-5: Simulation assumptions for FR2-2 PF2

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs-1) |
| Number of PRBs | 4 and 9 |
| Number of symbols | 1 and 2 |
| The number of UCI information bits | 4 and 22 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |

Table 2-6: Simulation results for FR2-2 PF2

| Number of TX antennas | Number of demodulation branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Test metric | Ideal SNR(dB) | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 120 | 100 | 1 | 4 | 4 | N/A | Prob(DTX to ACK)<1% and Prob(ACK miss)<1% | 1.8 | 3.8 |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER < 1% | -1.2 | 0.8 |
|  |  | TDLA10-650 Low | 480 | 400 | 1 | 4 | 4 | N/A | Prob(DTX to ACK)<1% and Prob(ACK miss)<1% | 1.5 | 3.5 |
|  |  |  |  |  | 2 | 9 | 22 | Enabled | UCI BLER < 1% | -1.3 | 0.7 |

Simulation results for PF3

Table 2-7: Simulation assumptions for FR2-2 PF3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |

Table 2-8: Simulation results for FR2-2 PF3

| Number of TX antennas | Number of demodulation branches | Modulation order | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of OFDM symbols | Number of PRB | Number of bits | Hopping | Additional DMRS configuration | Test metric | Ideal SNR(dB) | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| \1 | 2 | QPSK | TDLA30-650 Low | 120 | 100 | 14 | 1 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -2.2 | -0.2 |
|  |  |  |  |  |  | 14 | 1 | 16 | Enabled | 1+1 | Prob(UCI BLER)<1% | -2.6 | -0.6 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -0.5 | 1.5 |
|  |  |  | TDLA10-650 Low | 480 | 400 | 14 | 1 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -2.5 | -0.5 |
|  |  |  |  |  |  | 14 | 1 | 16 | Enabled | 1+1 | 14 | -3.0 | -1.0 |
|  |  |  |  |  |  | 4 | 3 | 16 | Enabled | 1+0 | Prob(UCI BLER)<1% | -0.4 | 1.6 |

Simulation results for PF4

Table 2-9: Simulation assumptions for FR2-2 PF4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hoppingstartingPRB | 0 |
| Number of PRBs | 1 and 16 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |

Table 2-10: Simulation results for FR2-2 PF4

| Number of TX antennas | Number of demodulation Branches | Propagation conditions and correlation matrix | SCS (kHz) | CBW (MHz) | Number of PRB | Additional DMRS configuration | Test metric | Ideal SNR(dB) | Impairment SNR(dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | TDLA30-650 Low | 120 | 100 | 1 | 1+0 | Prob(UCI BLER)<1% | 1.4 | 3.4 |
|  |  |  |  |  | 1 | 1+1 | Prob(UCI BLER)<1% | 1.3 | 3.3 |
|  |  |  |  |  | 16 | 1+0 | Prob(UCI BLER)<1% | -10.8 | -8.8 |
|  |  |  |  |  | 16 | 1+1 | Prob(UCI BLER)<1% | -11.1 | -9.1 |
|  |  | TDLA10-650 Low | 480 | 400 | 1 | 1+0 | Prob(UCI BLER)<1% | 2.5 | 4.5 |
|  |  |  |  |  | 1 | 1+1 | Prob(UCI BLER)<1% | 2.0 | 4.0 |
|  |  |  |  |  | 16 | 1+0 | Prob(UCI BLER)<1% | -10.6 | -8.6 |
|  |  |  |  |  | 16 | 1+1 | Prob(UCI BLER)<1% | -11.0 | -9.0 |

Conclusion

In this paper we provide our simulation results for FR2-2 PUCCH.

Reference

[1]   R4-2217373 WF on PUCCH demodulation requirements for PUCCH requirements