3GPP TSG-RAN WG4 Meeting #105	R4-2219647

# Toulouse, France, November 14th-18th, 2022

# Title:	TP for TS 38.181 - Clause 9.6 OTA transmitted signal quality

# Source:	THALES

# Type:	pCR

# Document for:	Decision

# Agenda Item:	6.2.2.3

Release	Rel-17

# Introduction

The following Satellite 5G NR (Non-Terrestrial Networks) technical documents have been approved at the 3GPP RAN-Plenary #96 (Budapest, 6th-9th of June 2022) for the Release-17 NTN satellite connectivity using FR1 S-band (n256) and FR1 L-band (n255):

- Technical Specification TS 38.108 (NR; Satellite Node radio transmission and reception);

- Technical Specification TS 38.101-5 (NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements);

- Technical Report TR 38.863 (Non-terrestrial networks (NTN) related RF and co-existence aspects);

The conformance testing in TS 38.181 is currently under work. This contribution therefore provides a Text Proposal for 9.6 OTA transmitted signal quality. For further information see TS 38.181 and agreed requirements in TS 38.108 as part of NTN Rel-17 WI.

# Text Proposal

Conformance testing is an important part of 3GPP work, and is currently under work in TS 38.181. This section is to propose a TP for Satellite Access Node (SAN) with the requirements described in TS 38.108.

>>>>>>>>>>>>>>>>>>>>>>>------- Start Text Proposal --------<<<<<<<<<<<<<<<<<<<<<

## 9.6	OTA transmitted signal quality

### 9.6.1	General

The requirements in clause 9.6 apply during the transmitter ON period.

### 9.6.2	OTA frequency error

#### 9.6.2.1	Definition and applicability

OTA frequency error is the measure of the difference between the actual SAN transmit frequency and the assigned frequency. The same source shall be used for RF frequency and data clock generation.

OTA frequency error requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 9.6.2.2	Minimum Requirement

The minimum requirement for SAN type 1-O is in TS 38.108 [x], clause 9.6.1.2.

#### 9.6.2.3	Test purpose

The test purpose is to verify that OTA frequency error is within the limit specified by the minimum requirement.

#### 9.6.2.4	Method of test

Requirement is tested together with OTA modulation quality test, as described in clause 9.6.3.

##### 9.6.2.4.1	Initial conditions

Directions to be tested: OTA coverage range reference direction ([D.35]).

#### 9.6.2.5	Test Requirements

The modulated carrier frequency of each NR carrier configured by the SAN shall be accurate to within 0.05 ppm observed over 1 ms.

### 9.6.3	OTA modulation quality

#### 9.6.3.1	Definition and applicability

OTA modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector.

OTA modulation quality requirement is defined as a directional requirement at the RIB and shall be met within the OTA coverage range.

#### 9.6.3.2	Minimum Requirement

The minimum requirement for SAN type 1-O, is in TS 38.108 [x], clause 9.6.2.2.

#### 9.6.3.3	Test purpose

The test purpose is to verify that OTA modulation quality is within the limit specified by the minimum requirement.

#### 9.6.3.4	Method of test

##### 9.6.3.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier:

-	B and T; see clause 4.9.1.

SAN RF bandwidth positions to be tested for multi-carrier and/or CA:

-	BRFBW and TRFBW in single-band operation, see clause 4.9.1;

Directions to be tested:

-	The OTA coverage range reference direction ([D.35]).

-	The OTA coverage range maximum directions ([D.36]).

Polarizations to be tested: For dual polarized systems the requirement shall be tested and met for both polarizations.

##### 9.6.3.4.2	Procedure

1)	Place the SAN at the positioner.

2)	Align the manufacturer declared coordinate system orientation ([D.2]) of the SAN with the test system.

3)	Orient the positioner (and SAN) in order that the direction to be tested aligns with the test antenna.

4)	Configure the beamforming settings of the SAN according to the direction to be tested.

5)	Set the SAN to output according to the applicable test configuration in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For SAN type 1-O declared to be capable of single carrier operation only, set the SAN to transmit a signal according to:

-	NR-FR1-TM3.1 if 64QAM is supported by SAN without power back off, or

-	NR-FR1-TM3.1 at manufacturer's declared rated output power if 64QAM is supported by SAN with power back off, and NR-FR1-TM3.2 at maximum power, or

- 	NR-FR1-TM3.2 if highest modulation order supported by SAN is 16QAM, or

-	NR-FR1-TM3.3 if highest modulation order supported by SAN is QPSK.

For SAN type 1-O declared to be capable of multi-carrier and/or CA operation, set the SAN to transmit according to the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test models on all carriers configured:

-	NR-FR1-TM3.1 if 64QAM is supported by SAN without power back off, or

-	NR-FR1-TM3.1 at manufacturer's declared rated output power if 64QAM is supported by SAN with power back off, and NR-FR1-TM3.2 at maximum power, or

-	NR-FR1-TM3.2 if highest modulation order supported by SAN is 16QAM, or

-	NR-FR1-TM3.3 if highest modulation order supported by SAN is QPSK.

For NR-FR1-TM 3.1, power back-off shall be applied if it is declared.

6)	For each carrier, measure the EVM and frequency error as defined in [annex H].

#### 9.6.3.5	Test requirements

##### 9.6.3.5.1	SAN type 1-O

For SAN type 1-O, the EVM of each NR carrier for different modulation schemes on PDSCH shall be less than the limits in table 9.6.3.5.1-1.

Table 9.6.3.5.1-1: EVM requirements for SAN type 1-O

| Modulation scheme for PDSCH | Required EVM |
| --- | --- |
| QPSK | 17.5 % |
| 16QAM | 12.5 % |
| 64QAM (NOTE) | 8 % |
| NOTE: EVM requirement for 64QAM is optional. |  |

EVM shall be evaluated for each NR carrier over all allocated resource blocks and downlink slots. Different modulation schemes listed in table 9.6.3.5.1-1 shall be considered for rank 1.

For NR, for all bandwidths, the EVM measurement shall be performed for each NR carrier over all allocated resource blocks and downlink slots within 10 ms measurement periods. The boundaries of the EVM measurement periods need not be aligned with radio frame boundaries.

Table 9.6.3.5.1-2, 9.6.3.5.1-3, 9.6.3.5.1-4 below specify the EVM window length (W) for normal CP for SAN type 1-O.

Table 9.6.3.5.1-2: EVM window length for normal CP, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Table 9.6.3.5.1-3: EVM window length for normal CP, FR1, 30 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length for symbols 1-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 256 | 18 | 8 | 40 |
| 10 | 512 | 36 | 14 | 40 |
| 15 | 768 | 54 | 22 | 40 |
| 20 | 1024 | 72 | 28 | 40 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 through 13. Symbol 0 has a longer CP and therefore a lower percentage. |  |  |  |  |  |

Table 9.6.3.5.1-4: EVM window length for normal CP for NR, FR1, 60 kHz SCS

| Channel bandwidth (MHz) | FFT size | CP length in FFT samples | EVM window length W | Ratio of W to total CP length (Note) (%) |
| --- | --- | --- | --- | --- |
| 10 | 256 | 18 | 8 | 40 |
| 15 | 384 | 27 | 11 | 40 |
| 20 | 512 | 36 | 14 | 40 |
| NOTE: These percentages are informative and apply to all OFDM symbols within subframe except for symbol 0 of slot 0 and slot 2. Symbol 0 of slot 0 and slot 2 may have a longer CP and therefore a lower percentage. |  |  |  |  |

>>>>>>>>>>>>>>>>>>>>>>>------- End Text Proposal --------<<<<<<<<<<<<<<<<<<<<<

# Conclusions

TP for TS 38.181, Clause 9.6 OTA transmitted signal quality.

# References

- TS 38.108, v17.2.0, Rel-17

- TS 38.181, v0.2.0, Rel-17

-END-