3GPP TSG-RAN WG4 Meeting #105-e	R4-2219661

# Toulouse, 14th – 18th Nov, 2022

Title: 	Discussion on LTE NTN IOT demod

Source: 	Huawei, HiSilicon

Agenda item:	9.5.8

Document for:	Discussion

Background

During RAN#96 meeting, revised WID [1] on NB-IoT/eMTC core & performance requirements for NTN was approved. In this contribution, we share our views about NB-IoT/eMTC UE demodulation performance requirements for NTN.

Discussion

Test scope

(N)PDSCH

In Rel-17, only time and frequency synchronization enhancements and timing relationships enhancements are considered based on RAN1 design. From our understanding, there is no any enhancement for LTE NTN IOT from the demodulation point of view. Therefore, we propose to only define limited and basic cases for LTE NTN IOT performance requirements.

Only define limited and basic cases for LTE NTN IOT (N)PDSCH performance requirements

(N/M)PDCCH

From our understanding, (N/M)PDCCH performance can be ensured if UE has passed the (N)PDSCH cases. To reduce the test effort, we propose to not introduce (N/M)PDCCH requirements for LTE NTN IOT.

Do not introduce (N/M)PDCCH requirements for LTE NTN IOT.

(N)PBCH

As we all known, (N)PBCH is untestable from the perspective of testing. Therefore, we propose that do not introduce (N)PBCH requirements for LTE NTN IOT.

Do not introduce (N)PBCH requirements for LTE NTN IOT.

CSI reporting

For CSI reporting requirements, considering the large Satellite-UE RTT delay, the reporting CSI may be outdated. There is order of tens of milliseconds delay for LEO-600 and LEO-1200 respectively at the same time there is order of hundreds of milliseconds delay for GEO. All the scenarios seem not acceptable, therefore, we propose to not consider any CSI reporting requirements for LTE NTN IOT.

Do not consider any CSI reporting requirements for LTE NTN IOT.

Test parameters

Scenario

In Rel-17 NR NTN WI, only LEO-600 scenario is considered based on the link budget analysis. So we propose to use the same scenario for LTE NTN IOT performance requirements definition.

Only consider LEO-600 scenario for LTE NTN IOT performance requirements definition.

Frequency/Timing

Considering that GNSS is mandatory for LTE NTN IOT UE, the following agreements from NR NTN WI can be reused:

- Do not consider Doppler shift for feeder link for DL

- Do not verify the UE compensation prior to the baseband processing. The maximum Doppler shift is residual frequency offset, i.e., 0.1ppm.

- Do not consider Doppler shift for feeder link for DL

- Do not consider the frequency Drift

- Do not define sampling offset model

Reuse following agreements from NR NTN WI:

- Do not consider Doppler shift for feeder link for DL

- Do not verify the UE compensation prior to the baseband processing. The maximum Doppler shift is residual frequency offset, i.e., 0.1ppm.

- Do not consider Doppler shift for feeder link for DL

- Do not consider the frequency Drift

- Do not define sampling offset model

Propagation condition

In Rel-17 NR NTN WI, two new propagation condition named NTN-TDLA100-200 and NTN-TDLC5-200 for NTN scenario are defined. So we propose to use the same propagation condition for LTE NTN IOT performance requirements definition.

Consider NTN-TDLA100-200 and NTN-TDLC5-200 for LTE NTN IOT performance requirements definition.

Antenna configuration

Since this WI are applicable for NB1/NB2 and Cat-M1 UE categories which only has single antenna, we propose to only define 1Rx requirements for LTE NTN IOT.

Only define 1Rx requirements for LTE NTN IOT.

Deployment

Based on RAN1 discussion, standalone deployment for NB-IoT for support in Rel-17 timeframe will be prioritized. So we propose to only consider standalone deployment for NB-IOT cases.

Only consider standalone deployment for NB-IOT cases.

Other test parameters

For other test parameters, the parameters from LTE IOT cases can be reused.

Summary

Based on above discussion, we propose to use following test parameters for LTE NTN IOT requirements definition.

- NB-IOT: Test 1 and Test 2 in sub-clause 8.12.1.1.2 in TS 36.101 with modification of the propagation condition.

- eMTC: Test 1 in sub-clause 8.11.1.1.1.1, Test 1 in 8.11.1.1.2.1, Test 1 and Test 2 in sub-clause 8.11.1.1.3.1 with modification of the propagation condition.

Use following test parameters for LTE NTN IOT requirements definition:

- NB-IOT: Test 1 and Test 2 in sub-clause 8.12.1.1.2 in TS 36.101 with modification of the propagation condition.

- eMTC: Test 1 in sub-clause 8.11.1.1.1.1, Test 1 in 8.11.1.1.2.1, Test 1 and Test 2 in sub-clause 8.11.1.1.3.1 with modification of the propagation condition.

Proposals

In this contribution, we discuss on NB-IoT/eMTC UE demodulation performance requirements for NTN. Our observations and proposals are:

- Only define limited and basic cases for LTE NTN IOT (N)PDSCH performance requirements

Do not introduce (N/M)PDCCH requirements for LTE NTN IOT.

Do not introduce (N)PBCH requirements for LTE NTN IOT.

Do not consider any CSI reporting requirements for LTE NTN IOT.

Only consider LEO-600 scenario for LTE NTN IOT performance requirements definition.

Reuse following agreements from NR NTN WI:

- Do not consider Doppler shift for feeder link for DL

- Do not verify the UE compensation prior to the baseband processing. The maximum Doppler shift is residual frequency offset, i.e., 0.1ppm.

- Do not consider Doppler shift for feeder link for DL

- Do not consider the frequency Drift

- Do not define sampling offset model

Consider NTN-TDLA100-200 and NTN-TDLC5-200 for LTE NTN IOT performance requirements definition.

Only define 1Rx requirements for LTE NTN IOT.

Only consider standalone deployment for NB-IOT cases.

Use following test parameters for LTE NTN IOT requirements definition:

- NB-IOT: Test 1 and Test 2 in sub-clause 8.12.1.1.2 in TS 36.101 with modification of the propagation condition.

- eMTC: Test 1 in sub-clause 8.11.1.1.1.1, Test 1 in 8.11.1.1.2.1, Test 1 and Test 2 in sub-clause 8.11.1.1.3.1 with modification of the propagation condition.

Reference

RP-221556, Revised WID on NB-IoT/eMTC core & performance requirements for NTN, RAN#96, MediaTek Inc.