# 3GPP TSG-RAN WG4 Meeting # 105					R4-2219787

# Toulouse, 14th  –18th November, 2022

# Source:		Ericsson

Title:			IoT NTN coexisting simulation initial results

Agenda item:		9.5.3

Document for:		Approval

# Introduction

In [1], the remaining simulation assumptions are stated as below:

- Number of active UL NTN UEs: 9 UEs for 15kHz, 18 and 36UEs for 3.75kHz

- ACLR/ACS modelling for Case 1 and Case 4 scenarios:

  - For Case 1: Flat model with adjusting the ACLR according to TR 36.942 and adjusting the ACLR with FACLR = 10 × LOG10(Bvictim/BAggressor), and assuming flat ACS. FACLR = ~20.5 dB with 1.5km isolation distance.

  - For Case 4: Symmetric bandwidth ACLR model according to TR 36.942 section 5.1.1.4.2 with Flat ACLR model and FACLR = 0 dB, assuming flat ACS.

- No further evaluation required for Case 3, 5

- For case 2 and case 6, companies are encouraged to bring more analysis together with simulation results.

  - Companies are encouraged to align the ACIR model and other parameters if any in offline manner before next RAN4 meeting.

  - For case 6, scaling factor shall be adopted for active TN cluster

- NB-IoT NTN UE ACLR/ACS requirements: Use the TN NB-IoT ACLR/ACS requirement as starting point and confirm with further evaluation results based on agreed simulation assumption next meeting.

  - Further finalize until next meeting on the method for mapping to UTRA. No need to map to GSM requirements.

- IoT NTN SAN ACLR/ACS: Use the NR NTN SAN ACLR/ACS requirements as starting point and confirm with further evaluation results based on agreed simulation assumption next meeting.

In this paper, we present our initial results on the IoT NTN /TN coexisting simulation.

# Discussion

## Simulation assumption

Table 3.2.2.1-1 [1] NRB configuration per BandWidth size and SCS

| NTN Technology | Configuration FR1 S-band | NRB (Subcarriers) |
| --- | --- | --- |
| NB-IoT | SCS 15 kHz | 1 (12) |
|  | SCS 3.75 kHz | 1 (48) |

Table 3.2.2.1-2 [1] Set-1 satellite parameters for co-existence study

| Satellite orbit |  |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- | --- |
| Satellite altitude |  |  | 35786 km | 1200km | 600 km |
|  |  |  | Payload characteristics for DL transmissions |  |  |  |
| Satellite EIRP density |  | 2GHz | 59 dBW/MHz | 40 dBW/MHz | 34 dBW/MHz |
| Satellite max TX power in dBm | BW (MHz) |  | 0.18 (NB-IoT) | 0.18 (NB-IoT) | 0.18 (NB-IoT) |
|  | SCS 15kHz |  | 30.55 | 32.55 | 26.55 |
|  | SCS 3.75kHz |  | 30.55 | 32.55 | 26.55 |
| Satellite Tx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |
| 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam |  |  | 0.4011 deg | 4.4127 deg | 4.4127 deg |
| ABS (Adjacent Beam Spacing) of adjacent beams from the central beam |  |  | 0.3474 deg | 3.8206 deg | 3.8206 deg |
| Satellite beam diameter |  |  | 250 km | 90 km | 50 km |
|  |  |  | Payload characteristics for UL transmissions |  |  |  |
| G/T |  | 2 GHz | 19 dB K-1 | 1.1 dB K-1 | 1.1 dB K-1 |
| Satellite Rx max Gain |  |  | 51 dBi | 30 dBi | 30 dBi |

Table 3.2.2.1-3 [1] Other parameters for NTN

| Parameters | NB-IoT NTN | Remark |
| --- | --- | --- |
| Carrier frequency | 2GHz |  |
| The number of active UE (UL) for GEO and LEO1 | 12 for 15kHz SCS 48 for 3.75kHz SCS (1 subcarrier per UE) |  |
| The number of active UE (DL) | 1 | Same with TN |
| Traffic model | Full buffer |  |
| DL power control | NO |  |
| UL power control | 36.942 section 5.1.1.6 (set 1) by bandwidth scale, target SNR at BS is 15 dB |  |
| UE TX power in dBm | -40 to 23 |  |
| NTN satellite Noise figure in dB | See Table 3.2.2.2-1 |  |
| Handover margin | N/A |  |
| Cell reselection margin | 3dB |  |

Table 3.2.2.1-4 [1] NTN satellite Noise figure in dB

| Satellite | GEO | LEO 600 |
| --- | --- | --- |
| G/T (dB K-1) | 19 | 1.1 |
| G_Rx (dBi) | 51 | 30 |
| NF (dB) | 7.4 | 4.3 |

## Simulation result

The simulated scenarios are according to WF [1] and quoted as below.

Table 3.1-2 [1]: Aggressor and victim

| No. | Combination | Aggressor | Victim | Notes |
| --- | --- | --- | --- | --- |
| 1 | TN with NTN | TN DL | NTN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. |
| 2 | TN with NTN | TN UL | NTN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. |
| 3 | TN with NTN | NTN DL | TN DL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. |
| 4 | TN with NTN | NTN UL | TN UL | Applicable for satellite operating in e.g. S-band, for e.g. coexistence with n1 FDD. |
| 5 | TN with NTN | NTN UL | TN DL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. |
| 6 | TN with NTN | TN DL | NTN UL | Applicable for satellite operating in S-band, for e.g. coexistence with n34 TDD. |

The ACIR simulation results is listed from Table 2 to Table 7, the ACIR point is picked for the 1 dB SNR loss point for either SNR loss of 5-tile curve or curve of average of the SNR loss, depending on which curve hit the SNR 1 dB loss point first.  For example, the ACIR of 24.7 is chosen coexisting ACIR point in Figure 1.  In Table 1, the ACLR and ACS from TN and NTN IoT network is assumed and in Table 2, the ACIR reference number is listed using the assumption in Table 1.  In Table 3 to Table 8.

Figure 1: ACIR for GEO IoT UL victim and TN Urban UL AAS aggressor

Table 1: ACLR and ACS assumption

|  |  | TN | TN NB-IOT | NTN |
| --- | --- | --- | --- | --- |
| BS | ACLR | 45 dB | 40 dB | 14 |
|  | ACS | 46 dB | 46 dB | 38 |
| UE | ACLR | 30dB | 37 | 20 |
|  | ACS | 33 | 28 | 30 |

Table 2: ACIR reference number for NTN and TN coexisting

|  | TN BS ACLR (45 dB) | TN BS ACS (46 dB) | TN UE ACLR (30 dB) | TN UE ACS (33 dB) | TN NB-IoT ACLR (20 dB) | TN NB-IoT ACS (28 dB) | TN NB-IoT BS ACLR (40 dB) | TN NB-IoT BS ACS (46 dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NTN IoT BS ACLR( 14 dB) | X | 14,00 | X | 13,95 | X | 13,83 | X | 14,00 |
| NTN IoT BS ACS (38 dB) | 37,21 | X | 29,36 | X | 19,93 | X | 35,88 | X |
| NTN IoT UE ACLR (20 dB) | X | 19,99 | X | 19,79 | X | 19,36 | X | 19,99 |
| NTN IoT ACS (30 dB) | 29,86 | X | 26,99 | X | 19,59 | X | 29,59 | X |

Table 3 and Table 4 list the ACIR for the NTN IoT UL and TN UL coexisting corresponding to scenario 2 and 4. It can be observed the ACIR is bigger than the reference value but with isolation distance, the ACIR will be below the reference number.

Table 5 lists the ACIR for NTN IoT DL to TN DL coexisting and it shows without the isolation distance, the ACIR will be too high and exceeding the ACIR reference number. With isolation distance, the ACIR will be lower than ACIR reference number.

Table 6 lists the ACIR for NTN IoT UL as aggressor and TN DL as victim coexisting for scenario 5 and it does not see issue for coexisting.

Table 7 lists the ACIR for TN DL as aggressor and NTN IoT UL as victim coexisting for scenario 6 and the 2 cases of the NTN beam direction is simulated, one case (case 1)  is NTN satellite beam bore sight centre is at Nadir (0,0) and the other (case 2) is with 45 degree. This ACIR is dependent on the area scale factor or TN and NTN overlapping area which deciding how many TN BS will be seen by the NTN satellite. For LEO600, the scaling factor is 1.7 and this is because of the LEO600 and TN rural coverage is similar. For GEO and TN Rural, the scaling factor is 40 and 40 TN Rural is covered by GEO. For GEO and TN Urban, the scaling factor is 4000 and 4000 TN Urban is covered by GEO. From the discussion of the coexisting study from NTN, it shows that ACIR in this case will go higher and finally it seems 10% of TN Urban is covered by GEO is a reasonable assumption and ACIR will be lowered in this case.

Table 3: NTN IoT UL and TN UL coexisting for 12 NTN IoTs (15 kHz)

| No. | Aggressor | Victim | ACIR (without isolation distance) | ACIR (with isolation distance) | ACIR reference |
| --- | --- | --- | --- | --- | --- |
| 2 | TN URBAN UL AAS | GEO IoT UL | 25.4 |  | 37.21 |
| 2 | TN IoT URBAN UL | GEO IoT UL | 30.4 |  | 19.93 |
| 4 | GEO IoT UL | TN URBAN UL AAS Victim Boarder | 39 | 14 | 19.99 |
| 4 | GEO IoT UL | TN IoT URBAN UL Victim Boarder | 26.7 |  | 19.99 |
| Note: “Victim Boarder” for TN means looking at only one site at TN boarder close to NTN UEs (worst peak case). Statistic on the whole cluster would give lower ACIRs. |  |  |  |  |  |

Table 4:NTN IoT UL and TN UL coexisting for 48 NTN IoTs (3.75 kHz)

| No. | Aggressor | Victim | ACIR (without isolation distance) | ACIR (with isolation distance) | ACIR reference |
| --- | --- | --- | --- | --- | --- |
| 2 | TN URBAN UL AAS | GEO IoT UL | 27 |  | 37.21 |
| 2 | TN IoT URBAN UL | GEO IoT UL | 32.7 |  | 19.93 |
| 4 | GEO IoT UL | TN URBAN UL AAS Victim Boarder | 45 | 19.8 | 19.99 |
| 4 | GEO IoT UL | TN IoT URBAN UL Victim Boarder | 30.7 |  | 19.99 |

Table 5:NTN IoT DL and TN DL coexisting

| No. | Aggressor | Victim | ACIR (without isolation distance) | ACIR (with isolation distance) | ACIR reference |
| --- | --- | --- | --- | --- | --- |
| 1 | TN URBAN DL AAS | GEO IoT DL | 31 | 7.1 | 29.86 |
| 1 | TN IoT URBAN DL | GEO IoT DL | 52 | 15.5 | 29.86 |
| 3 | LEO600 IoT DL | TN RURAL DL AAS | 24.2 |  | 19.79 |
| 3 | GEO IoT DL | TN RURAL DL AAS | 13.2 |  | 19.79 |

Table 6:NTN IoT UL and TN DL coexisting

| No. | Aggressor | Victim | ACIR (without isolation distance) | ACIR (with isolation distance) | ACIR reference |
| --- | --- | --- | --- | --- | --- |
| 5 | GEO IoT UL | TN RURAL DL AAS | 31 | 0 | 19.79 |

Table 7: NTN IoT UL and TN DL coexisting

| No. | Aggressor | Victim | ACIR | ACIR Reference | Comments |
| --- | --- | --- | --- | --- | --- |
| 6 | TN RURAL DL | LEO600 IoT Case 1: Central beam center is considered at nadir point | 0 | 37.21 | the LEO600 and TN Rural is not so big difference in coverage , scale factor is 1.7 |
| 6 | TN RURAL DL | LEO600 IoT Case 2: 45° for LEO | 8.1 | 37.21 |  |
| 6 | TN RURAL DL | GEO IoT UL Case 1: Central beam center is considered at nadir point | 9.9 | 37.21 | 40 TN Rural can fit in a GEO coverage |
| 6 | TN RURAL DL | GEO IoT UL Case 2: 45° for LEO | 21.2 | 37.21 |  |
| 6 | TN URBAN DL | GEO IoT UL Case 2: 45° for LEO | 30 | 37.21 | 4000 TN Urban fits in GEO coverage, assuming 10% of the GEO coverage is covered by Urban |

From the coexisting simulation results, it shows reusing the ACLR and ACS from NB-IoT for NTN IoT is fine.

- Proposal: Use the legacy NB-IoT ACLR and BS ACS in IoT NTN specification.

# Conclusions

In this contribution, we present our initial simulation results on the coeixsiting for IoT NTN with below observations and proposal:

Observation 1 From the coexisting simulation results, it shows reusing the ACLR and ACS from NB-IoT for NTN IoT is fine.

- Proposal: Use the legacy NB-IoT ACLR and BS ACS in IoT NTN specification.

# References

- R4-2217473,WF on coexistence study for IoT over NTN,	ZTE

- TR 38.863