3GPP TSG-RAN WG4 Meeting #105	R4-2219901

Toulouse, France, November 14 – November 18, 2022

Agenda Item:	8.7.3.2

Source:	MediaTek Inc.

Title:	On beam type for RRC_INACTIVE and initial access

Document for:	Discussion

# Introduction

One of objectives in the Rel-18 WID on FR2 enhancement is to specify beam correspondence requirements for RRC_INACTIVE and initial access [1]:

Beam correspondence requirements for RRC_INACTIVE and initial access - Specify UE beam correspondence requirements for initial access and RRC_INACTIVE state, for SSB-based beam correspondence without UL beam sweeping [RAN4 RF] - For RRC_INACTIVE specify at least requirements for Random Access SDT and Configured Grant SDT - Requirements for other transmission within RRC_INACTIVE state are not precluded. - For initial access, specify requirements and verification of beam correspondence requirements based on msg1 spherical coverage (at least) - Study the potential impact on testability aspects (i.e., test time).

And the WF agreed in RAN4#104-bis-e list out issues still open up to now, including beam types, beam correspondence requirements and test issues [3][4]. In this contribution, we focus on open issues on beam type, and share our understanding and views on these issues.

# Discussion

For the beam type, RAN4#104-bis-e has achieved the following agreements:

<Way forward/Agreement>: It is encouraged that companies contribute in RAN4#105 about their understanding of how the beam refinement is done in RRC_INACTIVE and initial access. Possible options (but not limited to) are listed in the following. - Option 1: Beam refinement in RRC_INACTIVE and initial access is made in the same way as RRC_CONNECTED. - Option 1a: with the same SSB configuration as Rel-16 SSB BC (RRC_CONNECTED) case. - Option 2a: with some modification in SSB configuration. - Option 2: It is allowed not to refine beams in RRC_INACTIVE and initial access. - Option 2a: It is allowed to use only one antenna element. - Option 2b: Beam gain is 7 dB lower than RRC_CONNECTED. - Option 3: Somewhere in the middle. - Option 3a: Refinement is done but is not as good as RRC_CONNECTED. - Option 3b: Refinement is done in CG-SDT but is not in RA-SDT and initial access. - Option 3c: Refinement in DRX is not as good as continuous reception. - Option 4: Rough beam or Fine beam used in IA is up to UE implementation and requirements should be implementation agnostic.

Literally beam refinement is not a necessity if there is beam correspondence between uplink and downlink. In RRC_CONNECTED, uplink beam refinement can be performed with the assistance of SRS, however, SRS based uplink beam refinement is not feasible for RRC_INACTIVE or initial access, and of course, refinement can be done, but for obvious reasons, it can not be as good as RRC_CONNECTED. In fact, even whether or not to use wide beam or narrow beam can be up to UE’s implementation. Therefore, in our understanding, refinement is done but not as good as RRC_CONNECTED, i.e., Option 3a. And if Option 3a cannot be agreed, then Option 4 would be fine.

Proposal 1: RAN4 to take Option 3a for beam refinement in RRC_INACTIVE and initial access, and if it cannot be agreed, then leave it as UE implementation issue, i.e., Option 4.

# Conclusion

In this contribution we have the following proposal for beam type for RRC_INACTIVE and initial access:

Proposal 1: RAN4 to take Option 3a for beam refinement in RRC_INACTIVE and initial access, and if it cannot be agreed, then leave it as UE implementation issue, i.e., Option 4.

# References

- RP-222228 Revised WID: NR RF requirements enhancement for frequency range 2 (FR2), Phase 3, Nokia, Xiaomi

- R4-2214454 WF on test metric for BC in RRC_INACTIVE and initial access, Nokia

- R4-2217727 WF on BC in RRC_INACTIVE and initial access, Nokia

- R4-2217782 Email discussion summary for [104-bis-e][130] FR2_enh_req_Phase3_part1