3GPP TSG-RAN WG4 Meeting #105	     											     R4-2220229

Toulouse, France, November 14th – November 18th, 2022

Agenda item:			6.2.7

Source:	Moderator (THALES)

Title:	WF for Rel-17 SAN RF maintenance

Document for:	Agreement

# Introduction

This Way Forward considers remaining issues from R4-2220130, Email discussion summary for [105][304] NTN_Solutions_RF_Maintenance. The current list of topics/sub-topics/issues presented in R4-2220130, the Email discussion summary for [105][304] NTN_Solutions_RF_Maintenance were:

- Topic #1: System parameters

  - Sub-topic 1-1: OBUE Generalities

    - Issue 1-1-1: Multi-carrier Consideration for OBUE

    - Issue 1-1-2: Applicability of ITU-R 1541

  - Sub-topic 1-2: OBUE and Spurious Discussion

    - Issue 1-2-1: OBUE/OoBE

    - Issue 1-2-2: Applicability of SAN spurious emissions

    - Issue 1-2-3: Applicability of SAN OBUE/OoBE

    - Issue 1-2-4: Relationship & Limits between SAN spurious and SAN OBUE/OoBE

    - Issue 1-2-5: OBUE/OoBE for SAN type 1-O

    - Issue 1-2-6: ΔfOBUE Removal

  - Sub-topic 1-3: Other Requirements

    - Issue 1-3-1: Co-location scenario

    - Issue 1-3-2: Protection of the own Satellite Access Node receiver (6.6.5.2.2)

    - Issue 1-3-3: OTA Tx spurious requirement for protection of the own SAN receiver (9.7.5.2.3 TS 38.108)

    - Issue 1-3-4: Applicability of SAN 1-H requirements with respect to all TAB connectors

  - Sub-topic 1-4: Synchronization Raster

    - Issue 1-4-1: Syncronization Raster Ambiguity

# Agreements

Issue 1-2-2: Applicability of SAN spurious emissions

- Proposals

  - Option 1: SAN Spurious emissions requirements are applicable only outside respective SAN band. Define the Spurious emissions limits excluding the SAN band (and not only SAN channel BW). (R4-2219281/P1)

  - Option 2:

- Discussion

  - Ericsson: We shall align the definition of spurious emission and OBUE with ITU recommendation.

  - ZTE: We have similar comments as Ericsson. Spurious emission can be within the band.

  - Thales: The passband size of L band and S band is limited. Multi-contiguous carriers used in the channel.

  - Ligado: We share similar view as Thales. Option 1 also complied with ITU recommendation.

  - Thales: Satellite operators can share the same satellite even with same spectrum. ITU recommendation is specified based on the total assigned bandwidth.

The following issues are also considered as follows:

Issue 1-2-1: OBUE/OoBE

Issue 1-2-3: Applicability of SAN OBUE/OoBE

Issue 1-2-4: Relationship & Limits between SAN spurious and SAN OBUE/OoBE

Issue 1-2-5: OBUE/OoBE for SAN type 1-O

Issue 1-2-6: ΔfOBUE Removal

Proposed WF (if we all agree to use the exact terms of SM.1541-6):

- [Remove ∆fOBUE]

- Introduce BWSAN abbreviation (Appendix 3 ITU RR, Clauses 1.13/1.15 in SM.1541-6). Adding a new symbol in TS 38.108 Clause 3.2:

“BWSAN		SAN total RF bandwidth for a given operating band.”

- Explicitly define 200% Out-of-Band region as a function of the SAN necessary bandwidth.

- Change (at least) the following phrases:

  - “Unless otherwise stated, the operating band out-of-band unwanted emission (OBUE) limits for SAN in FR1 are defined from channel SAN total RF bandwidth edge up to frequencies separated from the channel SAN total RF bandwidth edge by 200% of the SAN necessary bandwidth.”

  - “BWChannelSAN is the considered NR channel bandwidth or SAN total RF bandwidth for a given operating band.”

  - “The operating band out-of-band unwanted emission basic limits of the band where there are carriers transmitted, as defined in the tables of the present clause for the largest frequency offset (fmax), shall apply from channel SAN total RF bandwidth edge up to frequencies separated from the channel SAN total RF bandwidth edge by 200% of the SAN necessary bandwidth.”

- Clearly mention that spurious emission requirement excludes Out-of-Band emission region and SAN total RF bandwidth:

  - “spurious emission limits for FR1 shall exclude the SAN total RF bandwidth and the out-of-band domain”.

- For multi-transponder SAN a note could be added:

  - “In the case of multi-transponder SAN, the spurious emission limits should not apply to those emissions of a satellite that fall within either the necessary bandwidth or the out-of-band domain of another transponder on the same satellite [Appendix 3 ITU RR]”.

- Update the requirement table:

Table 6.6.4.2-1: SAN LEO and GEO Classes OBUEOut-of-Band domain emission basic limits

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Basic limits (dBm) | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 2× BWChannelSAN | 0.002 MHz f_offset < 2× BWChannelSAN + 0.002 MHz | maxSE limit, PSDChannel – ΔSat_ClassdB-40×log10 f_offset-0.002BWChannelSAN×2+1dBm | 4 kHz |
| NOTE 1: PSDChannel = Prated,c,sys – 10log10(BWChannelSAN) – 24, unit dBm/4kHz. NOTE 2: SE limit is spurious emission limit specified in spurious emission clause 6.6.5. NOTE 3: PSD attenuation as in ITU-R SM.1541-6 [9], Annex 5 OoB domain emission limits for space services. NOTE 4: ΔSat_ClassdB=0 dB for GEO class and ΔSat_ClassdB=3 dB for LEO class. |  |  |  |

Issue 1-3-4: Applicability of SAN 1-H requirements with respect to all TAB connectors

- Proposals

  - Option 1: (R4-2218456/P5)

  - the unwanted emission requirements are applied for sum of all TAB connectors;

  - the ACLR limits shall be applied to ACLR over all for sum of all TAB connectors

  - the operating band unwanted emissions (OBUE) limits shall be applied to the sum of the emission power over all TAB connectors

  - the sum of the spurious emissions over all TAB connectors for SAN type 1-H shall not exceed the limits defined in clause 6.6.5.2

  - Option 2:

- Discussion

  - Ericsson: Does this mean applied for each TAB connector?

  - CATT: Ericsson’ understanding is correct.

Proposed WF:

- Current testing specification is (e.g. TS 38.181 0.2.0, 6.6.3.2, 6.6.3.4 D3.1):

  - For SAN type 1-H where there may be multiple TAB connectors, they may be tested one at a time or multiple TAB connectors may be tested in parallel as shown in annex D.3.1 for SAN type 1-H. Whichever method is used the procedure is repeated until all TAB connectors necessary to demonstrate conformance have been tested.

  - In all cases the measurement is per TAB connector but the measurement may be done in parallel.

- RAN4 shall consider both SAN testing possibilities in order to facilitate 1-H testing:

  - Per each TAB connector

  - To all/multiple TAB connectors

- Please note that the requirements are not to be changed

# Annex: ITU Radio Regulations/ITU Background Information

Note 1: Appendix 3 ITU Radio Regulations explains the applicability of spurious domain emission limits to a satellite transponder/multiple transponders per single satellite:

12 For the case of a single satellite operating with more than one transponder in the same service area, and when considering the limits for spurious domain emissions as indicated in § 11 of this Appendix, spurious domain emissions from one transponder may fall on a frequency at which a second, companion transponder is transmitting. In these situations, the level of spurious domain emissions from the first transponder is well exceeded by the fundamental or out-of-band domain emissions of the second transponder. Therefore, the limits of this Appendix should not apply to those emissions of a satellite that fall within either the necessary bandwidth or the out-of-band domain of another transponder on the same satellite, in the same service area (see Fig. 2). (WRC-03)

Note 2: ITU SM.1541-6 (Appendix 2) information:

Note 3: Recommendation ITU SM.1541-6 describes:

Note 4: Recommendation ITU SM.1541-6 recognizes from RR:

Note 5: ITU SM.1541-6 (Appendix 2) information for multiple transponders:

# Annex: Online Discussion RAN4#105 Brk1 (15/11/2022)

Issue 1-1-1: Multi-carrier Consideration for OBUE

- Proposals

  - Option 1: Consider Multi-Carrier Case (R4-2219609/P3)

  - Option 2: Do not consider Multi-Carrier, consider only Single-Carrier Case.

- Agreement:

  - Consider both single-carrier and multi-carrier cases.

Issue 1-1-2: Applicability of ITU-R 1541

- Proposals

  - Option 1: Current SAN requirements should be updated based on the principle that the out of band emission specified in ITU-R 1541 should be applicable to both multi-carrier and single carrier cases. (R4-2219609/P3)

- Agreement:

  - Option 1 agreed

Issue 1-3-1: Co-location scenario

- Proposals

  - Option 1: Confirm the previous agreement that co-location scenario, nor the co-location requirement types are not applicable to SAN, both in Core and conformance testing specifications. (R4-2219967/P1)

  - Option 2:

- Recommended WF

  - TBA

Note: During RAN4#104-bis-e meeting, the following WF on the co-location requirements was agreed in R4-2217509 (WF on NTN solutions SAN RF conformance):

Issue 1-5-4: Co-location requirement Background: TS 38.108 v17.1.0 clearly says: “Co-location requirements are not applicable to SAN”. Option 1: remove Protection of the BS receiver of own” in TS 38.108 YES: Thales, Huawei NO: CATT, Ericsson (protection of own rx from own tx is an internal requirement in the first place) WF for next meeting: companies invited to present their view Agreement: -

Issue 1-3-2: Protection of the own Satellite Access Node receiver (6.6.5.2.2)

- Proposals

  - Option 1: Keep protection of the own Satellite Access Node receiver (6.6.5.2.2 TS 38.108)

  - Option 2:

- Discussion:

  - ZTE: We have agreements no co-collation requirements, but for we need to have protection of the BS receiver of own.

  - Ericsson: We share similar view as ZTE.

  - Huawei: Test set-up for co-location radiated requirements quite complicated, which not feasible for Rel-17 timeframe.

  - CATT: We agree with ZTE. For core requirement, we shall keep it and further discuss the possibility of introducing conformance test cases. We are fine to remove co-location requirements but suggests to specify protection requirements as TRP based.

  - Huawei: Co-location requirements are different compared to co-location scenarios.

Issue 1-3-3: OTA Tx spurious requirement for protection of the own SAN receiver (9.7.5.2.3 TS 38.108)

- Proposals

  - Option 1: Remove the OTA Tx spur requirement for protection of the own SAN receiver from Rel-17 specifications. (9.7.5.2.3 TS 38.108) - R4-2219967/P2

  - Option 2: Define protection of the SAN receiver of own as TRP requirement and remove all co-location related content in clause 9.7.5.2.3 Protection of the SAN receiver of own of TS 38.108. - R4-2218460/P3

- Agreement: Remove the co-location requirements from TS 38.108 Rel-17 specification. It’s not precluded further discuss co-location requirements in Rel-18.

  - Remove the OTA Tx spur requirement for protection of the own SAN receiver from Rel-17 specifications (9.7.5.2.3 TS 38.108).