3GPP TSG-RAN WG4 Meeting #105									                 R4-2220245

Toulouse FR , 14– 18 November 2022

Agenda item:			8.18.4

Source:	Qualcomm Inc

Title:	WF for the feasibility from UE aspect

Document for:	Approval

# Scope

This WF discusses UE feasibility aspects from 310

# Issue 2-1-1 TX modelling for UE-UE CLI for RAN1 and RAN4 simulation (co-channel)

This agreement was made in the 11/15 BS session

Agreement from 11/15 BS session

  - RAN4 inform RAN1 that the IBE-based model shall be used for TX modelling for UE-UE CLI for the co-channel case in RAN1 system-level simulation:

    - IBE models provided in clause 6.4.2.3 in TS38.101-1 and clause 6.4.2.3.4 in TS38.101-2 shall be followed.

    - The general and IQ Image part of in-band emission model shall be considered, while the carrier leakage part can be ignored.

# Issue 2-1-2 TX modelling for UE-UE CLI for FR2-1 (co-channel)

This agreement was made in the 11/15 BS session

Agreement from 11/15 BS session

  - RAN4 shall confirm the same approach as FR1 counterpart (i.e., IBE-based model) for FR2-1 modelling of UE TX aggressor toward co-channel victim.

# Issue 2-1-3: Receiver sub-band selectivity (co-channel)

## Issue 2-1-3.1: Configuring the UE channel bandwidth to be a sub-band for selectivity

This agreement was made in the 11/15 BS session

Agreement from 11/15 BS session

  - For legacy UE: Companies are encouraged to bring more analysis on the achievable selectivity performance considering FFT operation

    - The analysis shall be based on the assumption that there is no impact on legacy UE implementation.

  - For new SBFD capable UE, further analysis of the possibility to improve selectivity performance under the assumption that UE channel bandwidth not equal the sub-band bandwidth.

## Issue 2-1-3.2: Receiver sub-band selectivity

Various proposals related

Proposed agreement:

- For legacy UE: For receiver sub-band selectivity, no rejection/attenuation due to RF/BB filtering is assumed on interference in adjacent sub-band as legacy UEs do not operate this way.

  - Use typical model for UE selectivity value

  - The selectivity and performance of the FFT is included in RAN4 study for co-channel case

    - FFS whether the adjacent channel case requires the selectivity and performance of the FFT.

  - RAN4 should consider interferer with timing or frequency offset or both w.r.t. the desired signal for the co-channel case

    - FFS whether this applies to the adjacent channel case

- For new SBFD capable UE, further analysis of the possibility to improve selectivity performance under the assumption that UE channel bandwidth not equal the sub-band bandwidth.

- Companies come next meeting with technical proposals on the level of interference from an UL sub-band co-channel interferer to the UE DL sub-band. So far companies have proposed:

  - 33 dB at the ADC output (for FR1) based on typical performance. FFS for FR2-1

  - 25 dB (for FR1 and FR2-1)

  - 0 dB (for FR1 and FR2-1)

  - Other values not precluded for discussion next meeting.

# Issue 2-1-4: Effect of power contained in uplink sub-band on receiver model (blocker)

## Issue 2-1-4.1: AGC assumption for RX modeling (both co-channel or adjacent channel case)

Addressing Apple comment on AGC assumption

Proposed agreement:

UE receiver AGC designs may vary and companies may bring contributions based on their design approach.

## Issue 2-1-4.2: In-subband UE self-noise over the input power range (co-channel case)

How to model the receiver self-noise

9dB FR1 NF over the entire input power range was proposed. 10 dB for FR2-1.

We do not have a proposed agreement for this meeting.

# Issue 2-1-5: Clarification/summary on RX modelling (co-channel)

Proposal into the meeting seeks to clarify how to scale the flat frequency interference with interference and victim sub-band bandwidth. Based on the discussions about ACS-type value, those are proposed as FFS.

Proposed agreement (Clarification on co-channel RX model):

For FR1: Pinterference_co-channel_FR1 = Pinterferer – (X dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

- X value is FFS

For FR2-1: Pinterference_co-channel_FR2-1 = Pinterferer – (Y dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

- Y value is FFS

# Issue 2-2-1: Maximum input level in RX modelling for FR2-1 (adjacent channel)

How to handle maximum input level for the adjacent channel interferer case

Proposed agreement:

For FR2-1, the -25dBm maximum input level can still be used for adjacent channel model at UE Rx side.

# Issue 2-2-2: Clarification/summary on RX modelling (adjacent channel)

Clarify how to scale the frequency flat interference with interferer and victim bandwidths. Uses the already agreed 33/23 dB values.

Proposed agreement:

- For FR1: Pinterference_adjacent_channel_FR1 = Pinterferer – (33 dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

- For FR2-1: Pinterference_adjacent_channel_FR2-1 = Pinterferer – (23 dB + 10*log10(max(1,BWinterference /BWvictim_subband)))

# Issue 2-2-3: Remaining part of TX modelling (adjacent channel)

Clarify that the agreed FR2  30 dB came from 38.101-1 for 4GHz

Proposed agreement:

  - 30 dB is the total distortion power in the adjacent channel on each side of SBFD carrier. The ACLR1 distortion PSD is modeled as flat over that range.

  - FFS whether RAN4 need to model allocations that are less than fully allocated uplink sub-bands.

30 dB was chosen based on the 38.101-1 NR ACLR for 4 GHz simulation frequency.

# Issue 2-3-1: Typical vs worst case UE model parameters

Proposed agreement RAN4 uses typical UE model parameters

Proposed agreement:

RAN4 to implement UE model parameters to reflect typical performance rather than worst-case.

# References

[1] – R4-2217513, WF on SBFD feasibility study and RF impact: UE aspects

[2] – R4-2220136