3GPP TSG-RAN WG4 Meeting #105	R4-2220280

Toulouse, France, November 14 – 18, 2022

Source: 	Huawei

Title: 	WF on MU budget calculations for FR2-2 BS RF

Agenda Item:	6.3.3.1

Document for:	Approval

# Introduction

During RAN4#105 meeting, contributions in [1-22] were submitted for the topics [306] on the BS RF conformance for the NR extension operation to 71 GHz.

The topic’s summary was provided in [10]. Based on online and offline discussion, the following WF was formulated.

# Way Forward

The following WF is provided for Approval.

# Agreement from online sessions

- Sub-topic 1-1 Consideration of the composite test equipment and additional calibration

Consider both approaches case by case (per requirement) to verify when the power meter is usable and focused on final MU introduction for each requirement.

- Sub-topic 2-1 Consideration of the OTA chambers for the FR2-2 OTA test procedures

Further work on MU budget for both CATR and RC.

Note:	Other chambers not precluded with contribution driven in future RAN4 meeting.

- Sub-topic 4-1: Occupied bandwidth test requirement

Option 1 agreed, i.e. Span for OBW measurement shall be twice of channel bandwidth for FR2-2.

- Issue 4-1-2: Min number of OBW measurement points for 100 MHz and 400 MHz CHBW

Option 1 agreed, i.e. Minimum number of measurement points for OBW measurement shall be 2×BWChannel200 kHz  for 100 MHz and 400 MHz channel bandwidth for FR2-2

- Issue 4-1-3: Min number of OBW measurement points for 800 MHz, 1600 MHz, and 2000 MHz CHBW

Option 1 agreed, i.e. Minimum number of measurement points for OBW measurement shall be 2×BWChannel800 kHz  for 800 MHz, 1600 MHz, and 2000 MHz channel bandwidth for FR2-2

- Issue 4-2-1: EVM test requirement

Option 1, i.e. increase of EVM MU for FR2-2 is 0.1% to propose [1.1%]

- Sub-topic 4-1: Spurious emission within 380 MHz – 26 GHz

Option 1 agreed, i.e. For MU relevant for spurious emission within 380 MHz – 26 GHz measured in RC, re-use MU evaluation for FR2-1 also for FR2-2.

- Sub-topic 4-2: Consideration of the frequency sub-ranges for MU

For spurious measurement:110 GHz as breaking point.

# Open issues for TE equipment

The following open issues were identified for further discussion next meeting. Interested companies are encouraged to provide related inputs next meeting:

- Consider division of the TE MU for in-band and out-of-band categories with different MU values:

  - In-band: approach without mixers as baseline,

  - out-of-band: approach with mixers as baseline,

- Consider adding separate MU contribution for power meter and for spectrum analyser.

- For EIS and EIRP requirements with sufficient link budget add an additional calibration to improve absolute power MU. For other requirements the additional calibration is not applicable, since the signal will be too weak for the power meter. The calibration needs to be captured in TR 37.941 for specific requirements.

- Consideration of the LNA for high power measurements to be further analyzed. Further motivation to be provided.

NOTE:	MU contributors for the above LNA are to be kept in [] in the MU budget Excel spreadsheets.

# OBW MU

With measurement points resolution increased to 800 kHz, OBW MU to be increased to [2400 kHz] for those CHBW.

# MU values for TE equipment

Based on inputs submitted in [1, 3-9], the following ranges are provisionally agreed for the Test Equipment MU values and will be further discussed to arrive the final MU values:

Table 1: TE MU values for in-band measurements

| UID | Uncertainty source | Standard uncertainty values, 52.6 < f ≤ 71 GHz [dB] |
| --- | --- | --- |
| C1-1 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - high power (EIRP, TRP) | [0.3 - 2.36] |
| C1-7 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM, absolute ACLR) | [0.7 - 2.36] |
| C1-8 | RF power measurement equipment (e.g. spectrum analyzer, power meter) - relative (ACLR) | [0.7 - 2.36] |
| C1-9 | RF power measurement equipment standard uncertainty σ (dB) of the absolute level for a time domain wideband measurement for FR2 | [1.65 - 2.61] |
| C1-2 | RF signal generator | [0.3 - 2.37] |
| C1-3 | Uncertainty of the network analyzer | [0.4 - 0.87] |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.30 |

Table 2: TE MU values for out-of-band measurements

| UID | Uncertainty source | Standard uncertainty values, 71 < f ≤ 142 GHz [dB] |
| --- | --- | --- |
| C1-7 | Uncertainty of the RF power measurement equipment (e.g. spectrum analyzer, power meter) - low power (UEM, absolute ACLR) | [0.6 -2.49] |
| C1-3 | Uncertainty of the network analyzer | [0.3 - 1] |
| C1-4 | Uncertainty of the absolute gain of the reference antenna | 0.30 |

NOTE: the above out-of-band frequency range may be further sub-divided, depending on the conclusions on the final MU values for the OoB range.

# MU budget Excel spreadsheets

Tx and Rx Excel spreadsheets based on TR 37.941 were updated for the CATR and Reverberation chamber MU contributors. Related requirement specific MU calculations were updated accordingly.

[Updated Tx and Rx Excel spreadsheets to be attached to the final version of the WF].

The attached Excel spreadsheets to be used as baseline for further updates for TE MU, with the goal to capture the final expanded MU values for FR2-2.

# Draft CRs handling to TS 38.141-2

Draft CRs to TS 38.141-2 in [11-13, 16-18, 21, 22] to be revised with the goal to be Endorsed during this meeting, and further captured in the big Draft CR.

# Draft CRs handling to TR 37.941

Draft CRs to TR 37.941 in [14, 15, 19, 20] to be Noted, in order not to collide with the ongoing discussion on the MU budget.

# References

[1]	R4-2219843	FR2-2 BS conformance test consideration 	Keysight Technologies UK Ltd

[2]	R4-2218908	Discussion on OBW measurements for FR2-2	NEC

[3]	R4-2218834	On general aspects relevant for FR2-2 conformance testing	Ericsson

[4]	R4-2219449	FR2-2 Power measurement equipment MU with Mixer for BS conformance testing	Keysight Technologies UK Ltd

[5]	R4-2219450	FR2-2 Test equipment MU for BS conformance testing	Keysight Technologies UK Ltd

[6]	R4-2218321	Proposals on measurement uncertainty of BS OTA transmitter requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

[7]	R4-2218835	On aspects related to FR2-2 transmitter conformance testing	Ericsson

[8]	R4-2218322	Proposals on measurement uncertainty of BS OTA receiver requirements for extending current NR operation to 71 GHz	Nokia, Nokia Shanghai Bell

[9]	R4-2218836	On aspects related to FR2-2 receiver conformance testing	Ericsson

[10]	R4-2220132	Summary for [105][306] NR_exto71GHz_BSRF	Moderator (Huawei)

[11]	R4-2218909	CR to 38.141-2: OTA OBW requirements for FR2-2	NEC

[12]	R4-2218320	Draft CR to TS 38.141-2 in clauses 6.3 to 6.5 for extending current NR operation to 71GHz	Nokia, Nokia Shanghai Bell

[13]	R4-2218837	Draft CR to TS 38.141-2: Addition of FR2-2 aspects in clause 4	Ericsson

[14]	R4-2219451	DraftCR to 37.941: 71 GHz Extension TE MU update	Keysight Technologies UK Ltd

[15]	R4-2218833	Draft CR to TR 37.941: Addition of aspects related to EIRP measurement in CATR relevant for FR2-2 in sub-clause 7.3, 8.3, 9.2.3 and 9.2.7	Ericsson

[16]	R4-2219452	draftCR to 38.141-2: 71 GHz Extension BS EVM Clause 6.6 and Annex L (6.6, L)	Keysight Technologies UK Ltd

[17]	R4-2219970	draft CR to TS 38.141-2 on FR2-2 implementation (sections 1,2,3,6.7.1, 6.7.2, 6.7.3, 6.7.4, 7.6)	Huawei, HiSilicon

[18]	R4-2218463	Draft CR for 38.141-2, On FR2-2 BS RF conformance testings in clauses 7.1-7.5 and respective MU and TT	CATT

[19]	R4-2218464	Draft CR for 37.941, On FR2-2 RX directional requirements in clauses 10.1-10.5 and respective MU and TT summary	CATT

[20]	R4-2218838	Draft CR to TR 37.941: Addition of aspects related to EIS measurement in CATR relevant for FR2-2 in sub-clause 10.2.3 and sub-clause 10.2.7	Ericsson

[21]	R4-2219345	draft CR to TS38.141-2  Clause 7.7, 7.8	ZTE Corporation

[22]	R4-2219346	draft CR to TS38.141-2  Clause 7.9	ZTE Corporation