3GPP TSG-RAN WG4 Meeting #105		R4-2220313

Toulouse, France, November 14 – November 18, 2022

Agenda item:			4.2.8

Source:	Moderator (Qualcomm Incorporated)

Title:	WF on Rel-17 NR NTN RRM Core Requirements maintenance

Document for:	Approval

# Introduction

The WF covers the contributions submitted under the following AIs

- 6.2.4	RRM core requirement maintenance	[NR_NTN_solutions-Core]

  - 6.2.4.1	Measurement procedure requirements	[NR_NTN_solutions-Core]

* R2-2210866 Reply LS on network indication for applying enhanced cell reselection requirements

  - 6.2.4.2	Others 	[NR_NTN_solutions-Core]

# Topic #1: Open Issues

### Issue 2: Inter-operability testing (IOT) bit for inter-satellite measurement

Agreement:

- Send LS to RAN2 to consider an inter-operability testing (IOT) bit, e.g. interSatMeas-r17, to indicate whether the UE has been tested successfully with inter-satellite measurement.

### Issue 3: Cell re-selection

Agreement:

- The measurement period requirements apply provided that neighbour cell explicit list associated with each SMTC is clear to UE.

- RAN4 to make the following change to “4.2C Cell Re-selection for NR UE for Satellite Access”:

- Replace the current statement of “For intra-frequency and inter-frequency cells the serving cell may not provide explicit neighbour list but carrier frequency information and bandwidth information only” with “The requirements in this clause apply provided that the neighbour cell explicit list associated with SMTC should be configured by the serving cell”

### Issue 5-1: Rel-16 vs. Rel-17 RRM Relaxation - RAN2 LS R2-2210866

Agreement:

- On relaxed measurements for GEO, RAN4 agrees to reuse the existing R16 relaxed measurement requirement, i.e. relaxedMeasurement-r17 is not supported in Rel-17 NR NTN RRM requirement.

### Issue 5-2: GSO vs. NGSO - RAN2 LS R2-2210866

Agreement:

- For intra-frequency measurement, RAN4 to confirm that the NR NTN relaxed intra-frequency measurements only apply to GSO neighbour cells and can be configured only by GSO serving cell.

- For inter-frequency measurement, RAN4 to confirm that the NR NTN relaxed inter-frequency measurements only apply to GSO neighbour cells and can be configured by GSO and NGSO serving cell.

- (Note) The applicability of the signalling for relaxed RRM measurement for IoT NTN shall be separately discussed and decided under WI of LTE_NBeMTC_NTN.

### Issue 6: Enhanced RRM Requirements

Agreement:

- RAN4 to ask RAN2 to clarify the description on the capabilities of enhanced RRM requirements. E.g. RAN4 works under the assumption that UEs not supporting the feature shall apply NTN requirements, not legacy TN non-HST requirements.

### Issue 7: Additional delay and Requirement Applicability in HO and CHO due to uncertainty of Target cell’s epoch time

Agreement:

- Wait for RAN2 conclusion and the following options can be further discussed as part of maintenance..

  - Option 1:

- No need to consider the additional delay introduced in the HO/CHO procedure for NTN caused by the cases where the UE has to wait for the epoch time to be reached or re-acquire a new ephemeris information.

  - Option 2:

- Latency and Interruption requirements in HO and CHO shall be extended. FFS on the details.

  - FFS on delay requirement for blind HO/CHO case if target satellite ephemeris information and epoch timing is only provided to UE via HO command.

# References

[1] R4-2220050, “Topic summary for [105][204] NR_NTN_solutions_RRM_1,” 3GPP TSG-RAN WG4 Meeting #105