3GPP TSG-RAN WG4 Meeting # 105						                                 R4-2220322

Toulouse, France, November 14th – November 18th , 2022

Agenda item:			6.6.5

Source:	vivo

Title:						WF on Rel-17 NR RedCap RRM Core requirement maintenance

Document for:	Approval

Topic #1: Redcap RRM Core requirement maintenance

Sub-topic 1-1 Remaining issues for Redcap mobility requirements

Issue 1-1-1: Clarification of SDT FR1 requirements

- Proposals

  - Option 1: Apple

For UE configured with eDRX in RRC_INACTIVE state in FR1, the TA validation requirements for RedCap CG-SDT shall be revised as,

| Measurement | FR1 |
| --- | --- |
| RSRP1 | (T1 – min(640ms, M1*T)) ≤ T1’ ≤ (T1 + min(640ms, M1*T)) |
| RSRP2 | (T2 – min(640ms, M1*T)) ≤ T2’ ≤ T2 |
| Note: T is as defined in table 5.1B.2.2-1 |  |

- WF:

  - Discuss the issue directly in CR.

Issue 1-1-2: Requirements for HO to a BWP which has different SSB with the one used for measurement

- Proposals

  - Option 1: The scenario when handover is performed to a BWP which has different SSB than the one used during measurement should be considered as handover to an unknown cell.  (Apple MTK Qualcomm)

  - Option 2: Have additional delay: Additional delay is allowed if the frequency difference between NCD-SSB and CD-SSB is larger than a threshold. (ZTE)

    - Option 2a: An additional delay is allowed if some conditions are met, such as the frequency difference between NCD-SSB and CD-SSB is larger than a threshold. (vivo)

  - Option 3: Have additional delay - when handover is performed to a known cell where the SSB associated with the first active BWP is different from the SSB used during intra-frequency or inter-frequency measurements of the target cell, then Tsearch = Trs ms if the target cell is intra-frequency cell and Tsearch = 2* Trs ms if the target cell in inter-frequency cell. (Qualcomm)

  - Option 4: No additional delay is needed when UE handover to a BWP which has different SSB type with the one used for measurement. (CMCC Ericsson Huawei Nokia)

- WF: FFS: One additional sample is needed for known intra-frequency and inter-frequency handover

Issue 1-1-3: Known condition for RedCap Handover

- Proposals

  - P1 (Ericsson): In RedCap, the known cell condition should be updated as follow.

In FR2, the target cell is known if it has been meeting the following conditions:

-	At least one of the SSBs measured has the same PCI as the NR target cell,

-	During the last 5 seconds before the reception of the handover command:

-	the UE has sent a valid measurement report for the target cell and

-	One of the SSBs measured which has the same PCI as the NR target cell being configured remains detectable according to the cell identification requirements are described in clause 9.2B for intra-frequency handover and clause 9.3B for inter-frequency handover,

-	One of the SSBs in the NR target cell also remains detectable during the handover delay according to the cell identification requirements are described in clause 9.2B for intra-frequency handover and clause 9.3B for inter-frequency handover.

otherwise it is unknown.

- WF:

  - Discuss the corresponding CR directly.

Issue 1-1-4: RRC reestablishment delay/conncection release with redirection delay with RedCap specific initial BWP

- Proposals

  - Option 1: When the Redcap specific initial BWP is configured for RA, extend RRC reestablishment delay by X=6ms;, extend RRC connection release with re-direction delay by X=6ms. (Apple xiaomi vivo QC)

  - Option 2: When the Redcap specific initial BWP is configured for RA, extend RRC reestablishment delay/RRC connection release with re-direction delay by 3 ms. (Nokia MTK)

    - Option 2a:  extend RRC connection release with re-direction delay by 3 ms. (CMCC)

  - Agreement

    - When the Redcap specific initial BWP is configured for RA, extend RRC reestablishment delay by 6 ms; extend RRC connection release with re-direction delay by 6 ms

Sub-topic 1-2 Remaining issues for Redcap timing requirements

Issue 1-2-1: Requirements for the case SSB and MG overlapping

- Proposals

  - Option 1: Add clarification: For the SSB not within the UE’s active BWP case, Te requirement for should be applied when the measurement gap is configured, and at least one measurement gap occasion is for intra-frequency measurement during the 160ms (Xiaomi)

  - Option 2: The current specification is broad enough to cover cases including in option 1 of issue 3-1-1 of [R4-2217140] (vivo)

- WF

  - No consensus. Close the topic

Issue 1-2-2: Timing requirements when SSB is not in the active BWP

- Proposals

  - Option 1: UE shall meet UL Tx timing accuracy requirement based on intra-freq reference SSB outside active BWP if max (MGRP, SMTC period) x CSSFintra_RedCap <= 160 ms (QC)

- WF

  - Session chair: No consensus based on current discussion and no changes will be considered unless there is a consensus.

  - Discuss it directly in the CR

Sub-topic 1-3 Remaining issues for Redcap measurement procedure

Issue 1-3-1: Serving cell threshold associated SSB

- Proposals

  - Option 1: Send LS to RAN2 to confirm the serving cell thresholds of s-MeasureConfig for Connected mode should be checked based on reference SSB measurement. (Apple)

- WF

  - LS R4-2218138 is approved. This topic is closed.

Sub-topic 1-4 Others

Issue 1-4-1: Mismatch between PTW length when measurement relaxation is configured and the maximum PTW length defined by RAN2

- Proposals

  - Observation: a mismatch between the values of PTW length in Table 4.2B.2.9.2-6 and Table 4.2B.2.10.2-6 and RAN2 agreements on maximum PTW length (Nokia)

- WF

  - Discuss this issue at the next meeting