3GPP TSG-RAN WG4 Meeting # 105															R4-2220362

Toulouse, France, November 14 – November 18, 2022

Agenda item:			9.5.9

Source:	Moderator (MediaTek inc.)

Title:	WF on NTN NB-IoT/eMTC RRM requirements

Document for:	Approval

# Introduction

This document is to capture all agreements in email thread [105][231] LTE_NBeMTC_NTN_RRM.

Agreement in the 1st round online

Agreement in the 2nd round

# Topic #1: RRM core requirements (AI 9.5.6)

### General

#### Issue 1-1: Measurement capabilities on number of NGSO satellites

  - The minimum of the UE capability on the total number of the NGSO satellites across the layers is [2].

  - For NB in IDLE and M1 in both IDLE and CONNCTED, the UE shall be capable of monitoring

    - for intra-frequency carrier, the number of target satellites UE needs to monitor is [2] including serving LEO satellite.

    - for inter-frequency carrier, the number of target satellites UE needs to monitor per carrier is [2] if one of the target satellites include the UE serving satellite; the number of target satellites UE needs to monitor is [1] otherwise.

### IDLE state mobility requirements

#### Issue 2-1-1: For NGSO, clarification on UE behavior during out of coverage for Discontinuous Coverage

When the UE is provided with t-serviceStart-r17 and has discontinuous coverage capabilities, then after t-service-r17 is reached and the UE is out of coverage, the UE may delay or resume cell measurements/search till when the UE is in coverage. Definition of in coverage is FFS

#### Issue 2-1-2: For NGSO, revisions on inter-frequency and intra-frequency measurement

- If the UE is configured with ‘t-Service-r17’ [2] in the serving cell and eDRX_cycle, then the UE shall meet the requirements defined for DRX cycle length of [2.56] s starting from at least [K] before ‘t-Service-r17’.

- FFS: the UE shall meet the IDLE mode requirements after time period of X + ‘t-ServiceStart-r17’, where X is time until the UE acquires the system information carrying Ephemeris.

#### Issue 2-1-3: For NGSO, whether to revise the paging requirement to drop paging after t-serviceStart-r17

FFS for following options for discontinuous coverage scenario:

- Option 1: No need to define paging interruption immediately after t-ServiceStart-r17.

- Option 2: The UE is allowed to drop paging during [2] DRX cycles immediately after ‘t-ServiceStart-r17’.

#### Issue 2-2-1: For NGSO, whether to revise the relaxed measurement requirements in IDLE mode

For NGSO, for RRM measurement relaxation, UE is allowed to follow the relaxed measurement requirements, where the relaxation is less compared to corresponding GSO relaxed requirements. How to define the NGSO relaxed requirements are FFS.

FFS the applicability of the following conditions for relaxed measurement requirements for both serving cell and neighbour cell, in the maintenance phase:

  - In NGSO, the cat-M/NB UE configured with ‘t-Service’ in IDLE mode is allowed to meet the relaxed serving cell measurement requirements provided that the UE has met the existing relaxation conditions and the serving cell is not going to stop serving the area, where the UE is located, at least during the last

    - [4] DRX cycles before ‘t-Service-r17’ if not configured with eDRX_IDLE cycle.

    - [1] DRX cycles before ‘t-Service-r17’ if configured with eDRX_IDLE cycle.

  - In NGSO, the serving cell measurement relaxation factor is reduced by factor N from corresponding TN requirements for eMTC and NB-IOT in IDLE mode when cell is served by a LEO satellite compared to corresponding GEO satellite, where N=[2].

#### Issue 2-2-2: For NGSO, whether to revise the relaxed neighbour cell measurement requirements in IDLE mode

FFS for following option:

- The eMTC and NB-IoT UE is allowed to meet the relaxed neighbour cell requirements provided that it has found more than [1] satellite including the serving satellite.

#### Issue 2-3-1: PUR, RSRP-based TA validation

- Agreement

  - Do not define requirements for the legacy RSRP-based TA validation for PUR for both GEO and LEO

### CONNECTED state mobility requirements

#### Issue 3-1-2: For NGSO, triggering RRC Re-establishment based on t-Service

- FFS: not to introduce addition triggering of RRC Re-establishment when t-Service expires.

#### Issue 3-2: Addtional delay upon target cell’s epoch time for Handover, RRC Re-establishment and RRC release with redirection

- For Re-establishment/re-direction or HO requirement, whether to update the requirements or not need to wait for further conclusion from RAN2.

#### Issue 3-3:	M1, CHO requirements

Define CHO requirements for eMTC over NTN by re-using the CHO requirements for normal LTE, except that the following:

- same as in eMTC HO requirements, time for MIB reading should be included in the interruption time when the field sameSFN-Indication or mib-RepetitionStatus [2] is not included

- the references are updated to corresponding eMTC delay requirements

### Timing requirements and RLM

#### Issue 4-1: For Cat-M1 UE in NGSO, whether to revise the RLM requirement upon t-service

- For NGSO and t-Service is provided, the RLM requirements for DRX is used when the time is approaching to t-Service and DRX is used.

- FFS whether non-DRX RLM requirements shall be applied when eDRX is configured.

#### Issue 4-2: Clarification on the timing adjustment for the initial transmission

  - RAN4 to confirm NB-IoT UE is allowed to adjust timing for the first transmission after an uplink transmission gap in a repetition period (R>1) for NPUSCH and NPRACH.

  - RAN4 to confirm Cat-M1 UE is allowed to adjust timing for the first transmission after an uplink transmission gap in a repetition period (R>1) for PUCCH or PUSCH.

# Topic #2: RRM perf reuirements (AI 9.5.7)

#### Issue 1: Measurement accuracy

- Reuse the legacy TN measurement accuracy requirements for IoT NTN.

  - For eMTC, the legacy measurement accuracy requirements (CRS based and RSS based) from LTE eMTC are reused for eMTC NTN.

  - For NB-IoT, the legacy measurement accuracy requirements (NRS based and NSSS-based) from LTE NB-IoT are reused for NB-IoT NTN

#### Issue 2: Duplex mode

- RAN4 to introduce test cases for UE operating in FD-FDD and HD-FDD duplex modes only.

  - RAN4 to introduce test cases for UE category M1 operating in FD-FDD and HD-FDD duplex modes only.

  - RAN4 to introduce test cases for UE category NB1 and NB2 operating in HD-FDD duplex mode only.

#### Issue 3: Satellite assistance information

FFS for following proposals:

- RAN4 to reuse the Rel-17 NR NTN test framework for IoT NTN tests which includes predefining configurations for serving cell’s satellite and neighbour cell’s satellite.

- RAN4 to introduce assistance information for test cases that require neighbour cell measurements for NB-IoT and eMTC. Details of assistance information is FFS.

- RAN4 to introduce assistance information for handover to target cell eMTC. Details of assistance information is FFS.

#### Issue 4: PHR reporting

- For NB-IoT operation with GSO, the legacy NB-IoT PHR reporting tables are used as baseline.

- For NB-IoT operation with NGSO, FFS need to adjust the reporting values compared to the legacy reporting values.

- For UE category M1 operation with GSO and NGSO, the legacy UE category M1 PHR reporting tables are used as baseline.

#### Issue 5: Side conditions

- RAN4 to introduce new side conditions for IDLE and CONNECTED mode measurements for both NB-IoT and eMTC NTN.

#### Issue 6: Reference channel

- RAN4 to introduce new RMC assuming the channel bandwidth of 1.4MHz for eMTC NTN.

#### Issue 7: Test setup – General

- Reuse the test case environment and setup of NR NTN as baseline.

#### Issue 8-1: List of Tests – General

- For NB-IoT and eMTC NTN, RAN4 to introduce same set of test cases (except TDD related) as defined for legacy NB-IoT and eMTC in LTE.

- FFS whether to define test case involving cell detection when the target cell is in enhanced coverage for NGSO.

- FFS whether to define test case for UL segmented transmission gap for UE dropping of UL resources. The new test is applicable for UEs that indicate this capability.

  - The transmission gap for UE dropping of UL resources does not apply when the timing advance is decreasing from one segment to the next segment.

  - RAN4 to reply the LS from RAN1 indicating the segmented transmission will be taken into account when developing test cases

#### Issue 8-2: List of Tests for IoT NTN for NB-IoT

- Further discuss the following test cases list for IoT NTN in the next meeting

|  |  |  | # |
| --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | HD – FDD Intra frequency case for UE Category NB1 in normal coverage | NB 1-1 |
|  |  | HD – FDD Inter frequency case for UE Category NB1 in normal coverage | NB 1-2 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation | NB 1-3 |
|  |  | HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX | NB 1-4 |
| RRC CONNECTED Mode Mobility | NA | NA | n.a. |
| RRC Connection Control | RRC Re-establishment | HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage | NB 2-1 |
|  |  | HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage | NB 2-2 |
|  | Random Access | Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage | NB 2-3 |
|  |  | Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage | NB 2-4 |
|  |  | Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage | NB 2-5 |
| Timing | UE Transmit Timing | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage | NB 3-1 |
|  |  | HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage | NB 3-2 |
|  | UE Timing Advance | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage | NB 3-3 |
|  |  | HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage | NB 3-4 |
| Signaling characteristic | RLM | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage | NB 4-1 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage | NB 4-2 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage | NB 4-3 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage | NB 4-4 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage | NB 4-5 |
|  |  | HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage | NB 4-6 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage | NB 4-7 |
|  |  | HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage | NB 4-8 |
| Measurement Performance Requirements | channel quality reporting accuracy | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage | NB 5-1 |
|  |  | HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage | NB 5-2 |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage | NB 5-3 |
|  |  | HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage | NB 5-4 |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage | NB 5-5 |
|  |  | HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage | NB 5-6 |

#### Issue 8-3: List of Tests for IoT NTN for Cat-M1

- Further discuss the following test cases list for IoT NTN in the next meeting

|  |  |  | # |
| --- | --- | --- | --- |
| RRC_IDLE state | Cell Re-Selection | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-1 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage | M1 1-2 |
|  |  | E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-3 |
|  |  | E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage | M1 1-4 |
|  |  | E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-5 |
|  |  | E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation | M1 1-6 |
| RRC CONNECTED Mode Mobility | Handover and Conditional Handover | E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-1 |
|  |  | E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition | M1 2-2 |
|  |  | E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-3 |
|  |  | E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA | M1 2-4 |
|  |  | E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-5 |
|  |  | E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-6 |
|  |  | E-UTRAN FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-7 |
|  |  | E-UTRAN HD-FDD inter frequency conditional handover for Cat-M1 UEs in CEModeA | M1 2-8 |
| RRC Connection Control | RRC Re-establishment | E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-1 |
|  |  | E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-2 |
|  |  | E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-3 |
|  |  | E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA | M1 3-4 |
|  | Random Access | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-5 |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage | M1 3-6 |
|  |  | E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-7 |
|  |  | E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage | M1 3-8 |
| Timing | UE Transmit Timing | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-1 |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA | M1 4-2 |
|  |  | E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-3 |
|  |  | E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB | M1 4-4 |
|  | UE Timing Advance | E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-5 |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA | M1 4-6 |
|  |  | E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-7 |
|  |  | E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB | M1 4-8 |
| Signaling characteristic | RLM | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-1 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-2 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A | M1 5-3 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A | M1 5-4 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-5 |
|  |  | E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-6 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for Cat-M1 UE in CEMode A | M1 5-7 |
|  |  | E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync in DRX for Cat-M1 UE in CEMode A | M1 5-8 |
| Measurement Procedure | Event triggered reporting | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-1 |
|  |  | E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-2 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA | M1 6-3 |
|  |  | E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX | M1 6-4 |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-5 |
|  |  | E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-6 |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA | M1 6-7 |
|  |  | E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used | M1 6-8 |
| Measurement Performance Requirements | RSRP accuracy | FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-1 |
|  |  | HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA | M1 7-2 |
|  |  | FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-3 |
|  |  | HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA | M1 7-4 |
|  | channel quality reporting accuracy | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-5 |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A | M1 7-6 |
|  |  | E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-7 |
|  |  | E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B | M1 7-8 |