3GPP TSG-RAN WG4 Meeting #105	R4-2220398

Toulouse, France, November 14 –18, 2022

Title: 	WF on RRM requirements for intra-band non-collocated EN-DC/NR-CA

Source: 	Huawei, HiSilicon, Samsung

Agenda item:	8.11.3

Document for:	Approval

Introduction

This contribution is to capture the progress on RRM requirements for intra-band non-collocated EN/DC/NR-CA in RAN4 #105 meeting.

Way-forward

Topic#1: FR1 non-collocated EN-DC/NR-CA for Type 2 UE for 2 layer MIMO case

Sub-topic 1-1 General aspects

Issue 1-1-1: RRM impacts for Type 2 UE

- Proposals:

  - Option 1:

    - For Type 2 UE, RAN4 to discuss MRTD/MTTD, interruption requirement, and scheduling restriction for FR1 non-collocated deployment for the following scenarios:

      - intra-band non-contiguous NR-CA

        - In addition, RAN4 to discuss the impact on RRM requirement on SCell BFD.

  - Option 2:

    - FFS

Sub-topic 1-2 MRTD/MTTD requirements

Issue 1-2-1: MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE

- Agreements

  - MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for Type 2 UE

    - MRTD=33us

    - MTTD=34.6us

Issue 1-2-2: MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE

- Agreements

  - MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for Type 2 UE

    - MRTD=33us

    - MTTD=35.21us

Issue 1-2-3: MRTD/MTTD requirements for non-collocated FR1 intra-band asynchronous NR-CA for Type 2 UE

- Proposals

  - Option 1:

    - Firstly focus on synchronous operation and revisit asynchronous operation topic after synchronous.

Issue 1-2-4: MRTD/MTTD requirement applicability for Rel-15/16/17 UE:

- Proposals

  - Option 1:

    - FFS the applicability of existing MRTD/MTTD requirements for different UE capabilities, i.e. UE supporting interBandMRDC-WithOverlapDL-Bands-r16 or not.

    - FFS whether to have further clarification in the spec.

Sub-topic 1-3 Other RRM requirements

Issue 1-3-1: Impacts on SCell activation requirements

- Proposals

  - Option 1:

    - The existing SCell activation delay for intra-band contiguous scenario needs to be clarified with co-location assumption.

    - RAN4 shall define the SCell activation delay for activating an FR1 intra-band non-collocated SCell.

  - Option 2:

    - FFS

Issue 1-3-2: Impacts on scheduling availability requirements

- Proposals

  - Option 1:

    - The scheduling availability needs to be revisited when intra-band non-collocated CA in FR1 is configured

  - Option 2:

    - FFS

Topic#2: FR1 non-collocated EN-DC/NR-CA for New Type UE for 4 layer MIMO case

Sub-topic 2-1 General aspects

Issue 2-1-1: Discussion on RRM requirements for New Type UE for 4 layer MIMO

- Proposals

  - Option 1:

    - Discussion on RRM requirements for “New Type UE” for 4 layer MIMO needs to wait for RF’s progress

  - Option 2:

    - RAN4 RRM session shall prioritize the discussion on MRTD requirement for new UE Type-3a/3b for both EN-DC and NR-CA.

  - Option 3:

    - RAN4 RRM session can discuss in parallel based on the latest RF’s progress.

Sub-topic 2-2 MRTD/MTTD requirements

Issue 2-2-1: MRTD/MTTD requirements for non-collocated FR1 intra-band non-contiguous NR-CA for New Type UE

- Proposals

  - Option 1:

    - Wait for RF progress on the reference architecture for 4 layer MIMO case.

Issue 2-2-2: MRTD/MTTD requirements for non-collocated FR1 inter-band synchronous EN-DC with overlapping DL bands for New Type UE

- Proposals

  - Option 1:

    - Wait for RF progress on the reference architecture for 4 layer MIMO case.

Issue 2-2-3: MRTD/MTTD requirements for non-collocated FR1 inter-band asynchronous EN-DC with overlapping DL bands for New Type UE

- Proposals

  - Option 1:

    - Wait for RF progress on the reference architecture for 4 layer MIMO case.