3GPP TSG-RAN WG4 Meeting #105	draft-R4-2220495

Electronic Meeting, 14 Nov. – 18 Nov. 2022

Agenda Item:	7.28.3

Source:	Charter Communications, Inc.

Title:	Way forward for NR-U PC3 MPR/A-MPR

Document for:	Approval

# 1	Introduction of Power Class 3

1.1	Topic #1: PC3 MPR and A-MPR

## Issue 1-1-1: PC3 1Tx MPR proposals

- Proposal

  - Option 1: Proposal from R4-2218048

  - Option 2: Proposal from R4-2218103

  - Option 3: Proposal from R4-2218530

- Agreement:

  - For the PC3 1Tx MPR requirement, use the average approach among companies’ results, and agree on the value with [ ]

  - FFS on whether to round up or down to the next 0.5dB

- Table 1: Baseline values based on averaging companies’ values.

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [0.83] | ≤ [1.66] |
|  | QPSK | ≤ [1.16] | ≤ [1.83] |
|  | 16 QAM | ≤ [1.50] | ≤ [2.33] |
|  | 64 QAM | ≤ [2.16] | ≤ [3.16] |
|  | 256 QAM | ≤ [4.00] | ≤ [4.66] |
| CP-OFDM | QPSK | ≤ [2.16] | ≤ [3.33] |
|  | 16 QAM | ≤ [2.66] | ≤ [3.50] |
|  | 64 QAM | ≤ [3.83] | ≤ [4.66] |
|  | 256 QAM | ≤ [6.16] | ≤ [6.50] |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

- Recommended WF

  - Propose if rounding up or down to the next 0.5dB should be applied.

  - Some clarification of the scenario setup, due to suggestion that cases requiring more backoff may not have been captured for wideband.

  - Bandwidths for all test numbers: 20, 40, 60, 80, and 100MHz

  - The examples of wideband configuration are only example of wideband Sub-band. See the full list of sub-band configuration in TS. 38.101-1 in Table 6.2F.2-2 MPR mapping for wideband operation

Table 2: Suggestion of simulation scenarios for MPR

| Test Number | Precoder | Modulation | Allocation | Sub-Band |
| --- | --- | --- | --- | --- |
| 1 | DFT-S-OFDM | Pi/2-BPSK | Full | All |
| 2 | CP-OFDM | Pi/2-BPSK | Interlace | All |
| 3 | DFT-S-OFDM | Pi/2-BPSK | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 4 | CP-OFDM | Pi/2-BPSK | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100, 11000 |
| 5 | DFT-S-OFDM | QPSK | Full | All |
| 6 | CP-OFDM | QPSK | Full | All |
| 7 | DFT-S-OFDM | QPSK | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 8 | CP-OFDM | QPSK | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 9 | DFT-S-OFDM | QPSK | Interlace | All |
| 10 | CP-OFDM | QPSK | Interlace | All |
| 11 | DFT-S-OFDM | QPSK | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 12 | CP-OFDM | QPSK | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 13 | DFT-S-OFDM | 16QAM | Full | All |
| 14 | CP-OFDM | 16QAM | Full | All |
| 15 | DFT-S-OFDM | 16QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 16 | CP-OFDM | 16QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 17 | DFT-S-OFDM | 16QAM | Interlace | All |
| 18 | CP-OFDM | 16QAM | Interlace | All |
| 19 | DFT-S-OFDM | 16QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 20 | CP-OFDM | 16QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 21 | DFT-S-OFDM | 64QAM | Full | All |
| 22 | CP-OFDM | 64QAM | Full | All |
| 23 | DFT-S-OFDM | 64QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 24 | CP-OFDM | 64QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 25 | DFT-S-OFDM | 64QAM | Interlace | All |
| 26 | CP-OFDM | 64QAM | Interlace | All |
| 27 | DFT-S-OFDM | 64QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 28 | CP-OFDM | 64QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 29 | DFT-S-OFDM | 256QAM | Full | All |
| 30 | CP-OFDM | 256QAM | Full | All |
| 31 | DFT-S-OFDM | 256QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 32 | CP-OFDM | 256QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 33 | DFT-S-OFDM | 256QAM | Interlace | All |
| 34 | CP-OFDM | 256QAM | Interlace | All |
| 35 | DFT-S-OFDM | 256QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 36 | CP-OFDM | 256QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |

  - Simulation scenario:

    - Define MPR based on 27dBc ACLR

    - Suggest to use Table 1 as baseline for simulation scenarios

    - Sub-band configuration 11100 (100MHz) wideband operation is a good case for this further investigation. All other configuration in column B, in TS. 38.101-1 in Table 6.2F.2-2, is good to check if possible.

## 1.2	Sub-topic 1-2: A-MPR

Issue 1-2-1: PC3 US LPI 1Tx (NS_53)

- Proposals

  - Option 1: Proposal from R4-2218048

  - Option 2: Proposal from R4-2218103

- Agreement:

  - For PC3 1Tx A-MPR for NS_53, agree on option 2.

Table 3: Agreed table for PC3 1Tx A-MPR for NS_53

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 16 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 5.5 | ≤ 8.5 | ≤ 4.5 | ≤ 7.5 |
|  | 256 QAM | ≤ 12.0 | ≤ 14.5 | ≤ 9.0 | ≤ 11.5 | ≤ 7.0 | ≤ 10.0 | ≤ 6.5 | ≤ 8.5 | ≤ 6.5 | ≤ 7.5 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - Two companies both say the table is limited by in-band PSD for all but for CP-OFDM 256QAM for 80MHz and 100MHz with Full RB allocation.

  - Evaluate A-MPR for 2Tx to understand if one or two A-MPR tables are needed to be specified.

    - Merging two non-equal tables are not precluded.

Issue 1-2-2: PC5 1Tx A-MPR for NS_53

- Proposals

  - Option 1: Proposal from R4-2219705

Table 4: Baseline table

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2-BPSK | ≤ [9.0] | ≤ [12.0] | ≤ [6.5] | ≤ [8.5] | ≤ [4.5] | ≤ [6.5] | ≤ [3.0] | ≤ [5.5] | ≤ [1.5] | ≤ [2.5] |
|  | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 2.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 3.0 | ≤ 5.5 | ≤ 1.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 2.0 | ≤ 3.0 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.0 | ≤ 7.0 | ≤ 5.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 3.5 | ≤ 4.5 |
|  | 16 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 4.5 | ≤ 6.5 | ≤ 4.0 | ≤ 5.5 | ≤ 4.0 | ≤ 4.5 |
|  | 64 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 6.5 | ≤ 8.5 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 | ≤ 12.0 | ≤ 7.0 | ≤ 8.5 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 | ≤ 7.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. |  |  |  |  |  |  |  |  |  |  |  |

- Agreement:

  - For PC5 1Tx A-MPR for NS_53, agree on Option 1.

- Recommended WF

  - Agree on pi/2 BPSK values

    - with the same value as QPSK (DFT-s-OFDM precoder), These values are in [].

    - Other values

  - If other companies have time to check the values for comparison, it would be highly appreciated.

Issue 1-2-3: US SP 1Tx (NS_54)

- Proposals

  - Option 1: Proposal from R4-2218048

  - Option 2: Proposal from R4-2218103

- Agreement:

Table 5: Baseline table, dependable of finalized MPR table

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  | RB Allocation (Note 5) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table [PC3 MPR] | ≤ [2.0] | ≤ [4.5] | ≤ [1.0] | ≤ [2.5] |
|  | QPSK |  | ≤ [3.0] | ≤ [5.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] | ≤ [2.0] | ≤ [3.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] | ≤ [2.5] | ≤ [3.5] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] | ≤ [3.0] | ≤ [4.0] |
|  | 16 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [3.0] | ≤ [4.0] |
|  | 64 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [4.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [6.5] | ≤ [7.0] | ≤ [6.5] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Applicable for 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 and 6015 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6005 MHz]. |  |  |  |  |  |  |

- Recommended WF

  - Check the results again using tables of scenarios as baseline, ‘Table 2: Suggestion of simulation scenarios for MPR’.

  - Evaluate and discuss if a dedicated 2Tx A-MPR tables is needed.

    - Merging two non-equal tables are not precluded.

Issue 1-2-4: PC3 1Tx A-MPR for NS_30

- Proposals

  - Option 1: Proposal from R4-2218048

  - Option 2: TBA

- Agreement:

Table 6: Baseline backoff results in [].

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK | ≤ [12.0] | ≤ [15.5] | ≤ [3.5] | ≤ [5.5] | See Table MPR table for PC3 |
|  | QPSK | ≤ [12.5] | ≤ [16.0] | ≤ [4.0] | ≤ [6.5] |  |
|  | 16 QAM | ≤ [12.5] | ≤ [16.0] | ≤ [4.5] | ≤ [7.0] |  |
|  | 64 QAM | ≤ [13.0] | ≤ [16.0] | ≤ [4.5] | ≤ [7.0] |  |
|  | 256 QAM | ≤ [13.0] | ≤ [16.0] | ≤ [4.5] | ≤ [7.0] |  |
| CP-OFDM | QPSK | ≤ [14.0] | ≤ [16.5] | ≤ [6.0] | ≤ [7.5] |  |
|  | 16 QAM | ≤ [14.0] | ≤ [16.5] | ≤ [6.0] | ≤ [7.5] |  |
|  | 64 QAM | ≤ [14.0] | ≤ [16.5] | ≤ [6.0] | ≤ [7.5] |  |
|  | 256 QAM | ≤ [14.0] | ≤ [16.5] | ≤ [6.5] | ≤ [7.5] |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. |  |  |  |  |  |  |

- Recommended WF

  - Use the same simulation scenario table as MPR (Table 2 in Issue 1-1-1) as baseline to ensure the maximum backoff is found.

Issue 1-2-5: South Korea LPI 1Tx (NS_60)

- Proposals

  - Option 1: Proposal from R4-2218103

  - Option 2: Proposal from R4-2218530

- Agreement:

  - Option 1

Table 7: Agreed results for LPI 1Tx A-MPR (NS_60)

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-ODFM | PI/2 BPSK2 | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 2.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 3.5 | ≤ 5.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.0 | ≤ 6.0 | ≤ 4.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies. NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

Issue 2-2-5: South Korea LPI 2Tx (NS_60)

- Proposals

  - Option 1: Proposal from R4-2218530

- Agreement:

Table 8: Propose table for LPI 2Tx A-MPR for NS_60 with values in [].

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | QPSK | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | 16 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | 64 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [3.0] | ≤ [6.0] | ≤ [3.0] | ≤ [5.5] |
|  | 256 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [4.5] | ≤ [7.0] | ≤ [4.0] | ≤ [6.0] | ≤ [4.0] | ≤ [5.5] |
| CP-OFDM | QPSK | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
|  | 16 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
|  | 64 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [4.5] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
|  | 256 QAM | ≤ [9.0] | ≤ [11.5] | ≤ [6.0] | ≤ [9.0] | ≤ [5.0] | ≤ [7.0] | ≤ [5.5] | ≤ [6.0] | ≤ [5.5] | ≤ [5.5] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies |  |  |  |  |  |  |  |  |  |  |  |

- Recommended WF

  - See if more companies may bring simulation results for next meeting.

  - Further discuss 2Tx A-MPR in relation to 1Tx A-MPR.

    - The two tables differ for the bandwidths’ 80 and 100MHz. Discuss if a dedicated needed for 2Tx A-MPR?

    - Merging 2Tx with 1Tx for NS_60 is not precluded.