3GPP TSG-RAN WG4 Meeting # 105.                                   R4-2220538

Toulouse, France, 14th – 18th November 2022

Source: 	Apple

Title: 	TP for TR 38.876: frequency error

Agenda Item:	8.13.2.1

Document for:	Discussion

# Introduction

This contribution presents a text proposal on frequency error for ATG UE for TR 38.876.

# Reference

[1] R4-2218693, Discussion on ATG general aspects, Apple

[2] R4-2218694, ATG UE Tx RF requirements, Apple

# Text proposal

----------------------------------------------------------Start of Text Proposal------------------------------------------------------------

## 7.1 ATG CPE specific

### 7.1.1 ATG CPE power class and requirement type

### 7.1.2 Tx requirements

7.1.2.x Frequency error

The doppler frequency for ATG UE can be determined by,

fD =  cosθ*vc*f_c

Where,

- fD is Doppler frequency, f_c is the carrier frequency, which is set as 5GHz for n79.

- v is the speed of the aircraft which is 1200km/h.

- θ is the elevation angle between UE and gNB.

Since the potential maximum cell range is up to 200-300km [x] and the normal commercial airplane altitude is 10km, the elevation angle is only about 3 degrees when UE doing initial access at cell edge. Then the maximum DL doppler frequency is approaching 5.5kHz and the UL doppler frequency is approaching 11kHz if without any compensation which may cause not negligible impact for link performance and access successful rate. ATG UE needs to perform frequency compensation. The frequency accuracy requirement will be written assuming Doppler frequency pre-compensation.

The ATG BS location information needs to be broadcasted for UE to do frequency compensation. UE shall rely on the ATG BS location broadcasted by the IE EphemerisInfo in NTN SIB 19 in 38.331 if pre-compensation is based on SIB19. Where, Ephemeris may be expressed in format of position and velocity state vector.

### 7.1.3 Rx requirements

---------------------------------------------------------End of Text Proposal--------------------------------------------------------------