3GPP TSG-RAN4 Meeting #105	R4-2220716

# Toulous, France, 14 – 18 November, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | 1 | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for CG-SDT testcase for FR1 revision from R4-2217245 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SmallData_INACTIVE-Perf |  |  |  |  |  | Date: |  |  | 2022-11-7 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Update detail timeline description for FR1 CG-SDT test case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases requirements are updated. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case requirements are not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.2.1, A.6.2.1.1, A.6.2.1.2, A.6.2.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change>

## A.6.2.1	Configured Grant based Small Data Transmissions (CG-SDT)

A.6.2.1.1 Test purpose and Environment

The purpose of this test is to verify that the UE properly perform TA validation for CG-SDT transmission in clause 5.5.3. In the test, UE is configured two CG-SDT configurations at different time. The test consists of 6 successive time periods, with time duration of T1, T2, T3, T4, and T5 repectively. There is one cell, which is the active NR cell in FR1. Figure A.6.2.1.1 shows the variation of the RSRP per time. Prior to the time point TA, the UE shall be fully synchronized to PCell (Cell 1).

Before starting the test at time point TA, test equipment configures RSRP to P0.

At time point TB, RSRP is changed from P0 to P1.

At time point TC, UE expect to receive RRC release with CG SDT configuration and RRC status is changed to INACTIVE status.

At time point TD, RSRP is changed from P1 to P0.

At time point TE, RSRP is changed from P0 to P1.

Test equipment triggers UL data arrival at UE lower layer at time point TF.

At time point TH, UE expect to receive a RRC release with CG-SDT configurations.

At time point TI, RSRP is changed from P2 to P3.

Test equipment triggers UL data arrival at UE lower layer at time point TJ.

Figure A.6.2.1.1: RSRP variation model for CG-SDT testing

A.6.2.1.2 Test Parameters

The test parameters for the PCell are given in Table A.6.2.1.2-1, Table A.6.2.1.2-2, and Table A.6.2.1.2-3.

Table A.6.2.1.2-1: NR configuration for FR1 SSB

| Config | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.2.1.2-2 : General test parameters

| Parameter | Unit |  | Test configuration |  | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TDD Configuration |  |  | 1 |  | N/A |  |  |
|  |  |  | 2 |  | TDDConf.1.1 |  |  |
|  |  |  | 3 |  | TDDConf.2.1 |  |  |
| BWchannel | MHz |  |  |  | 10: NRB,c = 52 |  |  |
| PDSCH Reference measurement channel |  |  | 1 |  | SR.1.1 FDD |  |  |
|  |  |  | 2 |  | SR.1.1 TDD |  |  |
|  |  |  | 3 |  | SR.2.1 TDD |  |  |
| RMSI CORESET Reference Channel |  |  | 1 |  | CR.1.1 FDD |  |  |
|  |  |  | 2 |  | CR.1.1 TDD |  |  |
|  |  |  | 3 |  | CR.2.1 TDD |  |  |
| Dedicated CORESET Reference Channel |  |  | 1 |  | CCR.1.1 FDD |  |  |
|  |  |  | 2 |  | CCR.1.1 TDD |  |  |
|  |  |  | 3 |  | CCR.2.1 TDD |  |  |
| SSB configuration |  |  | 1,2,3 |  | SSB.1 FR1 |  |  |
| OCNG Patterns |  |  | 1,2,3 |  | OP.1 |  |  |
| Initial BWP Configuration |  |  | 1,2,3 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Dedicated BWP configuration |  |  | 1,2,3 |  | DLBWP.1.1 ULBWP.1.1 |  |  |
| SMTC configuration |  |  | 1,2,3 |  | SMTC.1 |  |  |
| DRX configuration |  |  | 1,2,3 |  | DRX.7 |  |  |
| T1 | s |  | 1,2,3 |  | [1] |  |  |
| T2 | s |  | 1,2,3 |  | [1.28] |  |  |
| T3 | s |  | 1,2,3 |  | [2.36] |  |  |
| T4 | s |  | 1,2,3 |  | [2.56] |  |  |
| T5 | s |  | 1,2,3 |  | [2.36] |  |  |
| cg-SDT-RSRP-ChangeThreshold | dB |  | 1,2,3 |  | [8] |  |  |
| cg-SDT-TimeAlignmentTime |  |  | 1,2,3 |  | Infinity |  |  |
| CG-SDT resource period | ms |  | 1,2,3 |  | 640ms |  | configuredGrantConfig Peridocity at 38.331 Config 1,2 : sym640x14, config 3 : sym1024x14 |
| Propagation condition |  |  | 1,2,3 |  | AWGN |  |  |

Table A.6.2.1.2-3: SSB specific test parameters

| Parameter |  | Config |  | Unit |  |  | SSB#0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 |  | T3 | T4 | T5 |
|  | 1,2,3 |  | dBm/15kHz |  | -100 |  |  |  |  |  |
|  | 1,2 |  | dBm/SSB SCS |  | -100 |  |  |  |  |  |
|  | 3 |  | dBm/SSB SCS |  | -103 |  |  |  |  |  |
|  | 1,2,3 |  | dB |  | [0] | [10.35] |  | [0] | [15.25] | [6] |
| SS RSRP Note2 | 1,2,3 |  | dBm/SSB SCS |  | [-100] | [-89.65] |  | [-100] | [-84.75] | [-96] |
| Io | 1,2 |  | dBm/9.36 MHz |  | [-69.04] | [-61.31] |  | [-69.04] | [-56.67] | [-65.07] |
|  | 3 |  | dBm/38.16MHz |  | [-69.04] | [-61.31] |  | [-69.04] | [-56.67] | [-65.07] |
|  | 1,2,3 |  | dB |  | [0] | [10.35] |  | [0] | [15.25] | [6] |

A.6.2.1.3 Test requirements

The UE behaviour in each test during time durations shall be as follows:

During T4, UE shall transmit PUSCH at CG-SDT resource within 640ms + Z after time point TF.

During T5, UE shall not transmit PUSCH at CG-SDT resources after TJ until the end of the test at time point TK.

The rate of correct events observed during repeated tests shall be at least 90%.

<End of Change>