3GPP TSG-RAN WG4 Meeting #105	R4- 2220837

France Toulouse, 14-18 November 2022

Title:	LS on intraBandENDC-Support

Response to:

Release:	Rel-16

Work Item:

Source:	RAN4

To:	RAN2

Cc:	RAN

Contact Person:

Name: 	Hu Dan

E-mail Address: 	hudan11@huawei.com

Attachments:	N/A

1. Overall Description:

The IE intraBandENDC-Support in UE capability is related to three states {contiguous, non-contiguous, both} per band combination with paired UL and DL, where the indication applies for both the UL and DL. The UE is not able to report the band combinations as Case 3 and Case 4, which are with inconsistent contiguity of UL and DL, by indicating ‘non-contiguous’ or ‘contiguous’ (by UE not reporting the field).

- Case 3: All CCs are contiguous in DL but neither carrier is contiguous to each other in UL, including

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_(n)48CA | DC_48A_n48A |
| DC_(n)48DA | DC_48A_n48A |

- Case 4: One of LTE carriers and the NR carrier are contiguous in DL, contiguous and non-contiguous are both supported in UL:

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_48A_(n)48AA | DC_(n)48AA DC_48A_n48A |

RAN4 has concluded keeping the cases of b48+n48 as illustrated in the two tables above in the RAN4 specification, but the indication of the support of these cases by a UE is unclear.

In RAN4’s view, if UE indicates intraBandENDC-Support with “both” for case 3 and case 4, there should be no ambiguity from implementation capability perspective:

- For case 3: “both” includes UE supports non-contiguous in UL paired with contiguous in DL.

- For case 4: “both” includes UE supports contiguous in DL paired with contiguous/non-contiguous in UL. Meanwhile, RAN4 also considers that “both” capability for intra-band EN-DC combination for case 4 (with three band entries) is limited to two sub-blocks for UL or DL configurations, i.e. two contiguous spectrum sub-blocks with only one gap in-between the blocks, and one of the sub-blocks consists of a contiguous EN-DC configuration in Table 5.3B.0-1 in TS 38.101-3.

The signalling solution is not precluded from RAN4’s perspective.

2. Actions:

To RAN2:

RAN4 respectfully asks RAN2 to check whether the interpretation of ‘both’ can be supported by RAN2 for Case 3 and Case 4.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN4 Meeting#106	                          Feb 2023											 Athens

TSG-RAN4 Meeting#106bis-e                      April 2023										 E-meeting