3GPP TSG-RAN WG4 Meeting #106	R4-2300059

Electronic Meeting, 27 Feb. – 3 Mars. 2023

Agenda Item:	8.31.3

Source:	Charter Communications, Inc.

Title:	Simulation results on UE RF MPR and A-MPR for PC3

Document for:	Approval

# Introduction

In RAN4#104-e meeting, a WF for introducing Power class 3 (PC3) in NR-unlicensed spectrum was approved [1]. In RAN4#104-bis-e meeting, a WF with a few agreements and some way forward from initial results by companies [2]. In RAN4#105 meeting, the latest WF with a few more agreements and the WF with results from companies [3].

In this paper we will provide simulation results for NR-U MPR and A-MPR for NS_30 and NS_54. The paper will start off recapping the requirements for the two NS values. Tentative agreement tables for NS_54. So far, companies have only provided simulations of 1Tx [3].

Average (mean) values of simulations from companies in Table 1 for NR-U MPR.

Table 1: Baseline values based on averaging companies’ values.

| Pre-coding | Modulation | RB Allocation |  |
| --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [0.83] | ≤ [1.66] |
|  | QPSK | ≤ [1.16] | ≤ [1.83] |
|  | 16 QAM | ≤ [1.50] | ≤ [2.33] |
|  | 64 QAM | ≤ [2.16] | ≤ [3.16] |
|  | 256 QAM | ≤ [4.00] | ≤ [4.66] |
| CP-OFDM | QPSK | ≤ [2.16] | ≤ [3.33] |
|  | 16 QAM | ≤ [2.66] | ≤ [3.50] |
|  | 64 QAM | ≤ [3.83] | ≤ [4.66] |
|  | 256 QAM | ≤ [6.16] | ≤ [6.50] |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |

- Recommended WF

  - Propose if rounding up or down to the next 0.5dB should be applied.

  - Some clarification of the scenario setup, due to suggestion that cases requiring more backoff may not have been captured for wideband.

  - Bandwidths for all test numbers: 20, 40, 60, 80, and 100MHz

  - The examples of wideband configuration are only example of wideband Sub-band. See the full list of sub-band configuration in TS. 38.101-1 in Table 6.2F.2-2 MPR mapping for wideband operation

The way forward for NS_54 contained following discussion points:

- Check the results again using tables of scenarios as baseline. (See Table 3: Setup for MPR, the table is copied from the WF [3])

- Evaluate and discuss if a dedicated 2Tx A-MPR tables is needed.

  - Merging two non-equal tables are not precluded.

Table 2: Baseline table, dependable of finalized MPR table

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  | RB Allocation (Note 5) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK4 | See Table [PC3 MPR] | ≤ [2.0] | ≤ [4.5] | ≤ [1.0] | ≤ [2.5] |
|  | QPSK |  | ≤ [3.0] | ≤ [5.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] | ≤ [2.0] | ≤ [3.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] | ≤ [2.5] | ≤ [3.5] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [4.5] | ≤ [5.5] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] | ≤ [3.0] | ≤ [4.0] |
|  | 16 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [3.0] | ≤ [4.0] |
|  | 64 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [4.5] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [6.5] | ≤ [7.0] | ≤ [6.5] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Applicable for 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 and 6015 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6005 MHz]. |  |  |  |  |  |  |

# Discussion

## PC3 MPR

Simulations were done for unlicensed spectrum with PC3. The PA was calibrated with the agreed QPSK DFT-s-OFDM waveform. The detailed list of the setup is provided in the following:

- Calibration: QPSK DFT-s-OFDM 100RB with 1dB backoff

- ACLR: 27dB

- SEM: NR-U

- Image rejection: -28dB

- Carrier leakage rejection: -28dBc

Table 3: Setup for MPR

| Test Number | Precoder | Modulation | Allocation | Sub-Band |
| --- | --- | --- | --- | --- |
| 1 | DFT-S-OFDM | Pi/2-BPSK | Full | All |
| 2 | CP-OFDM | Pi/2-BPSK | Interlace | All |
| 3 | DFT-S-OFDM | Pi/2-BPSK | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 4 | CP-OFDM | Pi/2-BPSK | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100, 11000 |
| 5 | DFT-S-OFDM | QPSK | Full | All |
| 6 | CP-OFDM | QPSK | Full | All |
| 7 | DFT-S-OFDM | QPSK | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 8 | CP-OFDM | QPSK | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 9 | DFT-S-OFDM | QPSK | Interlace | All |
| 10 | CP-OFDM | QPSK | Interlace | All |
| 11 | DFT-S-OFDM | QPSK | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 12 | CP-OFDM | QPSK | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 13 | DFT-S-OFDM | 16QAM | Full | All |
| 14 | CP-OFDM | 16QAM | Full | All |
| 15 | DFT-S-OFDM | 16QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 16 | CP-OFDM | 16QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 17 | DFT-S-OFDM | 16QAM | Interlace | All |
| 18 | CP-OFDM | 16QAM | Interlace | All |
| 19 | DFT-S-OFDM | 16QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 20 | CP-OFDM | 16QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 21 | DFT-S-OFDM | 64QAM | Full | All |
| 22 | CP-OFDM | 64QAM | Full | All |
| 23 | DFT-S-OFDM | 64QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 24 | CP-OFDM | 64QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 25 | DFT-S-OFDM | 64QAM | Interlace | All |
| 26 | CP-OFDM | 64QAM | Interlace | All |
| 27 | DFT-S-OFDM | 64QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 28 | CP-OFDM | 64QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 29 | DFT-S-OFDM | 256QAM | Full | All |
| 30 | CP-OFDM | 256QAM | Full | All |
| 31 | DFT-S-OFDM | 256QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 32 | CP-OFDM | 256QAM | Wideband Full | e.g. 01, 110, 1100, 1110, 11100 |
| 33 | DFT-S-OFDM | 256QAM | Interlace | All |
| 34 | CP-OFDM | 256QAM | Interlace | All |
| 35 | DFT-S-OFDM | 256QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |
| 36 | CP-OFDM | 256QAM | Wideband interlace | e.g. 01, 110, 1100, 1110, 11100 |

## MPR Simulation Results

Figure 1: Simulation results for MPR PC3 with 27dB ACLR

Table 4: MPR for PC3 with 27dB ACLR

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | 100MHz Exception |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [0.5] | ≤ [1.0] |  |
|  | QPSK | ≤ [1.0] | ≤ [1.5] | [2.0] |
|  | 16 QAM | ≤ [1.5] | ≤ [2.0] | [2.5] |
|  | 64 QAM | ≤ [2.5] | ≤ [3.0] |  |
|  | 256 QAM | ≤ [4.5] | ≤ [5.0] |  |
| CP-OFDM | QPSK | ≤ [2.0] | ≤ [3.0] | [3.5] |
|  | 16 QAM | ≤ [2.5] | ≤ [3.0] | [3.5] |
|  | 64 QAM | ≤ [4.0] | ≤ [4.5] |  |
|  | 256 QAM | ≤ [6.5] | ≤ [7.0] |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. The MPR applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10]. NOTE 2: Full RB allocation MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and sub-bands are transmitted according to configuration A in Table 6.2F.2-2. NOTE 3: Partial RB allocation MPR applies when one or more RB’s in one or more sub-bands are not allocated or when the transmitted sub-bands for wideband operation are transmitted according to configuration B in Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |

Proposal 1: Proposed MPR PC3 simulation results presented in Table 4 for shared spectrum.

Compare to the baseline table, we have added our simulation for the 100MHz Full Exception simulations for the wideband operation when not all sub-bands are actively transmitting. An observation here, if we take into account the values from the average of companies simulations and rounds them all up, the partial and 100MHz exception from our simulations will be the same, hence the exception isn’t necessary in Table 4.

We suggest that the rounding should be to the closest 0.5dB, not all rounding up or down, as suggested in the way forward.

Proposal 2:  Round to the closest 0.5dB based on the mean results from companies.

## PC3 A-MPR

The networking signaling value NS_30 is used when operating in band n46, while the networking signaling values NS_53 and NS_54 is used when operating in band n96. Agreement that Power class 3 will be introduced for both of these unlicensed bands, as well as band n102.

## In-Band emissions

There are additional requirements for the transmit power density as seen in Table 6.2F.1-2 in TS 38.101-1 [3] for all unlicensed bands. A part of Table 6.2F.1-2 is shared below.

Approximately, for a full allocation in a 20MHz channel bandwidth, with maximum Power for PC3 (23dBm), the maximum mean power density (dBm/MHz) would be around 10, this means the in-band emission requirements for NS_53, will be very much leading the simulation results, while for NS_30 and NS_54 will probably not be restrictive/stringent requirement. The wider bandwidth is simulated, the more the power density is spread out in the channel bandwidth.

Table 6.2F.1-2: Additional requirements for transmit power density

| NR Band | NS value | Channel bandwidth (MHz) | Frequency range (MHz) | Maximum mean power density (dBm/MHz) |
| --- | --- | --- | --- | --- |
| n46 | NS_30 | 20, 40, 60, 80 | 5150 – 5350 | 11 |
|  |  |  | 5470 – 5725 |  |
| n96 | NS_53 | 20, 40, 60, 80 | 5925 – 7125 | -1 |
|  | NS_54 | 20, 40, 60, 80 | 5925 – 6425 | 17 |
|  |  |  | 6525 – 6875 |  |

## Out-of-Band emissions

When "NS_30" is indicated in the cell, the power of any UE emission for channels assigned within 5150-5350 MHz, 5470-5725 MHz and 5725-5850 MHz shall not exceed the levels specified in Table 6.5F.3.3.3-1-1, Table 6.5F.3.3.3-1-2 and Table 6.5F.3.3.3-1-3, respectively.

Table 6.5F.3.3.3-1: Additional requirements for shared access channels assigned within 5150-5350 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |

Table 6.5F.3.3.3-2: Additional requirements for shared access channels assigned within 5470-5725 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |
| 5460 < f ≤ 5470 | -27 |  |
| 5725 ≤ f | -27 |  |

Table 6.5F.3.3.3-3: Additional requirements for shared access channels assigned within 5725-5850 MHz

| Protected range (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20, 40, 60, 80, [100] MHz |  |
| f < 5650 | -27 | 1 MHz |
| 5650 ≤ f < 5700 | -27 to 10 |  |
| 5700 ≤ f < 5720 | 10 to 15.6 |  |
| 5720 < f ≤ 5725 | 15.6 to 27 |  |
| 5850 ≤ f ≤ 5855 | 27 to 15.6 |  |
| 5855 < f ≤ 5875 | 15.6 to 10 |  |
| 5875 < f ≤ 5925 | 10 to -27 |  |
| 5925 < f | -27 |  |
| NOTE: The minimum requirement when specified as a range denotes the emission requirement at the end points of the protected range. The requirement within the protected range is obtained by linear interpolation between the requirements at the end points. |  |  |

Additional requirements for the spurious emission limit for operations using the NS_54 the TS 38.101-1 [3] says; “When "NS_53" or "NS_54" or "NS_60" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5F.3.3.5-1.”

Table 6.5F.3.3.5-1: Additional requirements

| Frequency band (MHz) | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| f ≤ 5925 | -27 | 1 MHz |
| f ≥ 7125 | -27 |  |

Based on the defined NR-ARFCN number for band n96 (Table 5.4.2.3-3 in TS 38.101-1 [3]) , the lowest number for each bandwidth using the formula

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |
| 3000 – 24250 | 15 | 3000 | 600000 | 600000 – 2016666 |

This gives center frequencies for each edge channel depending of bandwidth of the lower edge of band n96:

- 20MHz Bandwidth: 5955MHz

- 40MHz Bandwidth: 5965.02MHz

- 60MHz Bandwidth: 5974.98MHz

- 80MHz Bandwidth: 5985MHz

- 100MHz Bandwidth: 5995.02MHz

For band n96, the additional requirements are offset from the channel bandwidth edge by 20MHz. All other channels will follow the MPR backoff limits.

In the way forward from November [3], one company suggested to also check the following center frequencies:

NOTE 5:	Applicable for 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 and 6015 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6005 MHz].

For band n46, simulations are done with channels centered based on the two Notes in the A-MPR table for power class 5 for NS_30, Table 6.2F.3.4-1 in TS 38.101-1 [3]. The notes are:

NOTE 2:	Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz.

NOTE 3:	Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz.

This will be the affected channels by the additional requirements. All other channels will use the MPR table for maximum backoff.

## Simulation agreements

From the Way Forward [1], simulation agreements were

### Way Forward on PC3 A-MPR for 1Tx:

- PC3 calibration point: 1dB MPR for QPSK DFT-s-OFDM 20MHz 100RB0 waveform at 30dB ACLR

- Post PA losses 4dB

Impairment assumptions

- CIM3: 60dBc

- CIM5: 70dBc

- IQ image: -28 dB

- Carrier leakage: -28dBc

## A-MPR Simulation Results

Charter’s simulation results for NS_54 with Power Class 3, see Table 5. These simulations are with 1Tx antenna.

Figure 2: Simulation results for A-MPR PC3 for NS_54

Table 5: A-MPR for NS_54 power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) | RB Allocation (Note 3) |  | RB Allocation (Note 5) |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full/Partial | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK | See Table PC3 MPR | ≤ [2.0] | ≤ [4.5] | ≤ [1.0] | ≤ [2.0] |
|  | QPSK |  | ≤ [3.0] | ≤ [5.0] | ≤ [1.5] | ≤ [3.0] |
|  | 16 QAM |  | ≤ [3.5] | ≤ [5.0] | ≤ [2.0] | ≤ [3.0] |
|  | 64 QAM |  | ≤ [3.5] | ≤ [5.0] | ≤ [2.5] | ≤ [3.0] |
|  | 256 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [4.5] | ≤ [5.0] |
| CP-OFDM | QPSK |  | ≤ [4.5] | ≤ [6.0] | ≤ [3.0] | ≤ [4.0] |
|  | 16 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [3.0] | ≤ [4.0] |
|  | 64 QAM |  | ≤ [5.0] | ≤ [6.0] | ≤ [4.0] | ≤ [5.0] |
|  | 256 QAM |  | ≤ [6.5] | ≤ [7.0] | ≤ [6.5] | ≤ [7.0] |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for all valid channels and bandwidths other than those enumerated in NOTE 3 and NOTE 5. NOTE 3: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to [5965 MHz], 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5975 and 5995 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [5985 MHz]. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Applicable for 60 MHz channels centered at the nearest NR-ARFCN corresponding to [5995 and 6015 MHz], and 80 MHz channels centered at the nearest NR-ARFCN corresponding to [6005 MHz]. |  |  |  |  |  |  |

For NOTE 2, all the other valid channels and bandwidths other than those enumerated in NOTE 3 will follow the MPR table for Power class 3 for shared spectrum. The results of NOTE 3 show the tentative agreed numbers from last meeting [3]. Also, from WF [3], a few more center frequencies were found by one company to test and those have been simulated and the results is shown in RB Allocation (NOTE 5).

Proposal 3: Proposed A-MPR PC3 simulation results presented in Table 5 for NS_54.

Charter’s simulation results for NS_30 with Power Class 3, see Table 6. This simulation is with 1Tx antenna.

Table 6: A-MPR for NS_30 power class 3

| Pre-coding | Modulation | RB Allocation (Note 2) |  | RB Allocation (Note 3) |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| DFT-s-OFDM | PI/2 BPSK | ≤ 12.0 | ≤ 15.5 | ≤ 3.5 | ≤ 5.5 | See Table MPR table for PC3 |
|  | QPSK | ≤ 12.5 | ≤ 16.0 | ≤ 4.0 | ≤ 6.5 |  |
|  | 16 QAM | ≤ 12.5 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 64 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
|  | 256 QAM | ≤ 13.0 | ≤ 16.0 | ≤ 4.5 | ≤ 7.0 |  |
| CP-OFDM | QPSK | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 16 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 64 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.0 | ≤ 7.5 |  |
|  | 256 QAM | ≤ 14.0 | ≤ 16.5 | ≤ 6.5 | ≤ 7.5 |  |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5230 and 5270 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 2 and NOTE 3. |  |  |  |  |  |  |

Proposal 4: Proposed A-MPR PC3 simulation results presented in Table 6 for NS_30.

# Conclusion

In this paper, Charter has the following proposals:

Proposal 1: Proposed MPR PC3 simulation results presented in Table 4 for shared spectrum.

Proposal 2:  Round to the closest 0.5dB based on the mean results from companies.

Proposal 3: Proposed A-MPR PC3 simulation results presented in Table 5 for NS_54.

Proposal 4: Proposed A-MPR PC3 simulation results presented in Table 6 for NS_30.

# Reference

[1] R4-2214437, WF on NR-U PC3 requirements, Charter Communications Inc.

[2] R4-2217131, WF on NR-U PC3 requirements and MPR/A-MPR, Charter Communications Inc.

[3] R4-2220495, Way forward for NR-U PC3 MPR/A-MPR, Charter Communications Inc.

[3] TS 38.101-1: User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone, V17.6.0 (2022-06)

[4] TR 37.890, Feasibility Study on 6 GHz for LTE and NR in Licensed and Unlicensed Operations V0.16.0 (2022-06)

[5] R4-2215651, NR-U MPR and A-MPR for PC3, Apple Inc.

[6] R4-2218048, Simulation results on UE RF MPR and A-MPR for PC3, Charter Communications Inc.