3GPP TSG-RAN4 Meeting #106	R4-2300071

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

Source: 	Ligado Networks

Title: 	Updates to TS 38.101-1 related to the introduction of n54

Agenda item:	8.33.2

Document for: 	Approval

# Introduction

In RAN#98e, WI related to E-UTRA Band 54 was completed [1, 2] after discussions and agreements during RAN4#103e – RAN4#105. A new work item (WI) to introduce a corresponding NR band, n54, was also approved [3].

Based on those discussions and agreements, this document proposes updates (highlighted in yellow text) to TS 38.101-1 related to Tx and Rx requirements for the introduction of n54. The system parameters (Clause 5 of TS 38.101-1) for n54  are proposed in R4-2300072.

# Discussion

### Changes to TS 38.101-1 sub-clause 6.2.1: UE Power Class and MOP evaluation

The filter data provided in [4] can be reused to assess the Tx requirement for n54. Based on the discussion [5] and agreement [6] reached for E-UTRA band 54, the same MOP value of 23 dBm +/- 2 dB as E-UTRA band 54 can be used for n54 in sub-clause 6.2.1 of TS 38.101-1.

Table 1: UE Power Class and MOP for n54

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n54 |  |  |  |  |  |  | 23 | ±2 |

### Changes to TS 38.101-1 sub-clause 6.5.3.3: Additional emissions requirements evaluation

Based on assessment provided in [7] it was agreed to specify additional spurious emission requirements in [8] for E-UTRA band 54. The same additional spurious emissions requirements are also applicable for n54, and the following changes along with the NS value of NS_62 can be added in sub-clauses 6.5.3.3 of TS 38.101-1 for n54:

##### 6.5.3.3.28	Requirement for network signalling value “NS_62”

When "NS_62" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.28-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.28-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1625 | -70+ 66/15 (f-1610) | 1MHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using 0 dBi antenna. |  |  |  |

### Changes to TS 38.101-1 sub-clause 6.2.3.1: NS Value and A-MPR evaluation

Based on the assessment provided in [4], online discussion [5] and agreement [6] for E-UTRA band 54, like band 54, no A-MPR needs to be specified for n54. The same NS value of NS_62 can be used to signal the additional spurious emission requirements for n54 as below:

Table 6.2.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_62 | 6.5.3.3.28 | n54 | 5 |  | N/A |

Table 6.2.3.1-1A: Mapping of network signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n54 | NS_01 | NS_62 |  |  |  |  |  |  |

### Changes to sub-clause 6.5.3.2: Spurious emissions for UE co-existence

Based on the agreements for E-UTRA Band 54 [6], general spurious emissions requirement of -50 dBm/MHz for UE coexistence can be specified for n54 for the protection of legacy US bands as follows:

Table 6.5.3.2-1: Requirements for spurious emissions for UE co-existence

| NR Band | Spurious emission for UE co-existence |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| n54 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 50, 51, 53, 66, 70, 71, 85, 103 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR NTN Band n255, n256 |  |  |  |  |  |  |
|  | NR Band n77 |  |  |  |  |  |  |

Given that E-UTRA Band 54 has protection specified, no additional updates are required to protect n54.

### Changes to sub-clause 7.3.2: Reference sensitivity power level evaluation

The same NF, Rx diversity and IM as that for E-UTRA band 54 can be assumed and therefore following can be added for n54 in sub-clause 7.3.2 of TS 38.101-1:

Table 7.3.2-1b: Two antenna port reference sensitivity QPSK PREFSENS for TDD, SDL and FDD with variable duplex operation bands

| Operating band / SCS / Channel bandwidth / REFSENS |  |  |  |  |
| --- | --- | --- | --- | --- |
| Operating band | SCS kHz | Channel bandwidth (MHz) | REFSENS (dBm)8 | Duplex Mode |
| n54 | 15 | 5 | -100 | TDD |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 5 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 | Duplex Mode |
| n54 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | TDD |

Table 7.3.2-4: Network signaling value for reference sensitivity

| Operating band | Network Signalling value |
| --- | --- |
| n54 | NS_62 |

### Changes to sub-clause 7.6.2, 7.6.3 and 7.6.4: Blocking requirements evaluation

Like E-UTRA Band 54, general requirements for in-band, out of band and narrow band blocking can be specified for n54.

Table 7.6.2-2: In-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -56 | -44 | -15 | -38 | -224 |
|  | Finterferer (offset) | MHz | -BWChannel/2 – FIoffset, case 1 and BWChannel/2 + FIoffset, case 1 | ≤ -BWChannel/2 – FIoffset, case 2 and ≥ BWChannel/2 + FIoffset, case 2 |  | -BWChannel/2-11 |  |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28,n34, n38,n39, n40, n41, n483, n50, n51, n53, n54, n65, n66, n67, n70, n74, n75, n76, n85, n91, n92, n93, n94, n100, n101 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  |  |  |
| n30 | Finterferer | MHz | NOTE 2 | FDL_low – 15 to FDL_high + 15 |  | FDL_low – 11 |  |
| n71 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 | FDL_low – 12 |  |  |
| n105 | Finterferer | MHz | NOTE 2 | FDL_low – 12 to FDL_high + 15 |  |  | FDL_low – 7 |
| NOTE 1: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 2: For each carrier frequency, the requirement applies for two interferer carrier frequencies: a: -BWChannel/2 – FIoffset, case 1; b: BWChannel/2 + FIoffset, case 1 NOTE 3: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. NOTE 4: For Band n105 channels overlapping the 612 - 617 MHz frequency range, Pinterferer is modified to -34 dBm. |  |  |  |  |  |  |  |

Table 7.6.3-2: Out of-band blocking for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| NR band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
| n1, n2, n3, | Pinterferer | dBm | -44 | -30 | -15 |
| n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n485, n50, n51, n536, n54, n65, n66, n67, n70, n71, n74, n75, n76, n85, n91, n92, n93, n94, n100, n101, n105 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 6000 MHz. NOTE 2: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. For band 50, the FDL_low of band 51 is applied as FDL_low for band 50. NOTE 3: For band 76 the FDL_high of band 75 is applied as FDL_high for band 76. For band 75, the FDL_low of band 76 is applied as FDL_low for band 75. NOTE 4: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 5: n48 follows the requirement in this frequency range according to the general requirement defined in Clause 7.1. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4800 MHz. NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2580 MHz and FInterferer < 2775 MHz. NOTE 7 For UE supporting both bands 25 and 70, the FDL_high of band 70 is applied as FDL_high for band 25, and the FDL_low of band 25 is applied as FDL_low for band 70. NOTE8: For bands 91 and 93 the FDL_high of bands 92 and 94 are applied as FDL_high for bands 91 and 93. For bands 92 and 94, the FDL_low of bands 91 and 93 are applied as FDL_low for bands 92 and 94 |  |  |  |  |  |

Table 7.6.4-1: Narrow Band Blocking

| NR band | Parameter | Unit | Channel Bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5 | 10 | 15 | 20 | 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 |
| n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n24, n25, n26, n28, n30, n34, n38, n39, n40, n41, n48, n50, n51, n53, n54, n65, n66, n67, n70, n71, n74, n75, n76, n85, n100, n101, n105 | Pw | dBm |  | PREFSENS + channel-bandwidth specific value below |  |  |  |
|  |  | dB | 16 | 13 | 14 | 16 | 16 |
|  | Puw (CW) | dBm | -55 |  |  |  |  |
|  | Fuw (offset SCS= 15 kHz) 4 | MHz | BWChannel2+0.2SCS+0.5+0.5SCS |  |  |  | NA |
|  | Fuw (offset SCS= 30 kHz)4 | MHz | NA |  |  |  | BWChannel2+BWGB,ChannelSCS+0.5+0.5SCS |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in Table 7.3.2-3 with PCMAX_L,f,c defined in clause 6.2.4 NOTE 2: Reference measurement channel is specified in Annexes A.3.2 and A.3.3 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.2-1a, Table 7.3.2-1b and Table 7.3.2-2 for two and four antenna ports, respectively. NOTE 4: Fuw shall be rounded to half of SCS. |  |  |  |  |  |  |  |

A CR for TS 38.101-1 incorporating the above changes for n54 has been submitted in R4-2300072.

Proposal 1: It is proposed that the CR with above changes to TS 38.101-1 for the Introduction of n54 in R4-2300072 be agreed.

# Proposal

Proposal 1: It is proposed that the CR with above changes to TS 38.101-1 for the Introduction of n54 in R4-2300072 be agreed.

# References

[1] RP-223315, RAN4 CRs to Introduction of LTE TDD band in 1670 – 1675 MHz, RAN#98-e

[2] RP-222830, Status Report for WI: Introduction of LTE TDD band in 1670 – 1675 MHz, RAN#98-e

[3] RP-223514, New WID: Introduction of NR band in 1670 – 1675 MHz, RAN#98-e

[4] R4-2212090, Proposed key changes to TS 36.101 to support introduction of new LTE TDD Band in 1670 – 1675 MHz, RAN4#104-e

[5] R4-2214238, Email discussion summary for [104-e][127] R18_LTE_TDD_1.6GHz, RAN4#104-e

[6] R4-2214440, WF on Introduction of LTE TDD band in 1670-1675 MHz, RAN4#104-e

[7] R4-2207966, Emissions Requirements for LTE TDD band in 1670 – 1675 MHz, RAN4#103-e

[8] R4-2210572, WF on Introduction of LTE TDD band in 1670-1675 MHz, RAN4#103-e