3GPP TSG-RAN4 Meeting #106	R4-2300078

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.102 | CR | 0001 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to the additional emissions requirements related to NS_02N |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ligado Networks |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_NBIOT_eMTC_NTN_req-Core |  |  |  |  |  | Date: |  |  | 2023-02-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Updates the additional emissions ranges mask which are in square brackets and removes the square brackets |  |  |  |  |  |  |  |  |
| Summary of change: |  | Using the NB-IoT general emission mask and interpolation, guardband required to meet FCC 25.216 requirement is determined as shown in table below and the NB-IoT NTN additional spurious emission mask accordingly updated. The additional guardband is adjusted from .1 MHz to .08 MHz. ΔfOOB (kHz); Emissions (30 kHz); Emissions (4 kHz). 0; 26; 17.25. 10; 22.9; 14.15. 20; 19.8; 11.05. 30; 16.7; 7.95. 40; 13.6; 4.85. 50; 10.5; 1.75. 60; 7.4; -1.35. 70; 4.3; -4.45. 80; 1.2; -7.55. 90; -1.9; -10.65. 100; -5; -13.75. The square brackets are also removed |
| Consequences if not approved: |  | Inefficient use of the available spectrum as additional 100 kHz (due to raster spacing of 100kHz) will have to be allocated for Geo-synchronous- orbit NTNs to compensate for the doppler is non-zero but not as high as NGSOs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5B.4.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-X |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The conformance specifications number for IoT NTN is TBD |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.5B.4.4.2	Minimum requirement (network signalled value "NS_02N")

When "NS_02N" is indicated in the cell, the power of any UE spurious emission shall not exceed the levels specified in Table 6.5B.4.4.2-1 and 6.5B.4.4.2-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) specified in 6.5B.4.2 from the edge of the channel bandwidth.

Table 6.5B.4.4.2-1: Additional requirements for "NS_02N"

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 200kHz |  |  |
| 1559≤ f ≤ 1605 | -50 | 700 Hz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -50 + 24/5 (f-1605) | 700Hz |  |
| 1559 ≤ f ≤ 1605 | -40 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1605≤ f ≤ 1610 | -40 + 24/5 (f-1605) | 1MHz |  |
| NOTE: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

Table 6.5B.4.4.2-2: Additional requirements for "NS_02N"

| ΔfOOB (MHz) | Spectrum Emission Limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| ± [0.08 – 0.26] | -2 for PC3 -5 for PC5 | 4 kHz |
| ± [0.26 – 0.8] | -12 for PC3 -15 for PC5 | 4 kHz |
| ± [>0.8] | -13 for PC3 and PC5 | 4 kHz |

NOTE:	ΔfOOB = 0.08 corresponds to an authorized bandwidth, as defined in C63.26-2015 [10], of 0.36 MHz.

------------------------------------------------------------- END OF CHANGES ------------------------------------------------------