3GPP TSG-RAN WG4 Meeting # 106	R4-2300100

# Athens, Greece, February 27 – March 3, 2022

Agenda Item:	9.25.5

Source: 		Qualcomm Incorporated

Title: 			Response to incoming LS on RACH-less handover in NTN

Document for:	Approval

# 1.	Introduction

We present our views on the LS (R1-2213001) from RAN1.

# 2. 	Discussion

RAN4 is asked to confirm whether RAN1’s assumption in Note 1 of R1-2213001 is correct.

Summary of R1-2213001, Reply LS on RACH-less handover in NTN

RAN1 response: - from RAN1 perspective the RACH-less handover is possible, assuming the following notes can be satisfied, when UE UL transmission synchronization can be maintained by applying pre-compensation using the assistance information, e.g., epoch time, ephemeris, common TA, of the target cell. - (1) Intra-satellite handover with the same feeder link. i.e., with same gateway/gNB - from RAN1 perspective the RACH-less handover may be possible, assuming the following notes can be satisfied, when UE UL transmission synchronization can be maintained by applying pre-compensation using the assistance information, e.g., epoch time, ephemeris, common TA, of the target cell. - (2) Intra-satellite handover with different feeder links, i.e., with gateway/gNB switch - (3) Inter-satellite handover with gateway/gNB switch - (4) Inter-satellite handover with same gateway/gNB Note 1: RAN1 assumes that the RAN4 UL synchronization requirement specified in Table 7.1C.2-1 of TS38.133 applies to the first UL transmission in the target cell. Note 2: gNB is expected to provide valid assistance information of the target cell to UE. Note 3: gNB is expected to ensure the UE can perform the UL transmission while respecting common TA and UE processing time. To RAN4: RAN1 respectfully asks RAN4 whether RAN1’s assumption in Note 1 is correct.

As can be seen from the highlighted parts of the corresponding RAN4 requirements (excerpted below), there is not a constraint at all in terms of whether the satellite is a serving cell or a target cell. As long as NW ensures UE can be provide with a set of required satellite assistant information and feederlink information of the target cell and at least one SSB from the target cell is made available to UE during the last 160ms before the UL transmission, the requirement specified in Table 7.1C.2-1 in TS38.133 applies to any UL transmission.

Proposal 1: RAN4 to confirm that Note 1 in the LS (R1-2213001) is correct.

# 3.	Conclusion

In this paper, we proposed the following.

Proposal 1: RAN4 to confirm that Note 1 in the LS (R1-2213001) is correct.