3GPP TSG-RAN WG4 Meeting #106											         R4-2300236

Athens, Greece, Feburary 27 – March 3, 2023

Agenda item:	9.24.2.4

Source:	Apple

Title:	On R18 MUSIM - network B requirements

Document for:	Discussion

# Introduction

On R18 MUSIM - network B requirements was widely discussed in the last RAN4 meeting with latest agreements captured in [1]. According to [1], there are still some open issues. In this contribution, we continue discussion on the open issues.

# Discussion

The key issue is about whether and how to define network B requirements:

Issue 4-1-1: Whether to define network B requirements - Proposals - Option 1: Deprioritize NW B requirement at R18. (Apple xiaomi) - Option 1a: If necessary, only idle/inactive mode measurement/cell reselection requirements need to be considered. (Apple oppo) - Option 2: No measurement requirements in network B will be defined by RAN4 at R18 (vivo oppo Huawei MTK Qualcomm) - Option 3: RAN4 need to define specific UE requirements for Network B when UE is allocated MUSIM gaps (Nokia) - Option 4: RAN4 to define measurement requirement for NW-B Idle mode when no MUSIM gap collision happens. (Ericsson) - WF - Non consensus and continue discussion Issue 4-1-2: Network B requirements if it will be defined - Proposals - P1: Framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example: (Apple) - P2: If requirements for measurements in NW B are to be defined, re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP) (oppo Huawei) - P3: The UE requirements for Network B are the same as/re-use the existing UE requirements (Nokia) - WF - Non consensus and continue discussion

MUSIM gaps can be used for RRM measurement, paging reception, system information reception and so on. In general, we are fine with deprioritizing network B requirements. if necessary, RAN4 shall focus on idle/inactive mode cell reselection/measurement requirements.

P2 in the recommended WF is to re-use the existing requirements for IDLE/INACTIVE as baseline with DRX cycle replaced by max(DRX cycle, MGRP). However, we believe existing requirements cannot be directly reused. Some additional delay can be expected due to gap collision and cancellation. MUSIM gap for NW B RRM measurement sometimes will be dropped due to collision with NW A gap or other DL/UL signal. This is similar to LBT failure in NR-U design.

Therefore we have the following proposals:

Proposal 1: regarding network B requirement, consider the following alternatives:

- Alt 1: no define any network B requirement.

- Alt 2: only define requirements for network B cell reselection in idle/inative mode.

Proposal 2: if network B requirements have to be defined for cell reselection, framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example:

# Conclusion

In this contribution, we provide further discussion on R18 MUSIM - network B requirements. After discussion, the following conclusions are provided:

Proposal 1: regarding network B requirement, consider the following alternatives:

- Alt 1: no define any network B requirement.

- Alt 2: only define requirements for network B cell reselection in idle/inative mode.

Proposal 2: if network B requirements have to be defined for cell reselection, framework of idle/inactive mode RRM requirements for NR-U can be used as starting point to accommodate MUSIM gap cancellation. Take serving cell measurement as an example:

# References

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM, vivo