3GPP TSG-RAN4 Meeting #106	R4-2300263

# Athens, Greece, February 27 – March 3, 2023

Agenda item:	5.3

Source:	Apple

Title:   	Discussion on updates for R17 per-FR gap capability

Document for:	Approval

# Introduction

Regarding UE per-FR gap capability, RAN2 endorsed Alt 1.3 (More fine grained capability for Per-FR-Gaps - limited by number of carriers) as a compromise in terms of signaling overhead and simplicity.

At RAN#98, RAN approved the corresponding R17 RAN2 CRs, adding a new R17 UE capability “independentGapConfig-maxCC-r17” to inform the network that the UE supports per-FR gaps when the number of configured serving cells is less than or equal to the capability value.

In this contribution, we discuss if and how to update TS 38.133 in light of the above conclusion.

# Discussion

The new UE capability specified in 38.306 [1] is shown below:

Given the new UE capability, it can be seen that UE support of per-FR gap depends on the number of CCs configured for FR1, FR2 or both FR1 and FR2. This is different from the R15 capability “independentGapConfig,” which is independent of the number of CCs configured.

For example, in clause 8.2.1 EN-DC Interruption of 38.133, there is the following statement

“For UE which support per-FR gaps, interruptions to the PSCell or activated SCG SCells may be caused by EUTRA PCell, EUTRA SCells or SCells on the same frequency range as the victim cell.”

Without knowing the number of CCs configured, it is not clear whether the UE supports per-FR gap or not, when the UE reports “independentGapConfig-maxCC-r17.” To ensure clarity in 38.133, we propose to update 38.133.

Proposal 1: 38.133 should be updated in light of new R17 UE capability “independentGapConfig-maxCC-r17.”

There are two alternatives for the updates to 38.133:

Alt. 1: Wherever there is a mention of “UE supports per-FR gap,” some clarification is added along the lines of “UE capable or not capable of per-FR gap is based on the number of CCs configured for FR1, FR2 or both FR1 and FR2, when UE indicates support of ‘independentGapConfig-maxCC-r17’ ”

Alt. 2: In clause 3.6	“Applicability of requirements in this specification version,” provide clarification of UE supporting or not supporting per-FR gap when UE indicates support of “independentGapConfig-maxCC-r17”

Alt. 1 requires modification of every such mention and is prone to errors while Alt. 2 provides an once-and-for-all modification. Therefore, we prefer Alt. 2.

Proposal 2: It is proposed to provide clarification in clause 3.6	“Applicability of requirements in this specification version.”

A corresponding R17 CR is provided in [2].

# Conclusion

In this contribution, we make the following proposals.

Proposal 1: 38.133 should be updated in light of new R17 UE capability “independentGapConfig-maxCC-r17.”

Proposal 2: It is proposed to provide clarification in clause 3.6	“Applicability of requirements in this specification version.”

# References

- R2-2213346, “Higher granularity for per-FR gap capability [MaxCCPerFRGap],” Qualcomm

- R4-2300264, “CR on updates for R17 per-FR gap capability,” Apple