3GPP TSG-RAN4 Meeting #106	R4-2300270

# Athens, Greece, February 27 – March 3, 2023

Agenda item:	9.8.3.2

Source:	Apple

Title:   	On L1 measurements for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#105, a WF was agreed [1] that includes aspects such as group based reporting, beam sweeping factor reduction, simultaneous reception of measurement RS and data, measurement restrictions, scheduling restrictions, and sharing factor reduction. In this contribution, we further discuss those issues.

# 2	Discussion

## 2.1 Group based reporting

The WF captures the following:

Companies are encouraged to bring analysis on what requirements are needed for group based reporting to enable simultaneous reception.

In RAN1 R17 group-based reporting, the UE can report up to four beam pairs with associated L1-RSRP values. In other words, the UE chooses the reported beam pairs based on measured L1-RSRP. As far as RAN4 requirement is concerned, we would like to discuss the following aspects:

- For a pair of beams that each has good L1-RSRP, the UE may not support simultaneous reception if the two AoAs are close and can generate strong mutual inference to each other. In addition, RSRP-based beam selection may be sub-optimal in terms of achieved capacity. For example, as shown in Fig. 2, capacity-based beam selection outperforms the RSRP-based beam selection for a 100MHz channel. Therefore, it is beneficial for RAN4 to 1) Specify some conditions for UE to report, such as minimum AoA offset; 2) Consider UE reporting based on some criterion other than RSRP.

Figure 1. Comparison of RSRP-based and capacity-based beam selection

- From RAN1 specification, it is unclear if the reported beam pair is still usable after some time and how the network treats it. Should the network always configure periodic/semi-periodic group-based reporting so the UE will update the suitable beam pair to the network regularly. If so, should RAN4 specify some requirement to verify UE behavior?

Proposal 1: It is proposed to discuss if RAN4 needs to specify requirement for UE group-based beam reporting, including AoA offset, beam reporting criterion other than the one based on RSRP, and regular UE beam reporting.

## 2.2 Beam sweeping factor N

We first share our views on the issue brought up in [2].

We agree that whether the beam sweeping factor can be reduced depends on the two possible designs for L1 measurement

- Design 1: use L3 measurement to down select panels

- Design 2: not use L3 measurement to down select panels

For Design 1, after the L3 measurement, the UE is likely to fine-tune RX beams for L1 measurement. In this case, if there is only one panel that is available for L1 measurement (corresponding to the implementation option that two panels do not have overlapping coverage), N cannot be reduced following previous agreement in R15. However, for the implementation option that two panels have overlapping coverage, it is still possible for the UE to use two panels to generate two RX beams for L1-measurement and thus to reduce N.

For Design 2, it is possible to reduce N as the UE can use two panel to generate two RX beams for L1-measurement. However, how much reduction can be achieved depends further on the panel implementation. For panel implementation in Fig. 2a, N can be reduced from 8 to 4, while panel implementation in Fig. 2b, N can be reduced from 8 to 6.

- (b)

Figure 2: (a) Panels with equal capability, each generating four beams; (b) Panels with unequal capability, one generating two beams and the other generating six beams;

Observation 1: Whether and how much beam sweeping factor N can be reduced depends on UE L1 measurement strategy and on UE panel implementation.

Proposal 2: It is proposed to have a new UE capability of beam sweep factor for multi-RX capable UE. It is up to UE to report it (<8).

## 2.3 Scheduling/measurement restriction

With a possible reduced beam sweeping factor N, the current L1-RSRP measurement period can be reduced, as shown below:

On the other hand, if there is no motivation to change the L1-RSRP measurement period, the current scheduling/measurement restrictions can be relaxed. For instance, given max(TReport, ceil(M*P*N)*TSSB) for SSB based L1 measurement, the network can assume the UE will use the first ceil(M*P*N’)*TSSB for measurement, and hence the scheduling/measurement restrictions can be lifted for the remaining time, i.e. max(TReport, ceil(M*P*N)*TSSB)  - ceil(M*P*N’)*TSSB.

It is also noted that we provide more details in our companion contribution in [3].

Proposal 3: Given a reduced beam sweeping factor, if the current L1-RSRP measurement period remains unchanged, the current scheduling/measurement restrictions can be relaxed.

## 2.4 Simultaneous reception of data and L1 measurement

As we commented at last meeting, the UE capability “simultaneousReceptionDiffTypeD-r16” only indicates whether the UE supports simultaneous reception of PDSCH with different QCL Type D reference signal and does not indicate whether the UE can support simultaneous reception of data and L1 measurement. Thus, it seems that a new UE capability is needed to indicate whether the UE can support simultaneous reception of data and L1 measurement. However, we prefer to postpone the UE capability discussion until the relevant requirement nears its completion and the impact on UE implementation is clearly understood.

Proposal 4: A new UE capability should be needed to indicate whether the UE can support simultaneous reception of data and L1 measurement. However, final decisions on UE capability are postponed until the relevant requirement nears its completion and the impact on UE implementation is clearly understood.

## 2.5 Sharing Factor Reduction

Regarding sharing factor reduction, it is better to discuss if L3 and L1 measurements can be performed simultaneously. There was some down-scoping effort in the last RAN plenary regarding L3 measurement and the RAN decision was “The work on L3 measurement related aspects for scheduling/measurement restriction requirements is not precluded.” Based on this decision, it is not clear if simultaneous L3 and L1 measurements (hence the need of sharing factor) should remain in the scope of R18. On the other hand, if this remains in scope, we can discuss in which scenario L3 and L1 measurement can happen at the same time and thus the sharing factor can be reduced.

Proposal 5: It is proposed to clarify if simultaneous L3 and L1 measurements is in scope after the RAN#98 decision.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to discuss if RAN4 needs to specify requirement for UE group-based beam reporting, including AoA offset, beam reporting criterion other than the one based on RSRP, and regular UE beam reporting.

Observation 1: Whether and how much beam sweeping factor N can be reduced depends on UE L1 measurement strategy and on UE panel implementation.

Proposal 2: It is proposed to have a new UE capability of beam sweep factor for multi-RX capable UE. It is up to UE to report it (<8).

Proposal 3: Given a reduced beam sweeping factor, if the current L1-RSRP measurement period remains unchanged, the current scheduling/measurement restrictions can be relaxed.

Proposal 4: A new UE capability should be needed to indicate whether the UE can support simultaneous reception of data and L1 measurement. However, final decisions on UE capability are postponed until the relevant requirement nears its completion and the impact on UE implementation is clearly understood.

Proposal 5: It is proposed to clarify if simultaneous L3 and L1 measurements is in scope after the RAN#98 decision.

# 4	References

R4-2220743, " WF on NR FR2 multi-Rx chain DL reception – Part 2," Qualcomm

R4-2215761, "Discussion on simultaneous DL reception from different directions for L1 measurement," MediaTek

R4-2300285, "On scheduling/measurement restrictions for multiple Rx chains," Apple