3GPP TSG-RAN WG4 Meeting #106	R4-2300298

# Athens, Greece,  February 27th - March 3rd 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | 0017 | rev | <-> | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial cotrrections in the out-of-band blocking requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_solutions-Core |  |  |  |  |  | Date: |  |  | <2023-02-17> |
| Category: | <F> |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Sub-clause with out-of-band blocking requirements has several editorial mistakes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Table 7.6.3-2 caption is fixed NOTE1 in Table 7.6.3-2 is fixed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Editorial mistakes will remain in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 7.6.3	Out-of-band blocking

For NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz out-of-band band blocking is defined for an unwanted CW interfering signal falling outside a frequency range 15 MHz below or above the UE receive band.

The throughput of the wanted signal shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in 3GPP TS 38.101-1 [5] Annexes A.2.2, A.3.2 and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3-1 and Table 7.6.3-2. The relative throughput requirement shall be met for any SCS specified for the channel bandwidth of the wanted signal.

Table 7.6.3-1: Out-of-band blocking parameters for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration2 | dBm | REFSENS + 6 dB | REFSENS + 7 dB | REFSENS + 9 dB |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: Power in transmission bandwidth configuration shall be rounded to the next higher 0.5dB value. |  |  |  |  |

Table 7.6.3-2: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: void NOTE 3: void NOTE 4: void |  |  |  |  |  |

For interferer frequencies across ranges 1, 2 and 3 in Table 7.6.3-1, a maximum of

exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a step size of  min(BWchannel/2,5) MHz withthe number of resource blocks in the downlink transmission bandwidth configuration, BWChannel the bandwidth of the frequency channel in MHz and n = 1, 2, 3 for SCS = 15, 30, 60 kHz, respectively. For these exceptions, the requirements in clause 7.7 apply.