3GPP RAN WG4 Meeting #106		R4-2300336

Athens, Greece, February 27th – March 3rd  2023

Agenda item:	8.31.1

Source:	Apple

Title:	Update of the regulatory requirements and summary of NS values

WI/SI:	NR_unlic_enh

Release:	Rel-18

Document for:	Decision

# 1	Introduction

3GPP Rel-16 NR-U WI [1] specified how the NR technology can be used on the unlicensed spectrum thus offering more resources in frequency bands, such as 5GHz and 6GHz.  While 5GHz is a well-known band for the unlicensed operation, 6GHz is a relative new band usage of which has been approved recently in different regulatory regions. Since not all the countries and regions were addressed during the corresponding Rel-16 and Rel-17 WI [2], RAN#96 meeting approved a new WI [3] with the main motivation of enabling LPI and VLP in countries not covered by previous efforts.

During the previous RAN WG4 meetings we provided overview of the 6GHz regulatory rules for the LPI and VLP modes identifying which existing NS values can be reused and which new NS values are still needed. After the TSG RAN#98 meeting another regulatory updated was submitted for TR 37.890 covering new countries, such as Kenya, Qatar, Jordan and Dominican Republic (see Annex A for the summary of the corresponding rules). Thus, in this discussion paper we present our further considerations on which NS values we need to support these countries.

# 2	Overview of the 6GHz regulatory rules

As mentioned in the Introduction part, there was a regulatory status update submitted to the latest TSG RAN meeting containing new countries. Here we provide a more detailed overview of new requirements with a summary of which NS flags can be re-used.

-	Kenya. This country allows both LPI and VLP modes in 5925-6425MHz frequency range, with the maximum Tx power of 23dBm and 14dBm; and the PSD restrictions are 10dBm/MHz and 1dBm/MHz.

-	Qatar and Jordan. These countries regulations are identical to Kenya in terms of the maximum Tx power, but there are no PSD restrictions at all.

-	Dominican Republic. This region 2 country also allows both LPI and VLP modes, which can be used in the whole frequency range of 5925-7125MHz. The LPI mode restrictions for the UE side are 24dBm max Tx power with 2dBm/MHz PSD, and the VLP mode can be used with 14dBm max Tx power and -8dBm/MHz PSD.

As can be seen from the summary, countries such as Kenya, Qatar and Jordan can be supported with the existing 3GPP band n102. Since Qatar and Jordan have only max Tx power restrictions with no PSD limitations, similar to e.g. Morocco, existing NS flags can be re-used. More precisely, NS_01 flag for the LPI mode and "flat 6dB back-off" flag for the VLP mode with PC5. As for Kenya, its LPI mode can be also supported with NS_01, while the VLP mode can be supported with the same NS flag as for Australia and New Zealand (which allows slightly higher back-off than just 6dB).

Dominican Republic allows the LPI and VLP modes in the full range and thus can be supported with band n96. The regulatory requirements for the UE side operating the LPI mode are identical to South Korea, thus we can potentially re-use NS_60. The VLP mode requirements are identical to Canada except the fact that unlike Canada, Dominican Republic has out-of-band emission requirements outside the operational range. On the other hand, according to the VLP A-MPR simulation results the limiting fact in this is not out-of-band emission requirements, but rather the PSD limit of -8dBm/MHz [6]. Thus, the same NS flag can be shared between these countries.

Proposal 1a:	Re-use 3GPP band n102 for the LPI/VLP operation in Kenya, Qatar and Jordan.

Proposal 1b:	For the LPI operation in Kenya, Qatar and Jordan NS_01 can be re-used.

Proposal 1c:	For the VLP operation in Qatar and Jordan, the corresponding "flat 6dB back-off" NS flag can be re-used.

Proposal 1d:	For the VLP operation in Kenya, same NS flag as for e.g. Australia and New Zealand can be re-used.

Proposal 2a:	Re-use 3GPP band n96 for the LPI/VLP operation in Dominican Republic.

Proposal 2b:	For the LPI operation in Dominican Republic, same NS flag as for South Korea, NS_60, can be re-used.

Proposal 2c:	For the VLP operation in Dominican Republic, same NS flag as for e.g. Canada can be re-used.

During the previous RAN WG4 meeting a preliminary agreement was made that the LPI mode in Japan can be supported with the existing NS_01 flag and for the VLP same flag as for e.g. Morocco and UK can be re-used. However, a deeper analysis of the regulatory requirements with additional A-MPR simulations revealed the need for having separate NS flags for Japan. The corresponding A-MPR results for Japan can be found in [5] and [6].

Proposal 2d:	Japan needs separate NS flags to support the LPI and VLP modes.

Based on the presented considerations, an updated summary of NS values is presented in the Table 2-1 below, where the highlighted parts are new countries and NS values. Same changes are also captured in draft CR for TR 38.849 [4].

Table 2-1: Updated NS values for PC5.

| Country | Mode |  |  |
| --- | --- | --- | --- |
|  | SP | LPI | VLP |
| Region 1 |  |  |  |
| EU/CEPT | N/A | NS_58 | [NS_y1] |
| UK | N/A | NS_01 | [NS_y2] |
| Morocco | N/A | NS_01 | [NS_y2] |
| UAE | N/A | NS_01 | N/A |
| Saudi Arabia | N/A | NS_01 | N/A |
| Kenya | N/A | NS_01 | [NS_y5] |
| Qatar | N/A | NS_01 | [NS_y2] |
| Jordan | N/A | NS_01 | [NS_y2] |
| Region 2 |  |  |  |
| US | NS_54 | NS_53 | N/A |
| Canada | NS_54 | NS_59 | [NS_y3] |
| Brazil | N/A | NS_53 | [NS_y4] |
| Peru | N/A | NS_53 | N/A |
| Chile | N/A | NS_53 | N/A |
| Costa Rica | N/A | NS_01 | [NS_y2] |
| Colombia | N/A | NS_53 | N/A |
| Dominican Republic | N/A | [NS_60] | [NS_y3] |
| Region 3 |  |  |  |
| South Korea | N/A | NS_60 | [NS_61] |
| Hong Kong | N/A | NS_58 | [NS_y1] |
| Australia | N/A | NS_01 | [NS_y5] |
| New Zealand | N/A | NS_01 | [NS_y5] |
| Malaysia | N/A | NS_01 | [NS_y2] |
| Japan | N/A | [NS_x1] | [NS_y6] |

Proposal 3:	Endorse an updated summary of NS values for PC5 (subject for further changes).

# 3	Conclusions

In this contribution we have presented summary of the regulatory requirements of the 6GHz band LPI and VLP modes for the countries that were not addressed during the previous meetings. The document also presented the updated summary of the NS values, which can be found in the draft CR for TR 38.849 with the associated A-MPR values [4].

Proposal 1a:	Re-use 3GPP band n102 for the LPI/VLP operation in Kenya, Qatar and Jordan.

Proposal 1b:	For the LPI operation in Kenya, Qatar and Jordan NS_01 can be re-used.

Proposal 1c:	For the VLP operation in Qatar and Jordan, the corresponding "flat 6dB back-off" NS flag can be re-used.

Proposal 1d:	For the VLP operation in Kenya, same NS flag as for e.g. Australia and New Zealand can be re-used.

Proposal 2a:	Re-use 3GPP band n96 for the LPI/VLP operation in Dominican Republic.

Proposal 2b:	For the LPI operation in Dominican Republic, same NS flag as for South Korea, NS_60, can be re-used.

Proposal 2c:	For the VLP operation in Dominican Republic, same NS flag as for e.g. Canada can be re-used.

Proposal 2d:	Japan needs separate NS flags to support the LPI and VLP modes.

Proposal 3:	Endorse an updated summary of NS values for PC5 (subject for further changes).

# 4	References

- RP-1912926, "WID on NR-based Access to Unlicensed Spectrum ", Qualcomm Inc.

- RP-213180, "Revised WID for Introduction of operation in full unlicensed band 5925-7125MHz", Apple Inc.

- RP-221813, "New WID: Introduction of evolved shared spectrum bands", Apple Inc.

- R4-2300338, "Introduction of new countries with associated NS values and A-MPR back-off", Apple Inc.

- R4-2300344, "NR-U MPR for Wideband and A-MPR for LPI", Apple Inc.

- R4-2300345, "NR-U A-MPR for VLP", Apple Inc.

Annex A: Summary of the regulatory requirements

| Region | Country | Permissible operation (Note 1) | Frequency range | Maximum mean EIRP for in-band emissions | Maximum mean EIRP density for in-band emissions | Maximum mean EIRP density for out-of-band emissions |
| --- | --- | --- | --- | --- | --- | --- |
| Region 1 | EU/CEPT | LPI (see 4.1.1) | 5945 – 6425MHz | 23dBm | 10dBm/MHz | -22 dBm/MHz (below 5935MHz) |
|  |  | VLP (see 4.1.1) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) | -45 dBm/MHz (below 5935MHz); |
|  | UK | LPI (see 4.1.3) | 5925 – 6425MHz | 24dBm | 11dBm/MHz | In accordance with directive 2014/53/EC |
|  |  | VLP (see 4.1.3) |  | 14dBm |  |  |
|  | UAE | LPI (see 4.1.5) | 5925 – 6425MHz | 24 dBm |  |  |
|  | Morocco | LPI (see 4.1.7) | 5925 – 6425MHz | 23 dBm |  |  |
|  |  | VLP (see 4.1.7) |  | 14 dBm |  |  |
|  | Saudi Arabia | LPI (see 4.1.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 10dBm/MHz |  |
|  | Kenya | LPI (see 4.1.8) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.1.8) |  | 14dBm | 1dBm/MHz |  |
|  | Qatar | LPI (see 4.1.9) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.1.9) |  | 14dBm |  |  |
|  | Jordan | LPI (see 4.1.10) | 5925 – 6425MHz | 23dBm |  |  |
|  |  | VLP (see 4.1.10) |  | 14dBm |  |  |
| Region 2 | US | SP (see 4.2.1) | 5925 – 6425MHz 6525 – 6875MHz | 36dBm (AP) 30dBm (CL) | 23dBm/MHz (AP) 17dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | LPI (see 4.2.1) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5 dBm/MHz (AP) -1 dBm/MHz (CL) |  |
|  | Canada | SP (see 4.2.2) | 5925-6875 MHz | 36dBm | 23dBm/MHz |  |
|  |  | LPI (see 4.2.2) | 5925-7125 MHz | 30dBm | 5 dBm/MHz |  |
|  |  | VLP (see 4.2.2) |  | 14dBm | -8dBm/MHz |  |
|  | Brazil | LPI (see 4.2.3) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.3) |  | 17 dBm | -5 dBm/MHz |  |
|  | Peru | LPI (see 4.2.4) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  | Chile | LPI (see 4.2.5) | 5925 – 6425MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) |  |
|  |  | VLP (4.2.5) |  | 17 dBm |  |  |
|  | Costa Rica | LPI (see 4.2.8) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) |  |  |
|  |  | VLP (see 4.2.8) |  | 14dBm |  |  |
|  | Colombia | LPI (see 4.2.9) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 5dBm/MHz (AP) -1dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  | Dominican Republic | LPI (see 4.2.10) | 5925 – 7125MHz | 30dBm (AP) 24dBm (CL) | 8dBm/MHz (AP) 2dBm/MHz (CL) | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.2.10) |  | 14dBm | -8dBm/MHz |  |
| Region 3 | South Korea | LPI (see 4.3.2) | 5925 – 7125MHz | 24dBm | 2dBm/MHz | -27 dBm/MHz (outside operational range) |
|  |  | VLP (see 4.3.2) | 5925 – 6425MHz | 14dBm | 1dBm/MHz | -34 dBm/MHz (outside operational range of the VLP mode) |
|  | Hong Kong | LPI (see 4.3.3) | 5925 – 6425MHz | 23dBm | 10dBm/MHz | In accordance with ETSI EN 303 687 |
|  |  | VLP (see 4.3.3) |  | 14dBm | 1dBm/MHz |  |
|  | Australia | LPI (see 4.3.4) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.4) |  | 14dBm | 1dBm/MHz |  |
|  | New Zealand | LPI (see 4.3.5) | 5925 – 6425MHz | 24dBm | 11dBm/MHz |  |
|  |  | VLP (see 4.3.5) |  | 14dBm | 1dBm/MHz |  |
|  | Japan | LPI (see 4.3.6) | 5925 – 6425MHz | 23dBm |  | -27dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |
|  |  | VLP (see 4.3.6) |  | 14dBm |  | -37dBm/MHz (below 5925MHz) -13dBm/MHz and -19dBm/MHz (above 6425MHz) |
|  | Malaysia | LPI (see 4.3.7) | 5925 – 6425MHz | 23dBm | 10dBm/MHz |  |
|  |  | VLP (see 4.3.7) |  | 14dBm | 1dBm/MHz 10dBm/MHz (for the narrowband usage) |  |