3GPP TSG-RAN4 Meeting #106	R4-2300340

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

Agenda item:	4.1

Source:	Apple

Title:	On issues with edge channels for CA_NS_10

Release:	Rel-16

Document for:	Approval

# 1  Introduction

The requirements for CA_NS_10 flag were originally introduced during RAN#90 in [1]. The content is based on the original contribution [2] which proposed A-MPR region definitions for contiguous and non-contiguous carrier aggregation with CA_48C. During the original RAN4 meeting the proposals were tweaked for improved uplink performance. In this paper we want to outline a potential issue with the implemented equations for the A-MPR regions and provide a solution.

# 2  Discussion

The network signalling flag CA_NS_10 was implemented with the big CR [1]. The content for CA_48C is based on [2]. Simulations results for CA_48C and the corresponding region definition and A-MPR values were provided with this contribution. When the A-MPR regions were implemented in TS36.101 the equations were slightly tweaked compared to the original proposal.

We found that there exists a potential error with the implemented region definitions so that not all edge channels obtain A-MPR but instead need to rely on MPR alone. More specifically, we observed that some CA combinations located at the upper edge of the band do not obtain any A-MPR while those are in most need of additional power back-off due to the near proximity to the out-of-band emission requirements. For illustration the non-contiguous resource allocation located in clause 6.2.4A.10 is shown below. The troubling portion of the equations are highlighted with red colour.

Table 6.2.4A.10-4: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC< FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC < FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

To lay out the issue with the highlighted parts of the region definitions, the following tables provide calculations of various parameters such as channel spacing, BACA, the carrier centre frequency FC and the outermost centre frequencies for lower and upper band edge.

Table 1: Channel spacing and Aggregated Channel Bandwidth (BWCA)

| Channel Bandwidth, MHz | Foffset, low, MHz | Foffset, high, MHz | Channel Spacing, MHz | BWCA, MHz |
| --- | --- | --- | --- | --- |
| 5 + 20 | 3.25 | 10 | 11.7 | 24.95 |
| 10 + 20 | 5.50 | 10 | 14.4 | 29.90 |
| 15 + 20 | 7.75 | 10 | 17.1 | 34.85 |
| 20 + 20 | 10 | 10 | 19.8 | 39.80 |
| 20 + 5 | 10 | 3.25 | 11.7 | 24.95 |
| 20 + 10 | 10 | 5.50 | 14.4 | 29.90 |
| 20 + 15 | 10 | 7.75 | 17.1 | 34.85 |

Table 3: CC-Combo at lower band edge.

The frequencies ful/dl are obtained from Table 4.3.1.2.16A-1 in TS36.508

| CA Configuration, MHz | CC1 ful/dl, low | CC2 ful/dl, low | Low Edge: FL + BWCA/2 | Condition | FC, low MHz |
| --- | --- | --- | --- | --- | --- |
| 5 + 20 | 3553.3 | 3565.0 | 3562.475 | FL + BWCA/2 ≤ FC | 3562.525 |
| 10 + 20 | 3555.5 | 3569.9 | 3564.950 | FL + BWCA/2 ≤ FC | 3564.950 |
| 15 + 20 | 3557.8 | 3574.9 | 3567.425 | FL + BWCA/2 ≤ FC | 3567.475 |
| 20 + 20 | 3560.0 | 3579.8 | 3569.900 | FL + BWCA/2 ≤ FC | 3569.900 |
| 20 + 5 | 3560.0 | 3571.7 | 3562.475 | FL + BWCA/2 ≤ FC | 3562.475 |
| 20 + 10 | 3560.0 | 3574.4 | 3564.950 | FL + BWCA/2 ≤ FC | 3564.950 |
| 20 + 15 | 3560.0 | 3577.1 | 3567.425 | FL + BWCA/2 ≤ FC | 3567.425 |

Table 4: CC-Combo at upper band edge.

The frequencies ful/dl are obtained from Table 4.3.1.2.16A-1 in TS36.508

| CA Configuration, MHz | CC1 ful/dl, high | CC2 ful/dl, high | High Edge: FH - BWCA/2 | Condition | FC, high MHz |
| --- | --- | --- | --- | --- | --- |
| 5 + 20 | 3678.3 | 3690.0 | 3687.525 | FC ≤ FH - BWCA/2 | 3687.525 |
| 10 + 20 | 3675.6 | 3690.0 | 3685.050 | FC < FH - BWCA/2 | 3685.050 |
| 15 + 20 | 3672.9 | 3690.0 | 3682.575 | FC < FH - BWCA/2 | 3682.575 |
| 20 + 20 | 3670.2 | 3690.0 | 3680.100 | FC < FH - BWCA/2 | 3680.100 |
| 20 + 5 | 3685.0 | 3696.7 | 3687.525 | FC < FH - BWCA/2 | 3687.475 |
| 20 + 10 | 3680.1 | 3694.5 | 3685.050 | FC < FH - BWCA/2 | 3685.050 |
| 20 + 15 | 3675.1 | 3692.2 | 3682.575 | FC < FH - BWCA/2 | 3682.525 |

It can be observed that for all combinations the lower border allows to place the combinations at lowest possible raster point. However, the situation is different for the upper band edge. Here exist several combinations where FC is exactly located at ‘FH - BWCA/2’. For the combinations ‘10 + 20’, ‘20 + 10’, ‘15 + 20’ and ‘20 + 20’ the carrier centre frequency is not included due to the use of ‘<’.

If the equation ‘≤’ would be used, then those specific cases would be part of the A-MPR region. In the light of the original contribution [2] which proposed ‘≤’ for the upper edge, it seems to be an oversight that ‘≤’ is not used for all upper edge equations. We would like to propose to solve this issue by replacing ‘<’ with ‘≤’ as done in the companion CR [3].

Observation 1: If calculating the carrier centre frequency FC for CA configurations directly bordering the upper band edge it can be shown that certain configurations do not receive any A-MPR allowance. This arises from the equations excluding the upper portion of the band down to exactly the critical carrier centre frequency FC.

Observation 2: The original contribution discussing A-MPR and regions uses ‘≤’ instead of ‘<’ for all upper band edge regions.

Proposal: To resolve the issue of channels located at upper band edge not receiving any A-MPR allowance it is proposed to update the equations for A-MPR regions to use ‘≤’ instead of ‘<’.

# 3  Conclusions

This contribution describes an oversight found with the CA_48C A-MPR region definitions. The following observations and proposals are made:

Observation 1: If calculating the carrier centre frequency FC for CA configurations directly bordering the upper band edge it can be shown that certain configurations do not receive any A-MPR allowance. This arises from the equations excluding the upper portion of the band down to exactly the critical carrier centre frequency FC.

Observation 2: The original contribution discussing A-MPR and regions uses ‘≤’ instead of ‘<’ for all upper band edge regions.

Proposal: To resolve the issue of channels located at upper band edge not receiving any A-MPR allowance it is proposed to update the equations for A-MPR regions to use ‘≤’ instead of ‘<’.

# 4  References

- R4-1901424, “CR to add LTE Intra-band band combinations in TS 36.101”, Ericsson, 3GPP TSG-RAN WG4 Meeting #90

- R4-1902144, “TP to TR 36.716-01-01: A-MPR for CA_48C, NR SUL and LTE/NR DC in Rel-18”, Nokia, 3GPP TSG-RAN WG4 Meeting #90

- R4-2300333, “CR for TS 36.101 Rel-16 CAT-F: Corrections on CA_NS_10”, Apple, 3GPP TSG-RAN WG4 Meeting #106