3GPP TSG-RAN4 Meeting #106	R4-2300342

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

Agenda item:	9.26.2.4

Source:	Apple

Title:	Simulation results for non-transparent MPR reduction schemes

Release:	Rel-18

Document for:	Approval

# 1  Introduction

The WI [1] on NR coverage enhancements aims to reduce MPR/PAR through diverse techniques. The current objective is to analyze the performance increase of frequency domain spectrum shaping with and without spectrum extension. This contribution provides simulation results for FR1. Results are provided for the case of 25% spectrum extension. Additionally, zero spectrum extension is provided for comparison.

# 2  Discussion

2.1 Simulation setup

The simulations were done with the following setup. Details on the individual simulations are found in the specific sections.

Table1: Simulation setup

| Channel BW | 20, 50, 100 MHz |
| --- | --- |
| Modulation | QPSK |
| SCS | 15, 30 kHz |
| Waveform | DFT-S-OFDM |
| DMRS config | ZC, 2 symbols |
| Extension factors | 0, 0.25 |
| EVM | QPSK: 14.2% (The simulations do not include all impairments of the Tx chain such as e.g. transmitter chain non-linearity and phase noise. Therefore, a reduced EVM budget is used mainly covering the PA and IQ impairments.) |
| Channel | PUSCH |
| Spectral shaping filter | - 3-tap, FD implementation - (0.335 1 0.335) - (0.28 1 0.28) - 2-tap: (1, 0.28) for FDSS QPSK w/o SE - No filter (reference case) |
| Power class | PC3 |
| Calibration | 1dB MPR: DFT-s-OFDM QPSK 20MHz, 100RB |
| Carrier Leakage, Image, CIM3, CIM5 | 28dBc, 28dBc, 60dBc, 70dBc |

In case of spectrum extension, the EVM is calculated by only deploying the in-band portion of the signal while the extension is ignored. The IBE and equalizer ripple requirements consider the whole waveform and therefore include the extension part.

At high output power the spectral domain shaped waveform is typically EVM or IBE limited. It is important to consider that the simulations typically do not include all impairments of the transmit chain. The main impairments are power amplifier non-linearity and IQ image. Other aspects such as transmitter chain non-linearity and phase noise for sub-6 GHz are typically not modelled and certain headroom needs to be left for those additional factors. Incorporating this aspect, a reduced EVM limit is used as specified in Table 1.

2.2 Considerations on net-gain with spectrum extension

The spectrum extension creates excess power which might not be utilised by a conventional receiver (which only considers the in-band portion of the signal) and therefore reduces the perceived power at the receiver. The power loss is strongly dependent on shaping filter and the extension factor as displayed in Figure 1.  This effect might need to be reflected in net-gain analysis. No loss would be expected if the base station uses the extension during demodulation process. For the net-gain equation with spectrum extension the assumption for demodulation should be clarified to determine whether the base station is expected to utilise the extension for improving receiver performance or not.

Figure 1: Power loss at conventional receiver not including spectrum extension for demodulation

Observation 1: The spectrum extension creates excess power which might not be utilised by a conventional receiver (which only considers the in-band portion of the signal) and therefore reduces the perceived power at the receiver. The power loss is strongly dependent on shaping filter and extension factor and might need to be reflected in net-gain analysis. No loss would be expected if the base station uses the extension during demodulation process.

Proposal: For the net-gain equation with spectrum extension the assumption for demodulation should be clarified to determine whether the base station is expected to utilise the extension for improving receiver performance or not.

2.3 Results for FDSS with spectrum extension

In this section the results for FDSS with the use of spectrum extension are provided. The results are shown for PRBs of 16, 32, 64 and 96. The PRBs represent the sum of Inband and Extension. The spectrum extension is set to 0.25%. The results are compared with the OBO with zero extension. The use of spectrum extension improves the power output capability for the same filter type. The spectral shaping filter with extension perform similar for small PRBs with better performance for [0.335 1 0.355] at the channel edges. For larger PRBs the filter [0.335 1 0.355] can performance 0.2-0.4 dB better.

Fig.2: OBO for 20MHz CBW and 15kHz SCS

Fig.3: OBO for 50MHz CBW and 30kHz SCS

Fig.4: OBO for 100MHz CBW and 30kHz SCS

Observation 2: Spectrum extension with 25% provides a clear performance boost for OBO compared to zero spectrum extension. Performance boost for edge allocations might be limited in some cases.

Observation 3: For large allocations the more aggressive filter displays an advantage over the less aggressive filter. For smaller allocations both filters offer almost the same performance.

# Conclusions

This contribution provides several simulation results for coverage enhancement with the use of spectrum shaping and spectrum extension. The following observations and proposals are made:

Observation 1: The spectrum extension creates excess power which might not be utilised by a conventional receiver (which only considers the in-band portion of the signal) and therefore reduces the perceived power at the receiver. The power loss is strongly dependent on shaping filter and extension factor and might need to be reflected in net-gain analysis. No loss would be expected if the base station uses the extension during demodulation process.

Proposal: For the net-gain equation with spectrum extension the assumption for demodulation should be clarified to determine whether the base station is expected to utilise the extension for improving receiver performance or not.

Observation 2: Spectrum extension with 25% provides a clear performance boost for OBO compared to zero spectrum extension. Performance boost for edge allocations might be limited in some cases.

Observation 3: For large allocations the more aggressive filter displays an advantage over the less aggressive filter. For smaller allocations both filters offer almost the same performance.

# 4  References

- RP-213579, “New WI: Further NR coverage enhancements”, China Telecom

- R4-2217745, “WF on enhancements to reduce MPR/PAR”, Nokia, RAN4#104-bis-e

- R4-2220842, “LS on RF simulation parameters to be used in RAN4 for power domain enhancements”, Nokia, RAN4#105