3GPP TSG-RAN WG4 Meeting#106      	R4-2300428

Athens Greece, 27 Feb – 3 Mar, 2023

Title:	UE RF specification impact due to dedicated spectrum less than 5MHz for FR1

Agenda item:	8.14.3

Source:	Nokia

Document for:	Approval

# 1	Background

In this contribution we discuss the necessary changes into 38.101-1 due to WI: NR support for dedicated spectrum less than 5MHz for FR1 [1]

# 2 	Discussion

In Table 1 we go trough all UE Tx and Rx requirements one by one and asses which requirements need modification with introduction of 3 MHz NR channel bandwidth. We also present as reference how LTE specification treats 3 MHz channel bandwidth.

Table 1: Specification impact analysis

| NR UE Tx/Rx requirement | Current LTE specification for 3 MHz | Proposal for NR 3 MHz |
| --- | --- | --- |
| 6.2.1 UE maximum output power | Channel bandwidth agnostic | No specification impact |
| 6.2.2 MPR | Dedicated MPR for 3 MHz | NR MPR is channel bandwidth agnostic --> no specification impact |
| 6.2.3 A-MPR | Can be channel bandwidth dependant, B26 and B28 has A-MPR. | Has specification impact to some bands. Considering bands in this WI n8, n26 and n28 needs A-MPR study. |
| 6.2.4 Configured transmitted power | Channel bandwidth agnostic | No specification impact |
| 6.3.1 Minimum output power | Channel bandwidth agnostic, -40 dBm. | Is channel bandwidth dependent, specification change needed but re-using -40 dBm can be considered. |
| 6.3.2 Transmit OFF power | Channel bandwidth agnostic, -50 dBm. | Value - 50 dBm is channel bandwidth agnostic --> no specification impact |
| 6.3.3 Transmit ON/OFF time mask | Channel bandwidth agnostic | No specification impact |
| 6.3.4 Power control | Channel bandwidth agnostic | No specification impact |
| 6.4.1 Frequency error | Channel bandwidth agnostic | No specification impact |
| 6.4.2.1 EVM | Channel bandwidth agnostic | No specification impact |
| 6.4.2.2 Carrier leakage | Channel bandwidth agnostic | No specification impact |
| 6.4.2.3 In-band emissions | Channel bandwidth agnostic | No specification impact |
| 6.4.2.4 EVM equalizer spectrum flatness | Channel bandwidth agnostic | No specification impact |
| 6.5.1 Occupied bandwidth | Channel bandwidth dependent | Specification impact, addition of 3 MHz into Table 6.5.1-1 |
| 6.5.2.2 Spectrum emission mask | Dedicated general SEM for 3 MHz | Specification impact, Dedicated general SEM for 3 MHz needed. |
| 6.5.2.3 Additional Spectrum emission mask | Needed for some bands | Not needed for bands n8, n26, n28 or n100 --> no specification impact |
| 6.5.2.4.1 NR ACLR | ACLR measurement bandwidth is channel bandwidth dependent | Specification impact, addition of 3 MHz into Table 6.5.2.4.1-1 |
| 6.5.2.4.2 UTRA ACLR | Measurement bandwidth is channel bandwidth dependent for some bands | Channel bandwidth agnostic for bands which is applicable --> no specification impact |
| 6.5.3.1 General spurious emissions | Boundary between E-UTRA out of band and spurious emission domain is channel bandwidth dependent | Boundary between E-UTRA out of band and spurious emission domain is defined generally --> no specification impact |
| 6.5.3.2 Spurious UEtoUE co-ex | Channel bandwidth agnostic | No specification impact |
| 6.5.3.3 Additional spurious emissions | Requirement value per band is channel bandwidth agnostic even though channel bandwidth is mentioned in tables, B26 and B28 have dedicated requirement but B28 does not have A-MPR for 3 MHz | n8, n26, n28 has additional spurious emissions, 3 MHz need to be listed --> specification impact |
| 6.5.4 Transmit intermodulation | Measurement bandwidth is channel bandwidth dependent | Channel bandwidth agnostic --> no specification impact |
| 7.3.2 Reference sensitivity | Channel bandwidth dependent | Channel bandwidth dependent, if SU is same as LTE then LTE 3 MHz REFSENS can be reused --> specification impact |
| 7.4 Max input level | Channel bandwidth agnostic | Specification impact, addition of 3 MHz into Table 7.4-1 |
| 7.5 Adjacent channel selectivity | Channel bandwidth dependent, 33dBc for 3 MHz | Specification impact --> 33 dBc can be reused, 3 MHz is added into Table 7.5-1 |
| 7.6.2 In-band blocking | Channel bandwidth dependent | Specification impact --> 5 MHz requirement can be used as a starting point for 3 MHz which is added into Table 7.6.2-1 |
| 7.6.3 Out of band blocking | Channel bandwidth dependent | Specification impact --> 5 MHz requirement can be used as a starting point for 3 MHz which is added into Table 7.6.3-1 |
| 7.6.4 Narrow band blocking | Channel bandwidth dependent | Specification impact --> 5 MHz requirement can be used as a starting point for 3 MHz which is added into Table 7.6.4-1 |
| Spurious response | Channel bandwidth dependent | Specification impact --> 5 MHz requirement can be used as a starting point for 3 MHz which is added into Table 7.7-1 |
| 7.7 Intermodulation characteristics | Channel bandwidth dependent | Specification impact --> 5 MHz requirement can be used for 3 MHz which is added into Table 7.8.2-1 |

As a conclusion on table 1 we can note that most of the technical work is concentrating on defining the A-MPR for 3 MHz BW for bands n8, n 26 and n28. Reference sensitivity can be defined after SU is agreed and is either re-used from LTE if SU is same as LTE or scaled from 5 MHz if SU is different from LTE. Otherwise, specification impacts are quite trivial.

# 3	Conclusion

In this contribution we have discussed the necessary changes into 38.101-1 due to WI NR support for dedicated spectrum less than 5MHz for FR1.

As a conclusion on table 1 we can note that most of the technical work is concentrating on defining the A-MPR for 3 MHz BW for bands n8, n 26 and n28. Reference sensitivity can be defined after SU is agreed and is either re-used from LTE if SU is same as LTE or scaled from 5 MHz if SU is different from LTE. Otherwise, specification impacts are quite trivial.

Proposal: Table 1 used to identify the UE RF requirements that need attention when 3 MHz is introduced into specifications.

# 4	References

- [1] RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, RAN#95e