3GPP TSG-RAN WG4 Meeting # 106                             	                                  R4-2300485

# Athens, Greece, February 27 – March 3, 2023

Agenda item:	9.29.3

Source:	Intel Corporation

Title:	Discussion on the scope of RRM requirement specifications in sidelink evolution

Document for:	Discussion

- Introduction

The latest RAN #99e updates WID [1] and provides the scope of Rel-18 sidelink evolutionas below. From RAN4 RRM perspectives, sidelink on unlicensed spectrum and co-channel coexistence for LTE sidelink and NR sidelink would be the main topic in RRM requirement specifications.

1. Specify mechanism to support NR sidelink CA operation based on LTE sidelink CA operation [RAN2, RAN1, RAN4] (This part of the work is put on hold until further checking in RAN#99) 2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - Focus on FR1 unlicensed bands (n46 and n96/n102). - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel. 3. Study enhanced sidelink operation on FR2 licensed spectrum [RAN1, RAN2] 4. Study and specify, if necessary, mechanism(s) for co-channel coexistence for LTE sidelink and NR sidelink including performance, necessity, feasibility, and potential specification impact if any [RAN1, RAN2, RAN4] - Reuse the in-device coexistence framework defined in Rel-16 as much as possible - Note, RAN1 continues the work on dynamic resource pool sharing based on existing agreements and WID with high priority for Type A devices and operating combination A 5. UE Tx and Rx RF requirement for supporting new features introduced in this WI, sidelink frequency bands for single-carrier operation and frequency band combinations for carrier aggregation operation [RAN4] 6. UE RRM core requirement for the new features introduced in this WI [RAN4]

Fig. 1 NR Sildelink Unlicensed (SL-U)                        Fig. 2 LTE V2X-NR V2X Coexistence

- This paper discusses the expected scope of RRM specification for sidelink evoulation and provides our views.

- Discussion

## Introduction

### V2X Requirements

The RRM requirements for NR sidelink operation in licensed band are captured in Sec. 12 V2X Requirements [2]. The main topics are related with reference synchronization source management, SLSS related transmission and measurement, congestion control measurement, interruption requirement for sidelink resource control and scheduling availability.

12. V2X Requirement. 12.1 Introduction. 12.2 UE transmit timing. - Ref. source: GNSS, NR Cell, E-UTRAN Cell, SyncRef UE. 12.3 Initiation/Cease of SLSS Transmission. - Ref. source: GNSS, NR Cell, E-UTRAN Cell, SyncRef UE. 12.4 Selection / Reselection of V2X Synchronization Reference Source. - Side conditions: SS-RSRP, V2X data drop rate / # of drop slots, .... 12.5 L1 SL-RSRP measurements. - Measurement accuracy. 12.6 Congestion Control measurements. 12.7 Interruption. 12.7.1 Interruptions to WAN due to V2X Sidelink Communication. 12.7.2 V2X Sidelink Communcation Dropping due to synchronization source change. 12.7.3 Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink. 12.7.4 Interruptions to WAN at transitions between active and non-active during SL-DRX. 12.7.5 Interruptions to V2X at transitions between active and non-active during DRX. 12.7.6 Interruptions to V2X sidelink due to Active BWP switching Requirements. 12.7.7 Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration. 12.7.8 Interruptions at NR sidelink discovery configuration. 12.8 Reliability of GNSS signal. 12.9 Scheduling availability. 12.9.1 Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink. 12.9.2 Scheduling availability of UE switching between Uu uplink and V2X sidelink. 12.10 Selection / Reselection of relay UE.

### Requirements in unlicensed band

The RRM requirements for NR-U operation is described under consideration of Clear Channel Assessment (CCA) of the serving cell involved. Most of changes in RRM procedures under CCA are resulted from channel unavaliabity due to LBT failure, and the resulting relaxations are specified in the relevant RRM core requirements. With the same analogy, when sidelink is configured in unlicensed band, there may be changes in existing V2X RRM requirement due to the following aspects. The OC/PSDB requirements are very common in unlicensed band operation, and it is for utilization of the band as much as possible by spreading and/or frequency hopping within the spectrum.

R1) Channel access with Listen-Before-Talk (LBT) procedure

R2) New S-SSB signals to meet minimum Occupied Channel Bandwidth (OCB)/PSD requirements

The RAN1 agreements on these R1 and R2 aspects can be summarized as below [5].

RAN1 #111 (14th November – 18th November 2022) Agreement - Type 2A channel access procedure is applicable for S-SSB transmissions from a UE without a shared channel occupancy, when the following constraints are met: - Time duration is at most 1ms per transmission - The duty cycle of the S-SSB transmissions is at most 1/20 - FFS: details of EDT - FFS: whether/how to define observation period, including whether or not observation period would be captured in the specifications if defined - FFS: Type 2A applicability for PSFCH without a shared channel occupancy and further limitations for combined transmissions of both S-SSB and PSFCH using Type 2A channel access procedure .. Agreement For UE-to-UE COT sharing, - When performing S-SSB transmission(s), a responding UE can utilize a COT shared by a COT initiating UE (using type 1 channel access) when the responding UE is intended to transmit S-SSB within RB set(s) corresponding to the shared COT. - .. RAN1 #110 (22nd August – 26th August 2022) Agreement If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions - FFS the number and locations of additional candidate S-SSB occasions - FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior Agreement For S-SSB and synchronization in SL-U: - No changes on R16 NR SL S-PSS/S-SSS sequence generation - Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS RAN1 #109-e (9th May – 20th May 2022) Agreement For S-SSB and synchronization in SL-U: - FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc. - Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission - Option 1: Using interlaced RB transmission - Option 2: S-SSB multiplexing with other SL transmissions in the same slot - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth - FFS: whether to support 4 symbols S-SSB - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above - FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission - FFS whether any changes to R16/R17 NR SL synchronization procedure

## V2X RRM impact from unlicensed operation

### UE Transmit Timing

UE transmit timing requirement is to define timing error limit when synchronization source is GNSS, NR Cell and SyncRef UE. Depending on the final RAN1 design on S-SSB under OCB/PSD requirements, there may be change in timing error limit value with SyncRef UE as a synchronization source as well as disclaimer statement on S-SSB availability of SyncRef UE under CCA restriction.

Observation 1: Depending on the final RAN1 design on S-SSB under OCB/PSD requirements, there may be change in timing error limit value with SyncRef UE as a synchronization source as well as disclaimer statement on S-SSB availability of SyncRef UE under CCA restriction.

Proposal 1: RAN4 to monitor RAN1 progress in new S-SSB under OCB/PSD requirements and assess the effects on UE transmit timing error requirement after RAN1 finalization.

### Initiation / Cease of SLSS Transmission

Initiation/Cease of SLSS tramssion requirement is to define the required time for measuring the RSRP of NR cell and PSBCH-RSRP of the selected SyncRef UE. Under unlicensed operation, RAN4 needs to relax the requirements with SyncRef UE as synchronization source considering LBT failure of SyncRef UE as well as disclaimer statement on S-SSB availability of SyncRef UE under CCA restriction.

Proposal 2: RAN4 needs to relax the requirements with SyncRef UE as synchronization reference source considering the LBT failure of SyncRef UE as well as disclaimer statement on S-SSB availability of SyncRef UE under CCA restriction.

### Selection / Reslection of V2X Synchronization Reference Source

This RRM requirements specify the required time Tdetect, SyncRef UE_V2X to identify intra-frequency SyncRef as well as allowed drop rate of V2X data and SLSS transmission for the purpose of selection/reslection to the SyncRef UE depending on the configuration of priority of synchronization sources. Consequently, RAN4 needs to evaluate the effects of CCA requirement to SyncRef UE identification and allowed drop rate of V2X data and SLSS transmission of sidelink UE in unlicensed band.

Observation 2: There may be change in Tdetect, SyncRef UE_V2X and allowed drop rate of V2X data and SLSS transmission for the purpose of selection/reslection to the SyncRef UE in SL-U operation.

Proposal 3: RAN4 needs to update the requirements on Tdetect, SyncRef UE_V2X and allowed drop rate of V2X data and SLSS transmission for the purpose of selection/reslection to the SyncRef UE in SL-U operation.

Proposal 4: RAN4 needs to investgate the relaxation on the measurement period Tmeasure, PSBCH-RSRP requirements on PSBCH-RSRP measurements for the identified SyncRef UE considering the LBT failure from CCA requirement of SyncRef UE as well as disclaimer statement on the drop rate of V2X data and SLSS transmission under CCA restriction.

### L1 SL-RSRP measurement

Observation 3: Depending on the final RAN1 design on S-SSB under OCB/PSD requirements, there may be change in L1 SL-RSRP requirements.

Proposal 5: RAN4 to monitor RAN1 progress in new S-SSB under OCB/PSD requirements and assess the effects on L1 SL-RSRP measurement after RAN1 finalization.

### Congestion Control measurement

The UE shall be capable of estimating the channel busy ratio for one or more transmission pools indicated by higher layers in TS 38.331[2], based on SL-RSSI measurements provided by the physical layer. The UE shall perform channel busy ratio (CBR) measurement based on SL-RSSI measurements as described in TS 38.215. Thus, this functionality should be also supported in unlicensed band operations like as RSSI and channel occupancy measurement in NR-U, but no imminent changes are expected since RSSI just measure energy levels.

Proposal 6: RAN4 needs to monitor RAN1 progress in physical channels for unlicensed band operation and check the necessity of requirement changes in RSSI measurement and channel busy ratio (CBR) measurement at least for side condition perspective in performance requirement.

### Interruption

Interruption to WAN requirement due to RRC reconfiguration of SL would not be changed since RRC reconfiguration message are sent over Uu for SL operation in unlicesend band as well as licensed band. The interruption on SL-U needs further study under CCA restriction.

Proposal 7: RAN4 needs to investigate the effects of LBT procedure to the existing interruption on SL especially for synchronization source change and transitions between active and non-active during DRX.

| V2X | Scenario | Requirement | Expected impact from SL-U |
| --- | --- | --- | --- |
| 12.7.1 | Interruptions to WAN due to V2X Sidelink Communication | UE is allowed an interruption of up to the duration of 2 ~ 9 slots depending on SCS on the PCell/serving cell during the RRC reconfiguration procedure of side link | Reuse (No Impact) |
| 12.7.2 | V2X Sidelink Communcation Dropping due to synchronization source change | UE is allowed to drop up to 1 ms for any V2X sidelink signals including PSSCH, PSCCH, PSBCH, PSFCH and SLSS signals | FFS on the interval [1 ms] or definition of interrupt interval considering LBT procedure |
| 12.7.3 | Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink | UE is allowed an interruption of up to the duration of 2 ~ 3 slots depending on SCS | Reuse (No Impact) |
| 12.7.4 | Interruptions to WAN at transitions between active and non-active during SL-DRX | Interruption on PCell/serving cell if configured due to V2X transitions between active and non-active during SL-DRX are allowed with up to 1% prob. of missed ACK/NACK when SL-DRX cycle is less than 640 ms, and 0.625% prob. of missed ACK/NACK is allowed when SL-DRX cycle is 640 ms or longer. When multiple SL-DRX cycles are configured, the shortest SL-DRX cycle is applied. Each interruption shall not exceed [1 ~ 3] slot depending on SCS / Sync. | Reuse (No Impact) |
| 12.7.5 | Interruptions to V2X sidelink at transitions between active and non-active during DRX | Interruption on V2X sidelink if configured due to PCell transitions between active and non-active during DRX are allowed with up to 1% prob. of missed ACK/NACK when DRX cycle is less than 640 ms, and 0.625% prob. of missed ACK/NACK is allowed when DRX cycle is 640 ms or longer. It is only applied when HARQ process on V2X sidelink is supported. Each interruption shall not exceed [1 ~ 3] slot depending on SCS / Sync | FFS on the degradation levels of prob. of missed ACK/NACK for SL-U considering LBT procedure |
| 12.7.6 | Interruptions to V2X sidelink due to Active BWP switching Requirement | Interruptions on the V2X sidelink due to BWP switch in FDM based intra-band concurrent V2X operation. [1 ~ 3] slot depending on SCS. | Reuse (No Impact) |
| 12.7.7 | Interruptions to WAN due to SyncRef UE detection and/or Sensing during SL DRX off duration | The requirements in clause 12.7.4 shall apply. | Reuse (No Impact) |
| 12.7.8 | Interruptions at NR sidelink discovery configuration | UE is allowed an interruption of up to the duration [2 ~ 9] slot depending on SCS on the PCell/serving cell during the RRC reconfiguration procedure that includes the NR sidelink discovery configuration message | Reuse (No Impact) |

### Schedlung avaliablity

The restrictions on the scheduling availability for V2X sidelink due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmission on a dedicated carrier would not be changed for unlicensed band operation. The same argument can be applied to the restrictions on the scheduling availability for V2X sidelink due to switching between Uu uplink and V2X sidelink and the restriction is described as slot level granularity.

For example, UE behavior is as below:

When switch from V2X sidelink slot to Uu uplink slot occurs in Uu slot ‘n’,

- UE is not expected to transmit uplink or receive downlink on the Uu slot ‘n’

Observation 4: The restrictions on the scheduling availability for V2X sidelink due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmission on a dedicated carrier would not be changed for unlicensed band operation.

## RRM issues from LTE/NR sidelink coexistence

### Introduction

Semi-static TDM or FDM coexistence may be implemented by disjoint configuration of LTE and NR sidelink resource pool in FR1 licensed band. Dynamic resoure pool sharing in RAN1 is also under development.

### Separate subframe / slots for LTE SLSS / NR S-SSB

When defining the sidelink resource pool in LTE, subframes in which SLSS resource is configured are excluded from the pool for trasnsmission mode 3 and 4 [4]. It is the same for NR sidelink operation where Ns-ssb slots, in which S-SS/PSBCH block (S-SSB) is configured, are excluded in resource pool configuration [5].

Observation 5: The resource pool for LTE/NR sidelink transmission and LTE SLSS/NR S-SSB do not overlap in time-domain. Thus, measurement requirements for sidelink operation would not be changed due to LTE/NR coexistence.

# Conclusion

Observation 1: Depending on the final RAN1 design on S-SSB under OCB/PSD requirements, there may be change in timing error limit value with SyncRef UE as a synchronization source as well as disclaimer statement on S-SSB availability of SyncRef UE under CCA restriction.

Proposal 1: RAN4 to monitor RAN1 progress in new S-SSB under OCB/PSD requirements and assess the effects on UE transmit timing error requirement after RAN1 finalization.

Proposal 2: RAN4 needs to relax the requirements with SyncRef UE as synchronization reference source considering the LBT failure of SyncRef UE as well as disclaimer statement on S-SSB availability of SyncRef UE under CCA restriction.

Observation 2: There may be change in Tdetect, SyncRef UE_V2X and allowed drop rate of V2X data and SLSS transmission for the purpose of selection/reslection to the SyncRef UE in SL-U operation.

Proposal 3: RAN4 needs to update the requirements on Tdetect, SyncRef UE_V2X and allowed drop rate of V2X data and SLSS transmission for the purpose of selection/reslection to the SyncRef UE in SL-U operation.

Proposal 4: RAN4 needs to investgate the relaxation on the measurement period Tmeasure, PSBCH-RSRP requirements on PSBCH-RSRP measurements for the identified SyncRef UE considering the LBT failure from CCA requirement of SyncRef UE as well as disclaimer statement on the drop rate of V2X data and SLSS transmission under CCA restriction.

Observation 3: Depending on the final RAN1 design on S-SSB under OCB/PSD requirements, there may be change in L1 SL-RSRP requirements.

Proposal 5: RAN4 to monitor RAN1 progress in new S-SSB under OCB/PSD requirements and assess the effects on L1 SL-RSRP measurement after RAN1 finalization.

Proposal 6: RAN4 needs to monitor RAN1 progress in physical channels for unlicensed band operation and check the necessity of requirement changes in RSSI measurement and channel busy ratio (CBR) measurement at least for side condition perspective in performance requirement.

Proposal 7: RAN4 needs to investigate the effects of LBT procedure to the existing interruption on SL especially for synchronization source change and transitions between active and non-active during DRX.

Observation 4: The restrictions on the scheduling availability for V2X sidelink due to switching between E-UTRA V2X sidelink and NR V2X sidelink transmission on a dedicated carrier would not be changed for unlicensed band operation.

Observation 5: The resource pool for LTE/NR sidelink transmission and LTE SLSS/NR S-SSB do not overlap in time-domain. Thus, measurement requirements for sidelink operation would not be changed due to LTE/NR coexistence.

- References

- RP-222806 WID revision: NR sidelink evolution

- TS 38.133 NR RRM requirement specifications

- RP-222805 Status Report to TSG: NR sidelink evolution

- TS 38.214 Sec. 8 Physical sidelink shared channel related procedures

- TS 36.213 Sec. 14 UE procedures related to Sidelink