3GPP TSG-RAN WG4 Meeting#106	R4-2300490

# Athens, Greece, February 27 – March 3, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | draft | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification of the 100kHz channel raster for bands below 3GHz [channel-raster] |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, TEI17 |  |  |  |  |  | Date: |  |  | 2022-2-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Text clarification to maintain consistency between TS 38.101-1 and TS 38.104 with updates for UEs operating under 3GHz will no longer be required in all cases to be on 100kHz channel raster, but the option to put them on channel raster is up to the network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 5.3.1: The first change adds clarifying text by using the term common resource block grid instead of same spectrum. Clause 5.3.4: This change clarifies the text by adding the terms common resource block grid and dedicated signalling to better align terminology between RAN1,2,4 specs in this section and make it more readable. Also adds similar reference text as in TS 38.104 to be more consistent. Clause 5.4.2.2: clarifies the terms in Table 5.4.2.2-1. Currently, it is misleading that nprb in TS38.211 is the number of RB within a BWP, and in the RAN4 spec it is the number or RB within the carrier. Also, k as referenced in TS 38.211 is misleading whereas it is simply the resource element index within an RB. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There will be inconsitency in wording between TS 38.101-1 and TS 38.104. UEs will continue to be required to be on 100kHz for all applicable channels and existing problems will not be resolved. This results in wasted spectrum from un-used RBs between spectrum blocks assigned to UEs by the network. It also leads to a forced spectrum gap in CA applications. Further problems exist for 15kHz SCS, where cells defined with carrier bandwidth using an odd (or even) number of RBs have no raster aligned locations for UE CHBW using an even (or odd) number of RBs. Eliminating the need for UEs to be required on 100kHz raster would resolve these issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.1, 5.3.4, 5.4.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 38.101-1 CR R4-2300489, TS 38.104 CR R4-2300488 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

## 5.3	UE channel bandwidth

### 5.3.1	General

The UE channel bandwidth supports a single NR RF carrier or parts thereof in the uplink or downlink at the UE. From a BS perspective, different UE channel bandwidths may be supported within the common resource block grid for transmitting to and receiving from UEs connected to the BS. Transmission of multiple carriers to the same UE (CA) or multiple carriers to different UEs within the BS channel bandwidth can be supported.

From a UE perspective, the UE is configured with one or more BWP / carriers, each with its own UE channel bandwidth. The UE does not need to be aware of the BS channel bandwidth or how the BS allocates bandwidth to different UEs.

The placement of the UE channel bandwidth for each UE carrier is flexible but can only be completely within the BS channel bandwidth.

The relationship between the channel bandwidth, the guardband and the transmission bandwidth configuration is shown in Figure 5.3.1-1.

Figure 5.3.1-1: Definition of the channel bandwidth and the maximum transmission bandwidth configuration for one NR channel

### 5.3.2	Maximum transmission bandwidth configuration

< text omitted >

### 5.3.4	RB alignment

For each numerology, its common resource blocks are specified in Clause 4.4.4.3 in [9], make up the the common resource block grid, given by carrierBandwidth in SIB1[13].  The starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to "Reference point A" in the unit of the numerology. The UE transmission bandwidth configuration is indicated by the higher layer parameter carrierBandwidth, given by dedicated signalling in TS 38.331  [13] and will fulfil the minimum UE guardband requirement specified in Clause 5.3.3.

< text omitted >

#### 5.4.2.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements.

The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequency is designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [2016667...3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster

| Frequency range (MHz) | ΔFGlobal (kHz) | FREF-Offs [MHz] | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 24250 – 100000 | 60 | 24250.08 | 2016667 | 2016667 – 3279165 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The mapping between the channel raster and corresponding resource element is given in Clause 5.4.2.2. The applicable entries for each operating band are defined in clause 5.4.2.3

#### 5.4.2.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on channel raster and the corresponding resource element is given in Table 5.4.2.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the UE.

Table 5.4.2.2-1: Channel raster to resource element mapping

|  | NRB mod 2 = 0 | NRB mod 2 = 1 |
| --- | --- | --- |
| Resource element index k | 0 | 6 |
| Physical resource block number nPRB |  |  |

is the PRB index, k is the resource element index within the PRB, and NRB is the number of RBs in the carrier or channel.

#### 5.4.2.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in Table 5.4.2.3-1, using the channel raster to resource element mapping in clause 5.4.2.2.

-	For NR operating bands with 60 kHz channel raster above 24 GHz, ΔFRaster = I ×ΔFGlobal , where I ϵ {1,2}.  Every Ith NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in table 5.4.2.3-1 is given as <I>.

-	In frequency bands with two ΔFRaster, the higher ΔFRaster applies to channels using only the SCS that equals the higher ΔFRaster  and the SSB SCS that is equal to or larger than the higher ΔFRaster.

< end of changes >