3GPP TSG-RAN WG4 #106																			R4-2300708

Athens, GR, Feb-Mar. 2022

Agenda item:	9.7.3.1

Source: 	Qualcomm Incorporated

Title: 	On initial access beam correspondence

Document for:	Discussion

# Introduction

Previous agreements [5] have identified msg1 as the primary focus of requirements. We share our views on msg1 RF requirement and some test aspects.

# Discussion

## Msg1 EIRP spherical coverage

Previous agreements [2] have established that ‘BC is defined at maximum output power’. It is generally understood that UE can refine its beams similarly for RACH and in connected mode. MSG1 is also a low PAPR sequence by design, so it would be sufficient to treat like DFT-s-QPSK. Moreover, TS38.101-2 treats RACH like DFT-s-QPSK for MPR:

Observation 1: MPR for PRACH is same as that of DFT-s-QPSK, which implies the same EIRP range applies.

It therefore follows that Msg1 EIRP spherical coverage requirements can be co-opted from that of connected mode DFT-s-QPSK. In [5], the spherical coverage requirement for RACH msg1 is confirmed to be the same as for PUSCH. For consistency, this similarity should be extended to the peak EIRP requirement also.

A second perspective is system benefit. It is well understood that FR2 networks are UL limited. The network benefit of the UE’s ability to refine its beams during IA is clear. We do not see the downside for the UE. The UE is presumably capable of meeting its requirements in connected mode, so functionality theoretically exists. The BC requirement is not dynamic, so there are no timeline constraints either.

Observation 2: The UE has inherent capability to support the full range of EIRP for msg1 based on connected mode requirements.

In both perspectives above, it is difficult to justify that a peak EIRP requirement does not apply to msg1.

Proposal 1: Msg1 peak EIRP requirements are the same as those for connected mode DFT-s-QPSK.

## MSG1 test condition

Previous agreements [2] have established that ‘BC is defined at maximum output power’. It is however necessary to establish how the UE will be encouraged to transmit at maximum output power. 38.213 (section 7.4) charts out the following equation to set transmission power for RACH:

where PPRACHtarget is configured, and pathloss ‘PL’ is calculated from the gNB’s advertized EIRP for the SSB transmission.

A simple manipulation of these parameters will allow the TE to request a high target power. Further, the RACH configuration can be set to ramp up power by a configurable step size and have as many as 200 tries. These avenues facilitate multiple methods to achieve 1 ms cumulative duration of Msg1 RACH attempts for power measurements.

Proposal2 : The detail of how to enable the UE to transmit at max. output power is left to RAN5.

There could be concern about the UE attempting different RACH directions if it does not get a RAR before the counter runs out – however in a controlled test environment where the UE only sees one SSB, this kind of beam sweeping behavior is not expected.

## DL Polarization during test

Existing practice for connected mode can be extended to IA. No new techniques or solutions seem necessary

Proposal 3: DL polarizations during msg1 EIRP verification follow same practice as PUSCH EIRP testing.

# Conclusions

Observation 1: MPR for PRACH is same as that of DFT-s-QPSK, which implies the same EIRP range applies.

Observation 2: The UE has inherent capability to support the full range of EIRP for msg1 based on connected mode requirements.

Proposal 1: Msg1 peak EIRP requirements are the same as those for connected mode DFT-s-QPSK.

Proposal2 : The detail of how to enable the UE to transmit at max. output power is left to RAN5.

Proposal 3: DL polarizations during msg1 EIRP verification follow same practice as PUSCH EIRP testing.

# References

[1]	RP-220967, ‘NR RF requirements enhancement for frequency range 2 (FR2), Phase 3’, Nokia, RAN#95-e, Mar. 2022

[2]	R4-2214454, ‘WF on BC in RRC_INACTIVE and initial access’, Nokia, RAN4#104-e, Aug. 2022

[3]	R4-2212331, ‘On initial access beam correspondence’, Qualcomm, RAN4#104-e, Aug. 2022

[4]	R4-2218041, ‘On initial access beam correspondence’, Qualcomm, RAN4#105, Toulouse, FR., Nov. 2022

[5]	R4-2220517, ‘WF on beam correspondence requirements for initial access and RRC_inactive ’, Nokia, RAN4#105, Toulouse, FR., Nov. 2022