3GPP TSG-RAN WG4 Meeting #106	R4- 2300716

Athens, Greece, 27th Feb – 3rd Mar, 2023

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for band n8

Agenda Item:	8.23.2

Document for:	Approval

# 1	Introduction

This paper presents the PC2 A-MPR simulation results for band n8 as per the WF in [1].  Accordingly, the A-MPR requirements to be specified are proposed.

# 2	Discussion

### 2.1 Additional Spectrum Emission Requirements for Band n8

The additional spectrum emission requirements specified in TS 38.101-1 are duplicated below.

##### 6.5.3.3.5	Requirement for network signalling values "NS_43" and “NS_43U”

When "NS 43" or “NS_43U” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.5-1: Additional requirements for "NS_43" and “NS_43U”

| Frequency range (MHz) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15 |  |
| 860 ≤ f ≤ 890 | -40 | 1 MHz |
| NOTE 1: Applicable for 5 MHz and 15 MHz channel BW confined between 900 MHz and 915 MHz and for 10 MHz channel BW confined between 905 MHz and 915 MHz |  |  |

It can be seen that the guard band is 10MHz for BW=5/15MHz, 15MHz for BW=10MHz.  Hence, BW=15MHz is the most stringent case, and BW=5MHz is the least.

### 2.2 A-MPR Simulation Results

The simulation results are obtained for NS_43 for both PC2 1Tx and PC3 1Tx, which are shown in Table 1 below. The absolute PC2 A-MPR values are plotted in the 3rd column, while the differences between PC2 and PC3 are plotted in the 2nd column.

Table 1: A-MPR simulation results for NS_43 with 1Tx

| BW | DIFF A-MPR (PC2-PC3) | PC2 A-MPR |
| --- | --- | --- |
| 15MHz |  |  |
| 10MHz |  |  |
| 5MHz |  |  |

### 2.3 A-MPR for band n8

The existing PC3 A-MPR requirements in TS 38.101-1 are duplicated below.

#### 6.2.3.6	A-MPR for NS_43 and NS_43U

Table 6.2.3.6-1: A-MPR regions for NS_43

| Channel Bandwidth (MHz) | Carrier Centre Frequency, Fc (MHz) | Region A |  |  | Region B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RBstart | LCRB | A-MPR | RBstart | LCRB | A-MPR |
| 5 MHz | 902.5 ≤ FC < 912.5 |  | > 15 | A1 |  |  |  |
| 10 MHz | FC = 910 |  | > 40 | A2 |  | > 5.4 MHz/12/SCS | A4 |
|  |  |  | > 45 | A3 |  | > 7.2 MHz/12/SCS | A5 |
| 15 MHz | FC = 907.5 | < 1.8 MHz /12/SCS > 12.24 MHz/12/SCS | > 0 | A6 | > 1.8 MHz/12/SCS < 6.12 MHz/12/SCS | ≥ 7.2 MHz/12/SCS | A6 |
| NOTE 1: The A-MPR values are specified in Table 6.2.3.6-2. NOTE 2: 15 kHz SCS unless otherwise stated NOTE 3: Void |  |  |  |  |  |  |  |

Table 6.2.3.6-2: A-MPR for NS_43

| Modulation/Waveform |  | A1 (dB) |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  | A6 (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner | Outer / Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A | ≤ 9 |
|  | QPSK | ≤ 2 |  |  |  |  |  | ≤ 2.5 |  |  |  | ≤ 9 |
|  | 16 QAM |  |  |  |  |  |  |  |  | ≤ 2.5 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 2.5 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |
| CP-OFDM | QPSK | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 16 QAM | ≤ 3.5 |  |  |  |  |  |  |  | ≤ 4 |  | ≤ 9 |
|  | 64 QAM |  |  |  |  | ≤ 4 |  |  |  |  |  | ≤ 9 |
|  | 256 QAM |  |  |  |  |  |  |  |  |  |  | ≤ 9 |

Table 6.2.3.6-3: Void

When NS_43U is signalled for 5 and 10 MHz channel bandwidths A-MPR is defined in Table 6.2.3.1-2 except for DFT-s-OFDM QPSK when LCRB > 5.4 MHz/12/SCS the A-MPR is 2.5 dB. For 15 MHz channel bandwidth Table 6.2.3.6-4 applies.

Table 6.2.3.6-4: A-MPR for NS_43U

| Modulation/Waveform |  | 15 MHz |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 9 |
|  | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |
| CP-OFDM | QPSK | ≤ 9 |
|  | 16 QAM | ≤ 9 |
|  | 64 QAM | ≤ 9 |
|  | 256 QAM | ≤ 9 |

It can be seen that: for BW=5MHz, A1 applies; for BW=10MHz, A2 is for Pi/2 BPSK, A3 for 64QAM, and A4 for QPSK; for BW=15MHz, A6 applies. For NS_43U, the PC3 A-MPR is max{A-MPR for NS_43, A-MPR for NS_100}.

Based on the simulation results for BW=5MHz, it’s proposed to modify the A-MPR values for A1 as follows:

| Modulation/Waveform |  | A1 (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A |
|  | QPSK | ≤ [2+1] |  |
|  | 16 QAM | ≤ [3] |  |
|  | 64 QAM | ≤ [3] |  |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | ≤ [3.5+0.5] |  |
|  | 16 QAM | ≤ [3.5+0.5] |  |
|  | 64 QAM | ≤ [4] |  |
|  | 256 QAM |  |  |

Based on the simulation results for BW=10MHz, it’s proposed to modify the A-MPR values for A2, A3, A4 and A5 as follows:

| Modulation/Waveform |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A |
|  | QPSK |  |  |  |  | ≤ [2.5+3] |  |  |  |
|  | 16 QAM |  |  |  |  |  |  | ≤ [2.5+2.5] |  |
|  | 64 QAM |  |  | ≤ [2.5+2.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  |  |  | ≤ [4+2] |  |
|  | 16 QAM |  |  |  |  |  |  | ≤ [4+2.5] |  |
|  | 64 QAM |  |  | ≤ [4+2] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |

Based on the simulation results for BW=15MHz, it’s proposed to modify the A-MPR values for A6 as follows:

| Modulation/Waveform |  | A6 |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9+3] |
|  | QPSK | ≤ [9+3] |
|  | 16 QAM | ≤ [9+3] |
|  | 64 QAM | ≤ [9+3] |
|  | 256 QAM | ≤ [9+3] |
| CP-OFDM | QPSK | ≤ [9+3] |
|  | 16 QAM | ≤ [9+3] |
|  | 64 QAM | ≤ [9+3] |
|  | 256 QAM | ≤ [9+3] |

# 3	Conclusion

In this paper, we have presented our initial simulation results for NS_43 for PC2 1Tx. Based on the existing PC3 A-MPR requirements, the following modifications for PC2 1Tx are proposed:

Proposal 1: For NS_43 PC2 1Tx BW=5MHz, modify the A-MPR values for A1 as follows:

| Modulation/Waveform |  | A1 (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK |  | N/A |
|  | QPSK | ≤ [2+1] |  |
|  | 16 QAM | ≤ [3] |  |
|  | 64 QAM | ≤ [3] |  |
|  | 256 QAM |  |  |
| CP-OFDM | QPSK | ≤ [3.5+0.5] |  |
|  | 16 QAM | ≤ [3.5+0.5] |  |
|  | 64 QAM | ≤ [4] |  |
|  | 256 QAM |  |  |

Proposal 2: For NS_43 PC2 1Tx BW=10MHz, modify the A-MPR values for A2, A3, A4 and A5 as follows:

| Modulation/Waveform |  | A2 (dB) |  | A3 (dB) |  | A4 (dB) |  | A5 (dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer | Inner | Outer | Inner | Outer | Inner | Outer | Inner |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 1.5 | N/A |  |  |  | N/A |  | N/A |
|  | QPSK |  |  |  |  | ≤ [2.5+3] |  |  |  |
|  | 16 QAM |  |  |  |  |  |  | ≤ [2.5+2.5] |  |
|  | 64 QAM |  |  | ≤ [2.5+2.5] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |
| CP-OFDM | QPSK |  |  |  |  |  |  | ≤ [4+2] |  |
|  | 16 QAM |  |  |  |  |  |  | ≤ [4+2.5] |  |
|  | 64 QAM |  |  | ≤ [4+2] |  |  |  |  |  |
|  | 256 QAM |  |  |  |  |  |  |  |  |

Proposal 3: For NS_43 PC2 1Tx BW=15MHz, modify the A-MPR values for A6 as follows:

| Modulation/Waveform |  | A6 |
| --- | --- | --- |
|  |  | Outer / Inner (dB) |
| DFT-s-OFDM | Pi/2 BPSK | ≤ [9+3] |
|  | QPSK | ≤ [9+3] |
|  | 16 QAM | ≤ [9+3] |
|  | 64 QAM | ≤ [9+3] |
|  | 256 QAM | ≤ [9+3] |
| CP-OFDM | QPSK | ≤ [9+3] |
|  | 16 QAM | ≤ [9+3] |
|  | 64 QAM | ≤ [9+3] |
|  | 256 QAM | ≤ [9+3] |

# References

- R4-2217121 WF on requirements for HPUE_Basket_FDD, China Unicom, RAN4#104bis-e