3GPP TSG-RAN WG4 Meeting #106	R4-2300973

Athens, Greece, February 27 – March 3, 2022

Agenda item:	9.25.1

Source: 		NTT DOCOMO, INC.

Title: 	Revised NR NTN enhancement workplan

Document for:	Approval

- Introduction

This contribution revises a workplan for the Rel-18 NR-NTN-enh work item [1] for RAN4. Since WID was revised and the objectives are clarified, RAN4 workplan shall be modified as well. There are four objectives related to RAN4, “Coverage enhancement”, “NR-NTN deployment in above 10 GHz bands”, “Network verified UE location” and “NTN-TN and NTN-NTN mobility and service continuity enhancements”.

- WID objective

Revised WID objectives are copied as follows (revised parts are highlighted)

Objective of SI or Core part WI or Testing part WI

The work item aims at specifying enhancements for NG-RAN based NTN (Non-Terrestrial Networks) according to the following assumptions with implicit compatibility to support HAPS (High Altitude Platform Station) and ATG (Air To Ground) scenarios:

- GSO (Geo Synchronous Orbit) and NGSO (Non Geo Synchronous Orbit). NGSO includes Low Earth Orbit (LEO) and Medium Earth Orbit (MEO).

- Earth fixed tracking area. Earth fixed & Earth moving cells for NGSO

- FDD mode

- UEs with GNSS (Global Navigation Satellite Systems) capabilities

- Both “VSAT” (Very Small Aperture Terminal) devices with directive antenna (including fixed and moving platform mounted devices and commercial handset terminals (e.g. Power class 3) are supported in FR1

- Only “VSAT” devices with directive antenna (including fixed and moving platform mounted devices) are supported in above 10 GHz bands.

Note 1: In Rel-17 WID, “VSAT” device with external antenna on moving platform is equivalent to a device that operate on platforms in motion, and this is referred to as ESIM (Earth Station In Motion).

Note 2: The Rel-17 NTN architecture is assumed.

The detailed objectives are to specify enhancing features to Rel-15, 16 & 17’s NR radio interface & NG-RAN as follows:

4.1.1	Coverage enhancement

The Rel-18 NTN objectives are focused on the applicability of the solutions developed by general NR coverage enhancement to NTN, and identifying potential issues and enhancements if necessary, considering the NTN characteristics including large propagation delay and satellite movement. Only NTN-specific characteristics are to be included in this coverage enhancement work, otherwise it should be part of another WI (e.g., UL enhancement of coverage).

The following sentence will be revisited in RAN#99 as part of the DL enhancements discussion:

“The evaluation should also take into account any related regulatory requirements, e.g., ITU limitation of power flux density.” No work on this topic will take place in RAN WGs before the discussion on DL enhancements in RAN#99.

The following reference scenario is considered for the definition of uplink coverage enhancements for NTN: parameter set-1 for LEO-1200 satellite operating at Line of Sight (LOS) and commercial smartphones with -5.5 dBi antenna gain and 3 dB polarisation loss (per antenna port).

Note: It is understood that the enhancements defined for LEO can also apply to GEO and MEO scenarios as appropriate. No additional work is expected for MEO/GEO.

The targeted services are VoIP using AMR 4.75 kbps and data transmission services with Low data rate of 3 kbps.

The detailed objectives are for NTN:

- To specify PUCCH enhancements for Msg4 HARQ-ACK (e.g. repetition) [RAN1, RAN4]

- To specify if necessary, enhancements to the Rel-17 procedures for study DMRS bundling for PUSCH taking into account NTN-specifics (e.g. time-frequency pre-compensation) and, if necessary, specify enhancements to the Rel-17 procedures [RAN1]

Have a 1-TU 6-month study phase focusing on the following (to derive clear & limited scope):

- Evaluate the coverage performance and identify the candidate physical radio channels that have coverage issues specific to NTN with following target services taking into account the studies in TR38.830 where appropriate, as well as general coverage enhancement techniques specified in Rel-18 [RAN1,RAN2,RAN4]

  - VoIP and low-data rate services for commercial handset terminals

The following items are shown as examples of areas to consider in the RAN2 study.

- Improved performance of low-rate codecs in link budget limited situation including reducing RAN protocol overhead for VoNR

  - NOTE: Intent is not to introduce a new codec.

RAN to determine by RAN#97 (for RAN1 items) and RAN#98 (for RAN2 items) whether the study phase has identified any need for NTN-specific coverage enhancements in Rel-18. If needed, the set of NTN-specific work item objectives will be further updated.

4.1.2	NR-NTN deployment in above 10 GHz bands

The following assumptions are taken a baseline for this work:

- GSO and NGSO (e.g. LEO, MEO, HEO) based satellite access to be considered

  - ESIM scenarios for NGSO in Ka band are not considered in this WI.

- Targeted UE types: fixed and mobile VSAT. VSAT UE characteristics from TR38.821 to be considered in priority but additional NTN UE classes may be considered if justified

  - Regarding mobile VSAT, three types of terminal and scenario exist; airborne, maritime and land based ESIM. Which type(s) to be specified depends on the outcome of the regulation analysis and co-existence study.

- FDD mode is assumed for satellite operation above 10 GHz, while TDD mode is assumed for terrestrial operation in FR2

- The ITU-R harmonized Ka band will serve as reference

- Co-existence between overlapping NTN and TN band portions is out of scope of this work item. This aspect will be captured in the specification.

The following covers the objectives for NR-NTN deployment in above 10 GHz bands. This work is expected to start after June 2022.

- Study and identify NTN example band: Analysis of regulations and adjacent channel co-existence scenarios. The example band shall be identified early in the WI. Additional bands can be introduced in a release-independent manner. [RAN4]

  - Consider the satellite harmonized Ka band as a reference, according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations, define an example band suitable for development of generic 3GPP minimum performance requirements (the example RAN4 band may be a portion of or the entire harmonized Ka band). [RAN4]

  - Study implications of FDD operation in FR2 and derive requirements for the identified example band appropriately. Satellite bands introduced in 3GPP for NTN for FDD shall not impact the existing 3GPP TDD specifications for terrestrial bands adjacent to the NTN band (see note 3 of the approved way forward RP-211596 in RAN#92-e). [RAN4]

  - Relevant coexistence scenarios and analysis to be considered in RAN4, if and where applicable, to ensure that satellite bands introduced in 3GPP for NTN shall not impact the existing specifications and shall not cause degradation (in the sense of RAN4 co-existence studies) to networks in 3GPP specified terrestrial bands adjacent to the NTN band. In that, it is assumed that the NTN-TN adjacent band coexistence will be performed at the harmonized Ka band edges. The outcome is expected to be applicable to all NTN-TN adjacent band scenarios (if any) in the whole Ka band range where applicable and regulations allow. [RAN4]

  - For all the above, RAN4 process as agreed for NTN in FR1 should be used for coexistence analysis in above 10 GHz bands [RAN4].

  - Definition of NTN band(s) above 10 GHz does not change the current FR1/FR2 definition, nor automatically apply to future terrestrial bands defined in this frequency region; (see proposal 2 of the approved way forward RP-211596 in RAN#92-e) [RAN4]

- Specify Rx/Tx requirements for satellite access node and different VSAT UE class (not only 60 cm aperture) as appropriate for the identified example band [RAN4]

- Identify values for physical layer parameters chosen from the existing FR1 and FR2 sets. The following set of parameters to specify, but not necessarily limited to, are listed.as follows [RAN4]:

  - time relationship related enhancement (e.g. K_offset)

  - subcarrier spacing for different UL/DL signals/channels

  - PRACH configuration index for FDD above 10 GHz.

4.1.3	Network verified UE location

Pending on the conclusion of the RAN SI FS_NR_NTN_netw_verif_UE_loc study item, study and evaluate, if needed, solutions for network to verify UE reported location information [RAN2,RAN1,RAN3].

RAN is expected to determine by RAN#98 whether the study has identified any need for Network verified UE location specification support in Rel-18.

Based on RAN1 conclusions of the study phase, RAN to prioritize the specification of necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view [RAN1, 2, 3, 4]. DL-TDoA methods for verification may be considered as lower priority and if time permits and condition in Note is satisfied.

Note 1: Enhancements assume reuse of the RAT dependent positioning framework

Note 2: The specification of DL-TDOA enhancements will be subject to the study of the impact of realistic UE clock drift onto DL-TDOA performance

Note 3: The target accuracy for position verification purposes is as documented in clause « recommendations » of the 3GPP TR 38.882 (i.e. 10 km granularity)

Note 4 : Multiple satellite in view by the UE may be considered if time allows

Note 5 : The enhancements may be subject to relevant SA WGs (e.g. SA3/SA3-LI) feedbacks on the reliability of UE reports involved

Note 6 : The enhancements should take into account the mirror-image ambiguity

Note 7 : Network verified UE location is an optional UE feature

4.1.4	NTN-TN and NTN-NTN mobility and service continuity enhancements

This work considers existing methods from NR TN as well as outcome of Rel-17 NR NTN WI outcome as baseline for NTN-TN mobility.

- Specify NTN-TN and NTN-NTN measurement/mobility and service continuity enhancements [RAN2,RAN3,RAN4]

  - For NTN-NTN mobility, specify cell reselection enhancements for earth moving cell, the timing based and location-based cell reselection for quasi-earth fixed cell in Rel-17 can be considered as the starting point. [RAN2, RAN3, RAN4]

  - Specify NTN-NTN handover enhancement for RRC_CONNECTED UEs in the quasi-earth-fixed cell and earth-moving cell to reduce the signalling overhead. [RAN2, RAN3]

  - Specify cell reselection enhancements for RRC_IDLE/INACTIVE UEs to reduce UE power consumption (NTN-TN mobility is prioritized). [RAN2, RAN3, RAN4]

Study and, if needed, specify enhancement to Xn[/NG] signalling to support feeder link switch-over, CHO, e.g. exchange of necessary information between gNBs. [RAN3]

Objective of Performance part WI

The performance part objectives are applicable to the NR-NTN deployment in above 10 GHz bands objective.

- Specify RRM performance requirements and test cases [RAN4]

- Specify UE demodulation and CSI reporting requirements [RAN4]

- Specify satellite access node demodulation requirements [RAN4]

- Specify satellite access node conformance tests [RAN4]

The RAN4 performance part for the remaining objectives needs to be further discussed once the scope is stabilized.

- RAN4 workplan

In this section, the RAN4 work plan for NR-NTN-enh WI is provided:

Coverage enhancement

In RAN#97e, HPUE related topic was discussed but it will be further discussed in RAN#99. Thus, the current workplan does not include HPUE related work so far.

Core part

| Meetings | Objectives |
| --- | --- |
| October 2022, RAN4#104-bis | RF: - Discuss RF related topic if necessary. |
| November 2022, RAN4#105 | RF: - Discuss RF related topic if necessary. |
| February 2023, RAN4#106 | RF: - Discuss RF related topic if necessary. RRM/Demod (0.06TU): Starting time depends on RAN1 study progress. - Discuss demoduration requirements and assumptions based on RAN1 study conclusion - Discuss RRM related topic if necessary. |
| April 2023, RAN4#106-bis | RF: - Discuss RF related topic if necessary. RRM/Demod (0.09TU): - Further discuss demoduration requirements and assumptions - Further discuss values for physical layer parameters for Demod - Discuss RRM related topic if necessary. |
| May 2023, RAN4#107 | RF: - Start drafting of CRs if necessary. RRM/Demod (0.09TU): - Further discuss demoduration requirements and assumptions and align on simulation results - Identify values for physical layer parameters for Demod - Start drafting of CRs |
| August 2023, RAN4#108 | RF: - Further drafting of CRs if necessary. RRM/Demod (0.09TU): - Further discuss demoduration requirements and assumptions - Further drafting of CRs |
| October 2023, RAN4#108-bis | RF: - Further drafting of CRs if necessary. RRM/Demod (0.09TU): - Further discuss demoduration requirements and assumptions - Further drafting of CRs |
| November 2023, RAN4#109 | RF: - Endorse CRs if necessary. RRM/Demod (0.09TU): - Finalize demodulation requirements and assumptions - Endorse CRs |

Performance part

| Meetings | Objectives |
| --- | --- |
| October 2023, RAN4#108-bis | - Discuss RRM test cases and related parameters if necessary - Discuss measurement accuracy if necessary - Discuss satellite access node demodulation requirements - Discuss satellite access node conformance tests |
| November 2023, RAN4#109 | - Further discuss and decide RRM test cases and related parameters if necessary - Further discuss and specify measurement accuracy if necessary - Further discuss and specify satellite access node demodulation requirements - Further discuss and specify satellite access node conformance tests |
| February 2024, RAN4#110 | - Further discuss and decide RRM test cases and related parameters if necessary - Further discuss and specify measurement accuracy if necessary - Further discuss and specify satellite access node demodulation requirements - Further discuss and specify satellite access node conformance tests |
| April 2024, RAN4#110-bis | Provide draft CRs |
| May 2024, RAN4#111 | Endorse CRs |

NR-NTN deployment in above 10 GHz bands

Core part

| Meetings | Objectives |
| --- | --- |
| October 2022, RAN4#104-bis | RF (0.5TU): - Discuss example band for NTN enhancement study according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations. - Study implications of FDD operation in FR2 and derive requirements for the to be identified example band appropriately. - Discuss coexistence study scenarios to be considered and related simulations assumptions. |
| November 2022, RAN4#105 | RF (0.5TU): - Further discuss example band for NTN enhancement study according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations. - Further study implications of FDD operation in FR2 and derive requirements for the to be identified example band appropriately. - Further discuss coexistence study scenarios to be considered and related simulations assumptions. |
| February 2023, RAN4#106 | RF (1TU): - Define example band for NTN enhancement study according to ITU allocation; taking into account deployment type (e.g. VSAT, ESIM), scenarios, and ITU-R/regional regulations. - Define requirements of FDD operation in FR2 for the identified example band appropriately. - Agree on coexistence study scenarios to be considered and related simulations assumptions. - Early discussion on calibration of simulations for coexistence study scenarios - Discuss Rx/Tx requirements for satellite access node and different VSAT UE class. - Discuss values for physical layer parameters for RF RRM/Demod (0.1TU): - Discuss demoduration requirements and assumptions - Discuss RRM core requirements - Discuss values for physical layer parameters for RRM/Demod - Discuss if there is any specific requirement associated to the exemplary bands agreed by RF group as well as the necessary simulations if any. |
| April 2023, RAN4#106-bis | RF (1TU): - Discussion on simulation results for coexistence study scenarios - Further discuss Rx/Tx requirements for satellite access node and different VSAT UE class. - Further discuss values for physical layer parameters for RF RRM/Demod (0.3TU): - Further discuss demoduration requirements and assumptions - Further discuss RRM core requirements - Further discuss values for physical layer parameters for RRM/Demod - Further discuss if there is any specific requirement associated to the exemplary bands agreed by RF group as well as the necessary simulations if any. |
| May 2023, RAN4#107 | RF (1TU): - Align on simulation results for coexistence study scenarios - Further discuss Rx/Tx requirements for satellite access node and different VSAT UE class and align on simulation results - Start drafting of CRs RRM/Demod (0.3TU): - Further discuss demoduration requirements and assumptions and align on simulation results - Further discuss RRM core requirements - Identify values for physical layer parameters for RRM/Demod - Further discuss if there is any specific requirement associated to the exemplary bands agreed by RF group as well as the necessary simulations if any. - Start drafting of CRs |
| August 2023, RAN4#108 | RF (1TU): - Further discuss Rx/Tx requirements for satellite access node and different VSAT UE class - Further drafting of CRs RRM/Demod (0.3TU): - Further discuss demoduration requirements and assumptions - Further discuss RRM core requirements - Further drafting of CRs |
| October 2023, RAN4#108-bis | RF (1TU): - Further discuss Rx/Tx requirements for satellite access node and different VSAT UE class - Further drafting of CRs RRM/Demod (0.3TU): - Further discuss demoduration requirements and assumptions - Further discuss RRM core requirements - Further drafting of CRs |
| November 2023, RAN4#109 | RF (1TU): - Agree on Rx/Tx requirements for satellite access node and different VSAT UE class - Endorse CRs RRM/Demod (0.3TU): - Agree on RRM core requirement - Finalize demodulation requirements and assumptions - Endorse CRs |

Performance part

| Meetings | Objectives |
| --- | --- |
| October 2023, RAN4#108-bis | - Discuss RRM test cases and related parameters - Discuss measurement accuracy - Discuss UE demodulation and CSI reporting requirements - Discuss satellite access node demodulation requirements - Discuss satellite access node conformance tests |
| November 2023, RAN4#109 | - Further discuss and decide RRM test cases and related parameters - Further discuss and specify measurement accuracy - Further discuss and specify UE demodulation and CSI reporting requirements - Further discuss and specify satellite access node demodulation requirements - Further discuss and specify satellite access node conformance tests |
| February 2024, RAN4#110 | - Further discuss and specify UE demodulation and CSI reporting requirements - Further discuss and specify satellite access node demodulation requirements - Further discuss and specify satellite access node conformance tests |
| April 2024, RAN4#110-bis | Provide draft CRs |
| May 2024, RAN4#111 | Endorse CRs |

Network verified UE location (added)

According to WID revision, the necessary enhancements to multi-RTT to support the network verified UE location in NTN assuming a single satellite in view shall be specified. The discussions related to DL-TDOA enhancements will be started based on RAN1 study progress.

Core part

| Meetings | Objectives |
| --- | --- |
| February 2023, RAN4#106 | RF: - Discuss RF related topic if necessary. RRM/Demod (0.03TU): - Discuss demoduration requirements and assumptions based on RAN1 study conclusion - Discuss RRM requirements and assumptions based on RAN1 study conclusion |
| April 2023, RAN4#106-bis | RF: - Discuss RF related topic if necessary. RRM/Demod (0.02TU): - Further discuss demoduration requirements and assumptions - Further discuss values for physical layer parameters for Demod - Further discuss RRM requirements |
| May 2023, RAN4#107 | RF: - Start drafting of CRs if necessary. RRM/Demod (0.02TU): - Further discuss demoduration requirements and assumptions and align on simulation results - Identify values for physical layer parameters for Demod - Start drafting of CRs |
| August 2023, RAN4#108 | RF: - Further drafting of CRs if necessary. RRM/Demod (0.02TU): - Further discuss demoduration requirements and assumptions - Further drafting of CRs |
| October 2023, RAN4#108-bis | RF: - Further drafting of CRs if necessary. RRM/Demod (0.02TU): - Further discuss demoduration requirements and assumptions - Further drafting of CRs |
| November 2023, RAN4#109 | RF: - Endorse CRs if necessary. RRM/Demod (0.02TU): - Finalize demodulation requirements and assumptions - Endorse CRs |

Performance part

| Meetings | Objectives |
| --- | --- |
| October 2023, RAN4#108-bis | - Discuss RRM test cases and related parameters - Discuss measurement accuracy - Discuss satellite access node conformance tests |
| November 2023, RAN4#109 | - Further discuss and decide RRM test cases and related parameters - Further discuss and specify measurement accuracy - Further discuss and specify satellite access node conformance tests |
| February 2024, RAN4#110 | - Further discuss and decide RRM test cases and related parameters - Further discuss and specify measurement accuracy - Further discuss and specify satellite access node conformance tests |
| April 2024, RAN4#110-bis | Provide draft CRs |
| May 2024, RAN4#111 | Endorse CRs |

NTN-TN and NTN-NTN mobility and service continuity enhancements

At this stage, RF/Demod related works are not expected to be discussed for this topic.

Core part

| Meetings | Objectives |
| --- | --- |
| October 2022, RAN4#104-bis | RF: - Discuss RF related topic if necessary. |
| November 2022, RAN4#105 | RF: - Discuss RF related topic if necessary. |
| February 2023, RAN4#106 | RF: - Discuss RF related topic if necessary. RRM/Demod (0.06TU): - Discuss RRM core requirements based on Rel-17 specification. |
| April 2023, RAN4#106-bis | RF: - Discuss RF related topic if necessary. RRM/Demod (0.09TU): - Further discuss RRM core requirements - Discuss values for physical layer parameters for RRM. |
| May 2023, RAN4#107 | RF: - Start drafting of CRs if necessary. RRM/Demod (0.09TU): - Further discuss RRM core requirements - Identify values for physical layer parameters for RRM - Start drafting of CRs |
| August 2023, RAN4#108 | RF: - Further drafting of CRs if necessary. RRM/Demod (0.09TU): - Further discuss RRM core requirements - Further drafting of CRs |
| October 2023, RAN4#108-bis | RF: - Further drafting of CRs if necessary. RRM/Demod (0.09TU): - Further discuss RRM core requirements - Further drafting of CRs |
| November 2023, RAN4#109 | RF: - Endorse CRs if necessary. RRM/Demod (0.09TU): - Agree on RRM core requirement - Endorse CRs |

Performance part

| Meetings | Objectives |
| --- | --- |
| October 2023, RAN4#108-bis | - Discuss RRM test cases and related parameters - Discuss measurement accuracy |
| November 2023, RAN4#109 | - Further discuss and decide RRM test cases and related parameters - Further discuss and specify measurement accuracy |
| February 2024, RAN4#110 | - Further discuss and decide RRM test cases and related parameters - Further discuss and specify measurement accuracy |
| April 2024, RAN4#110-bis | Provide draft CRs |
| May 2024, RAN4#111 | Endorse CRs |

References

[1] 3GPP, RP-223534, Thales, “NR NTN (Non-Terrestrial Networks) enhancements (revision of RP-222654)”.