3GPP TSG-RAN WG4 Meeting # 106	R4-2301048
Athens, Greece, February 27 – March 3, 2022

Agenda item:	9.16.5

Source:	CAICT

Title:	Channel model validation results for Bands n1, n5, n8, and n28

Document for:	Discussion

1	Introduction

In the Rel-18 MIMO OTA WID [1], bands n1, n5, n8, n28, n77 are listed as first priority for FR1 MIMO OTA performance requirements definition.

In this paper, we provide part of channel model validation results for bands n1, n5, n8, n28. Remaining of the channel model validation results will be submitted at following RAN4 meetings.

2	Discussion

The channel model validation results are provided in Figs. 1-3 and Table 2. The measurement setup, test procedures, and instrument settings are according to TS 38.151 V17.2.0. The measurement setup is composed by ETS-Lindgren 16-probe MIMO OTA chamber and Spirent Vertex channel emulator.

The following parameters of CDL-C UMi channel model have been validated:

Table 1. The validated CDL-C UMi channel model parameters

| Frequency | Band | Parameter |
| --- | --- | --- |
| 722MHz | n28 | Temporal correlation |
|  |  | PDP |
|  |  | Spatial correlation |
|  |  | Cross-polarization |
| 836.5MHz | n5, n8 | Temporal correlation |
|  |  | PDP |
|  |  | Spatial correlation |
|  |  | Cross-polarization |
| 2132.5MHz | n1 | Temporal correlation |
|  |  | PDP |
|  |  | Cross-polarization |

Fig. 1  Temporal correlation measurement results and reference curves for CDL-C UMi, fc ≤ 2.5 GHz

Fig. 2  PDP measurement results and reference values for CDL-C UMi, fc ≤ 2.5 GHz

Fig. 3  Spatial correlation measurement results and reference curves for CDL-C UMi, fc ≤ 2.5 GHz

Table 2. Cross-polarization verification results for CDL-C UMi, fc ≤ 2.5 GHz

| Frequency | Beam | Reference | Measurement result | Pass/fail limit |
| --- | --- | --- | --- | --- |
| fc = 722 MHz | Beam 1 | Input 1+2: V/H = 0 dB | -0.08 dB | ±1dB |
| fc = 836.5 MHz | Beam 1 | Input 1+2: V/H = 0 dB | 0.27 dB | ±1dB |
| fc = 2132.5 MHz | Beam 1 | Input 1+2: V/H = 0 dB | 0.59 dB | ±1dB |

Observation 1: CAICT has completed part of channel model validation for Bands n28, n5, n8, and n1. The following parameters of CDL-C UMi have been validated: TCF, SCF, PDP, Cross-polarization at 722MHz and 836.5MHz, as well as TCF, PDP, Cross-polarization at 2132.5MHz.

3	Conclusion

In this paper, we provide part of channel model validation results for bands n1, n5, n8, n28.

Observation 1: CAICT has completed part of channel model validation for Bands n28, n5, n8, and n1. The following parameters of CDL-C UMi have been validated: TCF, SCF, PDP, Cross-polarization at 722MHz and 836.5MHz, as well as TCF, PDP, Cross-polarization at 2132.5MHz.

References

- R4-222668, “R18 New WID: Enhancement of Multiple Input Multiple Output (MIMO) Over-the-Air (OTA) test methodology and requirements for NR UEs”, CAICT, OPPO, Keysight Technologies, 3GPP RAN#97e, Sep. 2022.