# 3GPP TSG-RAN WG4 Meeting # 106                                    R4-2301052

# Athens, Greece, Feb. 27-Mar. 3, 2023

# Title: 	Discussion on BS demodulation performance requirements

# Source: 	ZTE Corporation

# Agenda item:	9.13.5.3

# Document for:	Approval

# 1 Introduction

In RAN#95e meeting, the work item [RP-221369] on Air-to-ground network for NR was approved as one of Rel-18 RAN4 package.

During the last RAN4 meeting, some conclusions has been reached for ATG coexistence assumption. We want to share some views on ATG demodulation requirement.

- 2 Discussion

## 2.1 Doppler shift

With regard to Doppler shift, WI[1] specified flight speed up to 1200km/h. So in ATG network, it will have more large Doppler shift for CPE. For downlink transmission, we have analyzed the Doppler shift scenarios in [2] according to different deployments. And more importantly CPE will perform precompensation for Doppler shift in ATG network. Hence, it has just the residual Doppler offset for uplink transmission that is about 0.1 ppm like the NTN network.

Observation 1. CPE could perform precompensation for Doppler shift.

Proposal 1. Proposal to consider 0.1 ppm for uplink transmission.

## 2.2 Channel Model

The ATG base station is designed to serve the air line coverage, so the LOS channel model can be considered for ATG CPE deployment. Two alternatives can be considered for demodulation requirement evaluation of ATG. One is AWGN with Doppler shift trajectories corresponding to different scenarios can be considered. The other is using NTN channel  model or modified NTN channel model for ATG as NTN channel model is also used for communication between the the devices on ground and in the air.

Proposal 2. Proposal to consider AWGN or NTN channel model or modified NTN channel model for ATG demodulation requirement evaluation.

## 2.3 PUSCH demodulation requirements

In this section, we prefer to give some demodulation requirements for ATG.

From the WID[1] of objective, TDD bands(e.g. n77, n78, n79) and FDD bands(e.g. n1) have been specified for ATG network.We propose to extend the test cases for ATG network as follow.

- SCS =15 kHz, CBW = 40 MHz, and 106 RB allocation for FDD band.

- SCS =30 kHz, CBW = 100 MHz, and 273 RB allocation for TDD band.

- Proposal 3. RAN 4 defines the PUSCH demodulation requirements with:

- SCS =15 kHz, CBW = 40 MHz, and 216 RB allocation for FDD band.

- SCS =30 kHz, CBW = 100 MHz, and 273 RB allocation for TDD band.

And we want to provide simulation assumptions for PUSCH in this section.

Table 2-1: Simulation assumptions for PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern |  | 30 kHz SCS: 2D4S6U+38D |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| Antenna config |  | 2*2 |
| MIMO layer |  | 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain | PUSCH mapping type | A, B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Test metric |  | SNR @ %70 of maximum Throughput |

- 2.4 Simulation results

- Figure 1. PUSCH simulation results for MCS 13 and MCS 20.

- 3 Conclusion

In this contribution, we give some discussions on BS demodulation performance requirements for ATG , The conclusions are:

Observation 1. CPE could perform precompensation for Doppler shift.

Proposal 1. Proposal to consider 0.1 ppm for uplink transmission.

Proposal 2. Proposal to consider AWGN or NTN channel model  or modified NTN channel model for ATG demodulation requirement evaluation.

- Proposal 3. RAN 4 defines the PUSCH demodulation requirements with:

- SCS =15 kHz, CBW = 40 MHz, and 216 RB allocation for FDD band.

- SCS =30 kHz, CBW = 100 MHz, and 273 RB allocation for TDD band.

# 4	Reference

[1]  RP-221369, WID on Air-to-ground network for NR, CMCC.

[2]  R4-2301198 , Discussion on UE demodulation performance for ATG, ZTE.