3GPP TSG-RAN WG4 Meeting # 106	R4-2301104

# Athens, Greece, 27 Feb- 03 March, 2023

Source: 	Samsung

Title: 	Views on LP-WUR architectures

Agenda Item:	12.5.5

Document for:	Discussion

- Introduction

A new SID has been established to study the lower-power wake-up signal and receiver for NR while further revised as [1] in which below objective are included as RAN4 scope.

- Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4]

- Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4]

The issues RAN1 identified and the agreement RAN1 reached had been summarized as the following LS [2] which looks for RAN’4 feedback.

In this paper, we provide our views on different LP-WUR architectures and issues identified by RAN1, additionally we also summarize the clarification issues that may require/wait for RAN1’s further input.

- Discussion

2.1 Target for LP-WUR architecture design

- 2.1.1 Targeted coverage and power consumption

According to the SID [1], new signals will be introduced as LP-WUS to further reduce the power consumption for UE in contrast to the existing Rel-15/16/17 UE power saving mechanism. To further prolong the battery life, a separate receiver will be added at UE side to monitor the new wake-up signal with ultra-low power consumption. The main radio would be triggered on to receive signals once the WUR detects the wake-up signal. Otherwise, the main radio will maintain off state (from Rel-16) or off as a new state which can be further discussed during the SI.

If a separate new receiver for NR LP-WUS would be introduced into the existing UE architecture, the expected/targeted coverage (REFSENS) and power consumption are two crucial aspects which should be carefully evaluated, generally the two are mutually exclusive, i.e. better coverage calls for higher power consumption due to the adoption of power hungry component to improve coverage such as RF LNA, high accuracy LO/PLL, multi-bit ADC, etc. Hence the tradeoff between coverage and power consumption should be carefully taken into account. It is observed that a 20dB increase of normalized sensitivity requires 10 times power consumption increasing [3].

Observation 1: For LP-WUR architecture design, the tradeoff between coverage and power consumption should be carefully evaluated.

In our view, the coverage of LP-WUS should be comparable with the legacy signal of the main receiver, for sake of extensive application of LP-WUS. If the coverage of LP-WUS is more degraded compared with existing NR signal, it is anticipated that only UE at the cell center could benefit from this new feature, unfortunately the UE at the cell edge who suffers more sever power issue and has more urgent demand to prong the battery life is not able to utilize LP-WUS.

Proposal 1: The architecture design for LP-WUR should strive to comparable REFSENS with legacy signals, which is considered as one criteria for architecture selection.

To achieve comparable coverage with the existing signal of the main receiver, the LP-WUR may not achieve maximum energy efficiency. However, considering that the LP-WUR is assumed to be always on to monitor LP-WUS when the main receiver is off, the power consumption should still be much lower than the main radio. In addition, the average power consumption of LP-WUR can be further reduced by duty cycle monitoring mechanism of LP-WUS.

On the other hand, the targeted power consumption/coverage has impact on the components selection, such as whether replace PLL with FLL, the choice of LO (ring LO instead of crystal LO with sacrifice of the accuracy), and sample rate/bit-width of ADC. Hence more information from RAN1 could facilitate RAN4 discussion.

Proposal 2: Ask for RAN1’s further clarification on the targeted power consumption and coverage, which has huge impact on the architecture design (component selection) and requirement discussion.

- 2.1.2 Targeted Device type

It is understood that the design of the LP-WUR is a tradeoff between performance and power saving, which is also relevant to the use case. In addition, complexity and cost are another aspect to be considered which also depends on the use case for LP-WUS. In our view, WUR/WUS design and the device type are highly correlated, also the requirement differs for different device type. From the SID (reproduced as below) [1], the primary target is the IoT use case and wearables, however it is observed that eMBB/XR has also been discussing in RAN1. It would be helpful if RAN1 could clarify/ determine the targeted use cases (If both IoT/wearables and eMBB are considered, could IoT/wearables be the first priority?), given this would have influence on the supported maximum CBW (whether larger than 20MHz would be considered), the component selection, the applicable operating bands etc. for LP-WUS and LP-WUR.

  - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables

    - Other use cases are not precluded

Proposal 3: Ask for RAN1’ s further clarification/determination on the applicable device type for LP-WUS. If both IoT/wearables and eMBB are considered, could IoT/wearables be the first priority?

  - LP-WUR architectures for OOK detection [6][7][8][9]

RAN1 recommended three types of architecture for LP-WUR for RAN4 further discussion [2], i.e. RF envelop detection, heterodyne architecture with IF envelop detection and homodyne/zero-IF architecture with BB envelop detection. In addition, they are considered suitable for non-coherent OOK modulation (OOK demodulation is extensively used for ultra-low power receivers), and some of them can also be applicable for other modulations such as FSK, both OOK and FSK modulation scheme could be designed as OFDM-based waveforms [9]. The details for each type are discussed separately in the following part.

- 2.2.1 RF envelop detection

Figure 2.1 Architecture for RF envelop detection

Above diagram shows the typical architecture for RF envelop detection. The RF signal is converted into baseband signal directly via an RF envelop detector which simply detects the amplitude of the RF signal. The coverage is the worst among the three architectures due to envelop detector would transform all input to baseband and BB LPF is not able to filter the interference out, mitigating the interference requires very-Q matching network/RF BPF (corresponds to narrow bandwidth) to pick up the useful signal, which is quite challenging and calls for more power consumption. In addition, in absence of LO/PLL means frequency tuning cannot be supported, but the power consumption could be greatly reduced. This architecture can achieve the lowest complexity and power consumption compared to the other two.

- 2.2.2 Heterodyne architecture with IF envelop detection

Figure 2.2 heterodyne architecture with IF envelop detection

This architecture is widely used for base station RF architecture. The RF signal is down converted into IF signal via RF mixer, the IF signal is converted into baseband signal via IF envelop detection. The coverage for heterodyne is the best among these three architectures, which is guaranteed by the usage of RF LNA, IF/BB AMP and the filters. BB LPF is used to suppress the interference from adjacent channel/subcarrier. One inescapable downside of this architecture is the image (located at Lo+IF or Lo-IF), which calls for either the image rejection filter before the mixer or an image rejection mixer to eliminate the image interference.

- 2.2.3 Homodyne/zero-IF architecture with BB envelop detection

Figure 2.3 Homodyne/zero-IF architecture with BB envelop detection

This architecture is the mainstream for UE, while has also been widely used for base station. One advantage is the chain is shorter/simpler compared with heterodyne architecture and neither IF operation nor image filter is required, which makes it more implementable for on-chip integration. The RF signal is directly down converted into baseband signal via an RF mixer with identical LO frequency as RF signal, baseband envelope can be done either in analog domain or in digital domain. BB LPF is used to suppress the interference from adjacent channel/subcarrier. There is no image issue for this architecture, but LO leakage which would result in self-mixing and DC offset. Flicker (1/f) noise in baseband is another issue to be addressed. Generally these factors would lead to degradation of SNR. This architecture could be considered as a tradeoff solution between coverage and power consumption.

Observation 2: Homodyne/Zero-IF architecture with BB envelop detection could be considered as a tradeoff solution between coverage and power consumption.

- 2.2.4 Comparison

The comparison among these three architectures are summarized in [3][4] (Reproduced as below).

Table 1 Comparison of LP-WUR architectures (R1-2211908)

|  | RF ED | Zero-IF ED | IF ED |
| --- | --- | --- | --- |
| Power consumption range | <10uW | 300~600uW | <1000uW >Zero-IF ED |
| NF | High | Medium | Low |
| Sensitivity/coverage | >-70dBm | -96dBm~-102dBm | >-110dBm |
| cost/complexity | Low | Medium | High |
| Interference suppression capability | Low | Medium | High |

Table 2 Comparison of LP-WUR architectures (R1-2208669)

|  | Modulation | Power consumption | RF sensitivity | Interference rejection |
| --- | --- | --- | --- | --- |
| Type 1: amplitude detection at RF | ASK (OOK) | uw[4]~tens of uw | -50dBm[4]~-70dBm (LNA) | High-Q RF filter, multi-bit ADC |
| Type 2:amplitude detection at IF | ASK (OOK) | tens of uw[5] ~ mw[6] | -70dBm[5]~-110dBm[6] | RF filter, high-Q IF filter, multi-bit ADC |
| Type 3:amplitude detection at BB | ASK (OOK) | tens of uw [7]~ hundreds of uw[8] | -70dBm[7]~-90dBm[8] | RF filter, high-Q BB filter, multi-bit ADC |

In summary, though the architecture with RF ED can achieve the lowest power consumption compared with the other two, however the poor receiver sensitivity/coverage of this architecture cannot match with other NR signals. In addition, as elaborated in clause 2.3.2.1, RF-ED is not suitable for supporting multi-band operation (i.e. a wide frequency range) with good performance. Along with proposal 1, we make below proposal.

Proposal 4: It is proposed to exclude RF envelop detection architecture for LP-WUR.

  - Impact on RAN4 requirements

In the following part, we discuss the issues identified by RAN1, additionally we also summarize the clarification issues that may require RAN1’s further input.

LP-WUS transmission impact on existing gNB requirements

In general, if LP-WUS is multiplexed with the existing NR signal in frequency, and it is compatible with the exiting NR signal, guard band between LP-WUS and NR signal at BS side is needed if the orthogonality among carriers are no long guaranteed because of the different numerologies between NR signal and WUS. More specific, NR signal could support several numerologies, i.e. 15kHz, 30kHz and 60kHz for FR1, which might be different with LP-WUS numerology (applicable SCS for LP-WUS also needs RAN1’ input), while mixed numerology between NR signal and LP-WUS would result in interference to each other. In general, guard band should be reserved for performance protection in terms of mixed numerology, however defining specific guard band size may not be necessary. Equivalently, it should be guaranteed that the previous unwanted emission requirements (ACLR, operating band unwanted emissions, transmitter spurious emissions, corresponds to clause 6.6.3/4/5 of 38.104) shall also apply if the BS supports LP-WUS operation in NR in-band.

Proposal 5: If LP-WUS is multiplexed with existing NR signal in frequency, it should be guaranteed that all existing unwanted emission requirements (ACLR, OBUE, transmitter spurious emissions, corresponds to clause 6.6.3/4/5 of 38.104) shall also apply if the BS supports LP-WUS operation in NR in-band, while defining specific guard band size might be unnecessary and is not recommended for mixed numerologies.

LP-WUS receiving impact on UE side

      - Adjacent channel selectivity

We share our views on ACS issue based on the observations and proposals we made in above clauses, i.e. the architecture design for LP-WUR should strive to achieve comparable REFSENS with legacy signals, which is considered as one criteria for architecture selection/design and exclude RF-ED architecture. In addition, it is expected RF LNA/filters, IF AMP, BB AMP could reuse from the main radio, while the NF might be worse than main radio. With above assumption, the ACS for main radio could be considered as a starting point, while exact evaluation could be performed after we reach consensus on more indispensable condition, such as NF.

Observation 3: With the assumption that the architecture design for LP-WUR should strive to comparable REFSENS with legacy signals, the ACS for main radio could be considered as a starting point for LP-WUS, while exact evaluation could be performed after we reach consensus on more indispensable condition, such as NF.

      - Whether certain LP-WUR can support multi-band capability

A worldwide device supporting multi-band operation is expected to support LP-WUS functionality for these multi bands. From our observation, although all the recommended architectures from RAN1 are for FR1, it appears FR2 has not been explicitly excluded yet. Hence ask for RAN’1 clarification whether FR2 is under consideration of this SI is needed given that the RF architecture for FR2 is completely different with FR1.

For RF envelop detection architecture, in absence of mixer/LO means frequency tuning cannot be implemented, multi-band supporting with good performance can be very challenging due to high selectivity requirement for RF envelope detector and matching network/RF BPF to achieve good interference rejection performance. Hence it is assumed this architecture is not suitable for multi-band operation (i.e. a wide frequency range) with good performance.

For heterodyne architecture and homodyne/zero-IF architecture for LP-WUS, multi-band can be implemented via the mixer with frequency tuning of LO. In terms of main radio architecture, the RF BPF is band specific in order to eliminate the out-of-band interference, while switch is used to connect different RF filters/duplexers to RF LNA(s), same mechanism could apply for LP-WUR to support multi-band operation. In addition, these two architectures can also support LP-WUS to be configured on different carriers and in different position within a carrier.

Proposal 6: Heterodyne architecture and homodyne/zero-IF architecture can support multi-band operation for LP-WUS, as well as LP-WUS to be configured on different carriers and in different position with a carrier.

Proposal 7: RF envelop detector is not suitable for supporting multi-band operation (i.e. a wide frequency range) with good performance.

      - The feasible Noise Figure

As below equation of REFENS shown, NF (Noise figure) is a determined factor for evaluation of sensitivity, along with other parameters such as channel bandwidth, the required SNR (signal to noise radio) and the implementation margin, all of them require RAN1’s further input or the alignment between RAN1 and RAN4.

Sensitivity = -174dBm(kT) + 10*log(RX BW) + NF + SNR +IM

As mentioned above RF-ED has the worst REFSENS due to the large NF and the poor performance of interference suppression, as we proposed above, it is suggested to exclude this architecture, hence we do not discuss the feasible NF for this architecture anymore.

The agreement in RAN1 regards to NF is reproduced as below, the options are from 9 to 24. In RAN4, the common assumption of NF for REFSENS evaluation is 9dB for TDD band and FDD band with large frequency gap, 12dB for high frequency FDD band or FDD band with small frequency gap. It should also be noted that the NF is mainly dominated by the components at the beginning of the chain. To achieve comparable REFSENS with existing NR signal, it is expected that LNA/AMP cannot be omitted for LP-WUR, additionally the components such as LNA/AMP/filters of main radio could be reused to LP-WUR architecture, while a more power-saving LO might be adopted. Hence it is expected that the NF would be worse for LP-WUR than for main radio, but for sake of comparable REFSENS with existing NR signal, huge NF should not be pursued anymore. More specific, 3dB degradation for RENFENS is assumed due to lack of DRx, and also worse SNR is expected due to the potential adoption of small bit-width ADC and less accuracy LO, if NF is assumed as 15dB, correspondingly the REFSENS would degrade around 10dB compared to the main radio. Therefore, in our view, the feasible noise figure should be formulated based on the criteria that the architecture design for LP-WUR should strive to achieve comparable REFSENS with legacy NR signals.

- For LP-WUS coverage evaluation, the noise figure of LP-WUR is

  - Options : [9, 12, 15, 18, 21, 24], Other values can be reported by companies

- FFS: how to determine the NF option.

- The values provided is for the purpose of studying coverage of LP-WUS, and it can be further revisited depending on the receiver architecture discussion.

Proposal 8: The feasible noise figure should be formulated based on the criteria that the architecture design for LP-WUR should strive to comparable REFSENS with legacy NR signals.

      - The potential RF impairments

Frequency error

Higher frequency LO generators mainly includes ring oscillator, LC oscillator and crystal oscillator. Ring oscillator is easy integration but with low accuracy and higher noise performance, LC oscillator is available in a wide frequency range but not suitable for IC integration, crystal oscillator has the best frequency stability along with high power consumption. To serve power saving purpose, ring oscillator is recommended, however the RF signal would be down converted to IF frequency with certain frequency offset due to the inaccuracy of the ring oscillator.

In TS 38.521-1 Clause 6.4.1, it clarifies that frequency error is an item to evaluate the ability of both transmitter and receiver, to process frequency correctly.

Receiver: to extract the correct frequency from the stimulus signal, offered by the System simulator, under ideal propagation conditions and low level.

Transmitter: to derive the correct modulated carrier frequency from the results, gained by the receiver.

However in TS 38.101-1 there is only requirement introduced for transmitter, i.e. the mean value of basic measurements of UE modulated carrier frequency shall be accurate to within ±0.1PPM. Regarding Rx, it should be evaluated whether the demodulation performance could be guaranteed with more relaxed frequency error and for what extent the relaxation is.

Lo leakage (DC offset)

It is not a new thing for main radio, LO leakage leads to self-mixing which generates DC offset. It could be mitigated by inputting a compensate DC offset with same magnitude and opposite phase in mixer input, to counteract specific DC offset. For WUR, generally it can reuse the same mechanism as for main radio to alleviate DC offset, however the accuracy may could be relaxed if it is a big contributor for power saving, hence the contribution for power saving of such relaxation on counteracting DC offset could be further evaluated.

ADC

The power consumption of ADC depends on the bit-width and the sample rate, while larger bit-width corresponds to better detection performance but higher power consumption. In [5], the simulation result show that the detection performance of 4-bit ADC approximately approach to that of 8-bit ADC. Thus we think 4-bit ADC could be considered as a starting point.

      - SNR evaluation

SNR is the required signal-to-noise radio to achieve certain performance. It is understood that SNR is derived from link level simulation and determined by the receiver architecture and component selection, hence unify the model and assumed parameters for evaluation is necessary.

  - Ask for RAN1’s further clarification

During the discussion on above clauses, it is identified there are still many unclear items which require/wait for RAN1’s further input or clarification. It is understood some aspects are still being discussed in RAN1, but it would be very hard for RAN4 to proceed without clarification on below issues.

Thus, in order to facilitate RAN4’s discussion, we suggest to ask for RAN1’s further input or clarification on below issues for LP-WUS/WUR.

- The targeted power consumption

- The targeted coverage

- The possible waveform(s)

- The device type/use case

- The maximum supported CBW

- The maximum occupied RB number

- SCS

- The operating bands (Particularly whether FR2 could be excluded)

Proposal 9: It is proposed to reply RAN1’ LS along with some clarification items.

- 3. Conclusion

Observation 1: For LP-WUR architecture design, the tradeoff between coverage and power consumption should be carefully evaluated.

Proposal 1: The architecture design for LP-WUR should strive to comparable REFSENS with legacy signals, which is considered as one criteria for architecture selection.

Proposal 2: Ask for RAN1’s further clarification on the targeted power consumption and coverage, which has huge impact on the architecture design (component selection) and requirement discussion.

Proposal 3: Ask for RAN1’ s further clarification/determination on the applicable device type for LP-WUS. If both IoT/wearables and eMBB are considered, could IoT/wearables be the first priority?

Observation 2: Homodyne/Zero-IF architecture with BB envelop detection could be considered as a tradeoff solution between coverage and power consumption.

Proposal 4: It is proposed to exclude RF envelop detection architecture for LP-WUR.

Proposal 5: If LP-WUS is multiplexed with existing NR signal in frequency, it should be guaranteed that all existing unwanted emission requirements (ACLR, OBUE, transmitter spurious emissions, corresponds to clause 6.6.3/4/5 of 38.104) shall also apply if the BS supports LP-WUS operation in NR in-band, while defining specific guard band size might be unnecessary and is not recommended for mixed numerologies.

Observation 3: With the assumption that the architecture design for LP-WUR should strive to comparable REFSENS with legacy signals, the ACS for main radio could be considered as a starting point for LP-WUS, while exact evaluation could be performed after we reach consensus on more indispensable condition, such as NF.

Proposal 6: Heterodyne architecture and homodyne/zero-IF architecture can support multi-band operation for LP-WUS, as well as LP-WUS to be configured on different carriers and in different position with a carrier.

Proposal 7: RF envelop detector is not suitable for supporting multi-band operation (i.e. a wide frequency range) with good performance.

Proposal 8: The feasible noise figure should be formulated based on the criteria that the architecture design for LP-WUR should strive to comparable REFSENS with legacy NR signals.

Proposal 9: It is proposed to reply RAN1’ LS along with some clarification items.

- 4. Reference

[1] RP-222644, Revised SID on lower-power WUS WUR for NR, vivo, RAN#97e

[2] R1-2212999, LS to RAN4 on low-power wake-up receiver architectures, Apple, vivo, RAN1#111

[3] R1-2208669, Discussion on low-power wake-up receiver architecture, vivo, RAN1#110-bis-e

[4] R1-2211908, LP-WUS receiver architectures, ZTE, RAN1#111

[5] R1-2211031, Discussion on low-power wake-up receiver architecture, vivo, RAN1#111

[6] R1-2212071, Receiver architecture for LP-WUS, Samsung, RAN1#111

[7] R1-2212144, L1 signal design and procedures for LP-WUR, Qualcomm, RAN1#111

[8] R1-2211421, Discussion on LP-WUS receiver architecture, Intel Corporation, RAN1#111

[9] R1-2210909, Discussion on architecture of LP-WUS receiver, Huawei, Hisilicon, RAN1#111