3GPP TSG-RAN WG4 Meeting #106	R4-2301196

# Athens, Greece; 27th February – 3rd March 2022

Source: 		Nokia, Nokia Shanghai Bell

Title: 	BS requirements for 900 MHz LTE Band in the US

Agenda Item: 	10.4.4

Document for:	Approval

# Introduction

This document provides the list of expected changes (excluding system parameters) to 36.104 due to introduction of 900 MHz LTE Band in the US.

# Discussion

Below are (highlighted in yellow) expected changes (excluding system parameters) to 36.104 due to introduction of 900 MHz LTE Band in the US:

#### Clause 6.6.3.1	Minimum requirements for Wide Area BS (Category A):

For E-UTRA BS operating in Bands 5, 6, 8, 12, 13, 14, 17, 18, 19, 26, 27, 28, 29, 31, 44, 68, 71, 72, 73, 85, 87, 88, [106] emissions shall not exceed the maximum levels specified in Tables 6.6.3.1-1 to 6.6.3.1-3.

#### Clause 6.6.4.3	Additional spurious emissions requirements

| System type for E-UTRA to co-exist with | Frequency range for co-existence requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| E-UTRA Band [106] | 935 – 940 MHz | -52 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band [106]. |
|  | 896 – 901 MHz | -49 dBm | 1 MHz | This requirement does not apply to E-UTRA BS operating in band [106], since it is already covered by the requirement in clause 6.6.4.2. This requirement does not apply to E-UTRA BS operating in band 5 or 26. |

#### Clause 6.6.4.4	Co-location with other base stations

| Type of co-located BS | Frequency range for co-location requirement | Maximum Level | Measurement Bandwidth | Note |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band [106] | 896 – 901 MHz | -96 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 5 or 26 |
| LA E-UTRA Band [106] | 896 – 901 MHz | -88 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 5 or 26 |
| MR E-UTRA Band [106] | 896 – 901 MHz | -91 dBm | 100 kHz | This is not applicable to E-UTRA BS operating in Band 5 or 26 |

### Clause 7.6.1	General blocking requirement

| Operating Band | Centre Frequency of Interfering Signal [MHz] |  |  | Interfering Signal mean power [dBm] | Wanted Signal mean power [dBm] | Interfering signal centre frequency minimum frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap [MHz] | Type of Interfering Signal |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52, 54, 65, 66, 68, 70, [106] | (FUL_low -20) | to | (FUL_high +20) | -43 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 1-7, 9-11, 13-14, 18,19, 21-23, 24, 27, 30, 33-45, 48-54, 65, 66, 68, 70, [106] | (FUL_low -20) | to | (FUL_high +20) | -35 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |
| 1-7, 9-11, 13, 14, 18,19, 21-23, 24, 27, 30, 33-45, 48, 50, 52-54, 65, 66, 68, 70, [106] | (FUL_low -20) | to | (FUL_high +20) | -38 | PREFSENS +6dB (Note 1) | See table 7.6.1.1-2 | See table 7.6.1.1-2 |
|  | 1 (FUL_high +20) | to to | (FUL_low -20) 12750 | -15 | PREFSENS +6dB (Note 1) |  | CW carrier |

### Clause 7.6.2	Co-location with other base stations

| Co-located BS type | Centre Frequency of Interfering Signal (MHz) | Interfering Signal mean power (dBm) | Wanted Signal mean power (dBm) | Type of Interfering Signal |
| --- | --- | --- | --- | --- |
| WA E-UTRA Band [106] | 935 - 940 | +16** | PREFSENS + 6dB* | CW carrier |
| LA E-UTRA Band [106] | 935 - 940 | -6** | PREFSENS + 6dB* | CW carrier |
| MR E-UTRA Band [106] | 935 - 940 | +8** | PREFSENS + 6dB* | CW carrier |

# Conclusion

It is proposed to agree on proposed changes in this document.