3GPP TSG-RAN WG4 Meeting # 106                             R4-2301199

Athens, Greece, 27th February - 3rd March 2023

Title:	Reply LS on configuring margin for 1 Rx RedCap UEs

Response to:	R2-2213069/R4-2300016

Release:	Rel-17

Work Item:	NR_redcap-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Wu Yan

wu.yan7@zte.com.cn

Send any reply LS to:    3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

Overall description

For the list of cell-specific RSRP thresholds provided by RAN4, RAN2’s understanding is that the offset shall not apply to delta thresholds, i.e. s-SearchDeltaP-r16 and s-SearchDeltaP-Stationary-r17.

For Rel-16/Rel-17 not-at-cell-edge criterion evaluation, RAN2 understands that the offset should not apply to both cell selection thresholds (i.e. Qrxlevmin and Qqualmin) and not-at-cell-edge thresholds (i.e. s-SearchThresholdP-r16, s-SearchThresholdQ-r16, s-SearchThresholdP2-r17 and s-SearchThresholdQ2-r17) and RAN2 will update specifications to avoid the double offset.

RAN2 would also like to inform RAN4 that the following cell-specific RSRP thresholds should be included in the list to apply the offset.

    - rsrp-ThresholdSSB-SUL

    - rsrp-ThresholdMsg3

RAN4 would like to thank RAN2 for their reply LS on configuring margin for one Rx RedCap UE in R2-2213069/R4-2214484. It should be noted that RAN4 has discussed the value of cell-specific RSRP offset at the 104bis-e meeting and reached following consensus in the WF. The specific information is copied below:

Cell-specific RSRP thresholds and their values:

| No | Type of threshold | Threshold [dB] |
| --- | --- | --- |
| 1 | rsrp-ThresholdSSB, | + 1 |
| 2 | msgA-RSRP-ThresholdSSB, | + 1 |
| 3 | msgA-RSRP-Threshold | + 1 |
| 4 | absThreshSS-BlocksConsolidation | + 1 |
| 5 | sdt-RSRP-Threshold | + 1 |
| 6 | s-SearchDeltaP-r16 | - 1 |
| 7 | s-SearchDeltaP-Stationary-r17 | - 1 |
| 8 | s-SearchThresholdP-r16 | + 1 |
| 9 | s-SearchThresholdQ-r16 | + 1 |
| 10 | s-SearchThresholdP2-r17 | + 1 |
| 11 | s-SearchThresholdQ2-r17 | + 1 |
| 12 | Qrxlevmin and Qqualmin | - 1 |

According to the reply from RAN2, RAN4 needs to discuss the addition and deletion of the cell-specific RSRP threshold used in the RRC IDLE/INACTIVE state, and consider the value of the new parameters, including:

| Type of threshold | Threshold [dB] | operation |
| --- | --- | --- |
| s-SearchDeltaP-r16 | - 1 | delete |
| s-SearchDeltaP-Stationary-r17 | - 1 | delete |
| rsrp-ThresholdSSB-SUL | [+ 1] | add |
| rsrp-ThresholdMsg3 | [+ 1] | add |

# 2	Actions

To RAN2

ACTION: 	RAN4 would like RAN2 to take into account the above RAN4 replies/feedback in their future work on Redcap UEs.

# 3	Dates of next TSG-RAN WG RAN4 meetings

RAN4#106bis-e	17th - 26th Apirl 2023, Electronic Meeting

RAN4#107	22th - 26th May 2023, Aisa.