# 3GPP TSG-RAN WG4 Meeting # 106                                                              R4-2301221

# Athens, Greece, Feb. 27-Mar. 03, 2023

Title:	Reply LS on NR support for dedicated spectrum less than 5MHz for FR1

Response to:	R4-2300008/ R1-2212919

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:	Li Lu

E-mail Address:	<lu.li@zte.com.cn>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN WG1 have started working on the WI NR support for dedicated spectrum less than 5MHz for FR1 at RAN1#111 and have reached the following agreements:

Agreement In an LS to RAN4, in addition to reuse 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 responses on the maximum transmission bandwidth (the number of PRBs) for this channel BW. Agreement RAN1 would like to ask RAN4 if finer sync. raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as any input from RAN1 for RAN4’s answer to this question. Agreement Before getting RAN4 responses, RAN1 assume maximum transmission bandwidth, 15RBs or 16RBs for 3 MHz channel BW for evaluation and analysis. Note: include agreement into the LS Agreement Before getting RAN4 responses, RAN1 assume that the UE could know which RBs are used for SSB transmission after PSS/SSS is detected for evaluation and analysis. Note: it does not mean indication signaling is needed. Note: include this agreement into the LS Agreement For transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, a subset of PRBs of 20-PRB PBCH are used for PBCH transmission if the transmission BW of a channel is less than 20PRBs. - FFS which PRBs are used and how to use the PRBs - Note: PRBs for PSS/SSS are not punctured. Agreement For CORESET#0 configuration for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth, following options are for study, - Opt.1: Existing configuration table for 15kHz SCS, 5MHz minimum channel BW (i.e., table 13-1 in TS38.213) is reused for configuration - Opt.2: A new CORESET#0 configuration table is to be introduced for the configuration. Conclusion No enhancements are required for PRACH to operate NR on transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. - Note: PRACH formats and configurations not fitting into the transmission BW are not applicable Agreement Short PRACH formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS are supported for transmission bandwidths <5 MHz for 3MHz and 5MHz channel bandwidth. Conclusion No enhancements are needed for PUCCH to support transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth, - FFS: the necessity for PUCCH FH disabling. Agreement Study whether and how to recover PDCCH detection performance of CORESET#0 for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Non-interleaved CCE-to-REG mapping - Opt.3: A new interleaver to ensure PDCCH is fully mapped in the spectrum - Opt.4: New aggregation level(s) for fit in the spectrum - Opt.5: PDCCH rate matching - Opt.6.: no enhancement specified Agreement Study whether and how to recover PBCH detection performance for transmission bandwidths of <5MHz for 3MHz and 5MHz channel bandwidth. The following options are considered, - Opt.1: Power boosting - Opt.2: Multiple PBCH receptions - Opt.3: PBCH remapping - Opt.4: PBCH payload reduction - Opt.5: PBCH rate matching around the punctured PRBs - Opt.6: no enhancement specified

Based on the agreements, RAN1 have the following questions to RAN4:

Question 1: RAN1’s understanding is that in addition to reusing 5 MHz channel bandwidth, RAN1 suppose only 3 MHz channel bandwidth is supported, and would like to get RAN4 response on the maximum transmission bandwidth (the number of PRBs) for this channel BW.

RAN4 response: To be similar as LTE, the maximum transmission bandwidth configuration NRB for 3MHz channel bandwidth is 15.

Question 2: RAN1 have discussed aspects related to synch raster in the spectrum of interest. RAN1 would like to ask RAN4 if finer synch raster for the 3MHz and/or 5MHz channel bandwidth is feasible, as well as if RAN4 needs any input from RAN1.

RAN4 response: Because the current design of sync raster in NR has a cluster of three raster positions every 1.2 MHz for LTE refarming band, with the raster positions in each cluster having frequency offsets of either 50, 150 or 250 kHz apart from multiples of 1.2 MHz, which is much sparser than the channel raster e.g.100 kHz. If we follow the legacy sync raster design, it can be foreseen that more PRBs of SSB will be punctured. In order to minimize the punctured PRBs of SSB, RAN4 agreed to keep the sync raster and channel raster as 100 kHz.

2. Actions: RAN4 respectfully asks RAN1 to take the RAN4 replies into account in their work.

3. Dates of Next RAN WG4 Meetings:

TSG RAN WG4 Meeting #106	February 27th – March 3rd, 2023	Athens, Greece

TSG RAN WG4 Meeting #106bis	April 17th – April 26th, 2023	E-meeting