# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301224

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on BS RF for dedicated spectrum less than 5MHz for FR1

# Agenda Item:	9.14.4

# Document for:	Approval

- Introduction

During RAN#97-e meeting, a revised Work Item [1] has been approved on introduction of NR support for dedicated spectrum less than 5MHz for FR1. In this contribution, we want to share some initial views on BS RF requirements.

- Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n8, n26 and n28:

  - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum.

  - Minimize impact on RF requirements:

    - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator).

    - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n8, n26 and n28.

  - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz.

- Discussion

In the following Table 1, we share some initial views on the BS RF requirements for dedicated spectrum less than 5MHz for FR1 band and details could be found as following:

Table 1: Summary of the proposals for BS RF requirement for dedicated spectrum less than 5MHz for FR1

| Tx RF requirements part |  |
| --- | --- |
| Base station output power | All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of 3 MHz channel bandwidth. |
| Output power dynamics |  |
| Transmit ON/OFF power |  |
| Transmitted signal quality |  |
| OBW |  |
| Tx intermodulation |  |
| ACLR | To add ACLR requirement for 3MHz CBW. |
| Rx RF requirements part |  |
| REFSENS | To add REFSENS requirement for 3MHz CBW. |
| Dynamic range | To add dynamic range requirement for 3MHz CBW and define additional FRC. |
| ACS | To add ACS requirement for 3MHz CBW. |
| OOBB | To add OOBB requirement for 3MHz CBW. |
| RX IMD | To add RX IMD requirement for 3MHz CBW. |
| Rx spurious emission | No requirement impacts. |
| ICS | To add ICS requirement and define additional FRC for 3MHz CBW. |

- Conclusion

In this contribution, we want to share some views on BS RF requirements and the proposal is made as following:

Proposal 1: To use the proposals in Table 1 for BS RF requirements for NR support for dedicated spectrum less than 5MHz for FR1.

- Reference

RP-222645, Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell