# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301230

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on UE RF requirements for 900 MHz NR band

# Agenda Item:	8.32.3

# Document for:	Approval

- Introduction

During RAN#98-e meeting, a new Work Item [1] has been approved on introduction of 900 MHz NR Band in the US. In this contribution, we want to share some initial views on UE RF requirements.

- Specify a new 5G NR FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 5 MHz, 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Discussion

  - Maximum output power

For the new NR FDD operating band, as the objective stated, it assumes power class 3 (23 dBm).

Table 2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [n106] |  |  |  |  |  |  | 23 | [±2] |
| NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. |  |  |  |  |  |  |  |  |

Proposal 1: For the new NR FDD band, to define UE power class as Table 2.1-1.

  - Reference sensitivity

If we reuse band B8 duplexer, we can follow the reference sensitivity requirement for B8 defined in 36.101.

Table 2.2-1: Reference sensitivity

| SCS (kHz) | 3MHz (dBm) | 5MHz (dBm) |
| --- | --- | --- |
| 15 | [-99.2] | -97 |

Proposal 2: For the new NR FDD band, to follow the reference sensitivity requirement for B8 defined in 36.101.

  - Summary

In the following Table 2.3-1, we share some initial views on the UE RF requirements for the new NR FDD operating band and details could be found as following:

Table 2.3-1: Summary of the proposals for UE RF requirement for 900 MHz NR Band

| Tx RF requirements part |  |
| --- | --- |
| UE maximum output power | 23dBm maximum output power (i.e. PC3) should be needed for the new added FDD band. |
| MPR | The relative channel bandwidth for the new added FDD band [n106] equals to 0.56%, which is smaller than 3%, so the current MPR value can be reused for the new added FDD band [n106]. |
| Configured transmitted power | No requirement impacts. |
| Output power dynamics | No requirement impacts for 5 MHz CBW. To add output power dynamics requirements for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Transmit signal quality | No requirement impacts. |
| Output RF spectrum emissions |  |
| - Out of band emission | No requirement impacts for 5 MHz CBW. To add out of band emission requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| - Additional emission mask | The Report and Order establishes an out-of-band emissions (OOBE) limit outside a licensee’s frequency band of operation to be attenuated by at least 43 + 10 log (P) dB for uplink operations in the 897.5-900.5 MHz band. |
| - ACLR | No requirement impacts for 5 MHz CBW. To add ACLR requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Spurious emissions |  |
| - Spurious emissions for UE co-existence | To add the new added FDD band as the protected band or co-existence with other operating bands. |
| - Additional spurious emissions | To add the additional spurious emissions requirements for the new added FDD band. |
| Receiver characteristics |  |
| Reference sensitivity | If we reuse band B8 duplexer, we can follow the reference sensitivity requirement for B8 defined in TS 36.101. |
| Maximum input level | No requirement impacts for 5 MHz CBW. To add maximum input level requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| ACS | No requirement impacts for 5 MHz CBW. To add ACS requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Blocking characteristics | No requirement impacts for 5 MHz CBW. To add blocking requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Spurious response | No requirement impacts for 5 MHz CBW. To add spurious response requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Intermodulation | No requirement impacts for 5 MHz CBW. To add IMD requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Spurious emissions | No requirement impacts. |

- Conclusion

In this contribution, we give some initial discussion on UE RF requirements for 900 MHz NR band and the proposals are made as following:

Proposal 1: For the new NR FDD band, to define UE power class as Table 2.1-1.

Proposal 2: For the new NR FDD band, to follow the reference sensitivity requirement for B8 defined in 36.101.

In addition, the summary of the corrections on TS38.101-1 are given in Table 2.3-1.

- Reference

RP-223529, New WID on Introduction of 900 MHz NR Band in the US, Anterix