# 3GPP TSG-RAN WG4 Meeting # 106		  R4-2301231

# Athens, Greece, Feb. 27-Mar. 03, 2023

# Source:	ZTE Corporation

# Title:	Discussion on BS RF requirements for 900 MHz NR Band

# Agenda Item:	8.32.4

# Document for:	Approval

- Introduction

During RAN#98-e meeting, a new Work Item [1] has been approved on introduction of 900 MHz NR Band in the US. In this contribution, we want to share some initial views on BS RF requirements.

- Specify a new 5G NR FDD operating band operating with

  - UL: 896 – 901 MHz and DL 935 – 940 MHz

  - To support 5 MHz, 3 MHz channel bandwidths with 15 kHz SCS.

    - NOTE: 3MHz CBW will be added once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW.

- Study and specify the co-existence requirements

  - Support FCC OOB emission mask in FCC-20-67A1 for all the defined channel bandwidths in the proposed band to ensure co-existence with adjacent narrowband and broadband networks to avoid interference.

  - Study and if needed, specify the protection for B5 and B26 including MPR.

  - Assume reusing band B8 duplexer

- Specify UE RF core requirements to operate in North America with primary operations in the US.

  - Assume power class 3 (23 dBm).

- Specify necessary BS RF core requirement.

- Discussion

  - Operating band and band numbering

As indicated in the objective, a new 5G NR operating band should be defined.

Table 2.1-1: NR operating bands in FR1

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| [n106] | 896 MHz – 901 MHz | 935 MHz – 940 MHz | FDD |

Proposal 1: To define the new NR FDD band as in table 2.1-1.

  - Channel raster and sync raster

For NR-ARFCN, we propose to use the similar descending manner to define as following:

Table 2.2-1: NR-ARFCN

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n106] | 100 | 179200 – <20> – 180200 | 187000 – <20> – 188000 |

Proposal 2: For NR-ARFCN, it is proposed to define as Table 2.2-1.

Table 2.2-2: Applicable SS raster entries

| NR operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| [n106] | 15 kHz | Case A | 2342 – <1> – 2344 |

Proposal 3: The sync raster for 3 MHz channel bandwidth should depend on the consequence of Rel-18 NR_FR1_lessthan_5MHz_BW.

Proposal 4: It is proposed that the sync raster for 5 MHz CBW should follow the current design of sync raster in TS 38.101-1 and defined as Table 2.2-2.

  - Summary

The summary of the corrections on TS 38.104 are given in Table 2.3-1.

Table 2.3-1. Summary of related RF requirement

| General part |  |
| --- | --- |
| Operating bands | A new band number for 900 MHz NR Band is needed. |
| UE channel bandwidth | To support 5 MHz and 3 MHz channel bandwidths with 15 kHz SCS. |
| Channel arrangement | The NR-ARFCN and sync raster entries should be needed for the new added FDD band. |
| Tx RF requirements part |  |
| Base station output power | All of these requirements are defined as band or channel bandwidth agnostic except for output power dynamics, therefore it’s not expected to have any impacts with the introduction of 900 MHz NR Band. |
| Output power dynamics |  |
| Transmit ON/OFF power |  |
| Transmitted signal quality |  |
| OBW |  |
| Tx intermodulation |  |
| ACLR | The Report and Order establishes an out-of-band emissions (OOBE) limit outside a licensee’s frequency band of operation to be attenuated by at least 50 + 10 log (p) dB for downlink operations in the 936.5-939.5 MHz band. |
| Operating band unwanted emissions |  |
| Transmitter spurious emissions | To add the additional spurious emissions requirements for 900 MHz NR Band. To add the co-location with other base station for 900 MHz NR Band. |
| Rx RF requirements part |  |
| REFSENS | To add REFSENS requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Dynamic range requirement | To add dynamic range requirement for 3MHz CBW and define additional FRC once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| ACS | To add ACS requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Blocking requirement | To add blocking requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| OOBB | To add OOBB requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| RX IMD | To add RX IMD requirement for 3MHz CBW once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |
| Rx spurious emission | No requirement impacts. |
| ICS | To add ICS requirement for 3MHz CBW and define additional FRC once it is specified in Rel-18 NR_FR1_lessthan_5MHz_BW. |

- Conclusion

In this contribution, we give some initial discussion on BS RF requirements for 900 MHz NR band and the proposals are made as following:

Proposal 1: To define the new NR FDD band as in table 2.1-1.

Proposal 2: For NR-ARFCN, it is proposed to define as Table 2.2-1.

Proposal 3: The sync raster for 3 MHz channel bandwidth should depend on the consequence of Rel-18 NR_FR1_lessthan_5MHz_BW.

Proposal 4: It is proposed that the sync raster for 5 MHz CBW should follow the current design of sync raster in TS 38.101-1 and defined as Table 2.2-2.

Proposal 5: To use the proposals in Table 2.3-1 for 900 MHz NR band BS RF requirements.

- Reference

RP-223529, New WID on Introduction of 900 MHz NR Band in the US, Anterix