# 3GPP TSG-RAN WG4 Meeting # 106                            R4-2301235

# Athens, Greece, February 27 – March 3, 2023

# Source:        ZTE Corporation

# Title:           Discussion on FR2-1 UL 256QAM

# Agenda Item:   9.7.2

# Document for:  Discussion

- Introduction

In RAN 96 meeting, a new WID [1] for NR RF requirements enhancement for frequency range 2 (FR2), Phase 3 was approved, where for FR2 UL 256QAM, the objections are:

UL 256QAM

- Investigate and enable UL 256QAM for FR2-1 [RAN4]

  - Study the gain, operating SNR, phase noise model and implementation aspects

  - Specify the UE RF requirements

  - First priority: Targeted power classes are PC1, PC2 and PC5

  - Second priority: Targeted power class is PC3

In last RAN4 meeting, some issues such as operating SNR and SLS parameters were concluded and were captured in the WF [2]. However, there were still some open issues left, including the feasibility of UL 256QAM for 39 GHz, EIRP parameters for EVM, and etc..

In this contribution, we give some discussions on the Min EIRP and provide the SLS results for both 29GHz and 39GHz.

- Discussion

## 2.1 29GHz

One of the open issue is the feasibility for PC2/5 of 29GHz. For PC1, it was already agreed that UL 256 QAM for PC1 UEs of 29 GHz is feasible, which was included in the WF:

- UL 256 QAM for PC1 UEs of 29 GHz is feasible - FFS for PC2/PC5

In last meeting, we provided SLS results for UL 256QAM for PC1/2/3/5 UEs of 29GHz in [3], in terms of the simulations, which are shown below:

Figure 1, CDF of SINR for PC2 under Urban Macro with Fc = 29GHz

Figure 2, CDF of SINR for PC3 under Urban Macro with Fc = 29GHz

Figure 3, CDF of SINR for PC5 under Urban Macro with Fc = 29GHz

From figure 1, 2 and 3, we can find that 5% UE can achieve above 30dB SINR which show that PC2 and PC5 @29GHz are feasible for UL 256QAM. If the operating SNR is smaller than 30dB, then the percentage of UE meet the target SNR will increase. Even ~10% UE can achieve above 28dB SINR even for PC3.

Thus we observed PC2 and PC5 are feasible for UL 256QAM for 29GHz, and PC3 is also feasible if the operating SNR is not higher than 28dB.

Observation 1: PC2 and PC5 are feasible for UL 256QAM for 29GHz, and PC3 is also feasible if the operating SNR is not higher than 28dB.

## 2.2 Minimum EIRP

In RAN4 #105 meeting, one open issue is minimum EIRP requirement for EVM test.

Issue 3-1-1: How to define the minimum EIRP requirements for EVM test => FFS in next meeting Issue 3-1-2: The minimum EIRP requirements for EVM test => FFS in next meeting

For 16QAM and 64QAM, the EIRP for EVM was derived based on deviations of -20log(EVM) compared with QPSK. The comparisons for each modulation scheme for PC1 are shown in table 1.

Table 1: Comparisons UE EIRP for PC1

| Parameter | Unit | Average EVM level | Reference signal EVM level | SNR | Delta SNR | UE EIRP (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Pi/2 BPSK | % | 30.0 | 30.0 |  |  |  |
| QPSK | % | 17.5 | 17.5 | 15 | Baseline | 4 |
| 16 QAM | % | 12.5 | 12.5 | 18 | 3dB | 7 |
| 64 QAM | % | 8.0 | 8.0 | 22 | 7dB | 11 |

Therefore, with the same approaches as 16QAM and 64QAM, the EIRP for UL 256QAM could also be derived based on the deviations of -20log(EVM) compared with QPSK, so values are calculated in table 2.

Table 2: UE EIRP for UL256QAM for PC1/2/5

| Parameter | Unit | Level (PC1) | Level (PC2) | Level (PC5) |
| --- | --- | --- | --- | --- |
| UE EIRP for UL 256 QAM | dBm | 18 | 1 | 8 |

Proposal 1. The UE EIRP for UL 256 QAM for PC1/2/5 UE are defined below:

| Parameter | Unit | Level (PC1) | Level (PC2) | Level (PC5) |
| --- | --- | --- | --- | --- |
| UE EIRP for UL 256 QAM | dBm | 18 | 1 | 8 |

## 2.3 39GHz

In RAN4 #105 meeting, the feasibility of UL 256QAM for 39GHz is not confirmed, so we give the the system level simulation for 39GHz for both indoor and outdoor deployment.

- Issue 1-2-2: UL 256QAM feasibility for 39GHz based on SLS - FFS for PC1/PC2/PC5 for 39GHz in next meeting.

With regard to system level simulation, the Min peak EIRP was approved to be taken into consideration[2] and the updated simulation parameters are listed in table 3. Figure 4 to figure 9 show the system level simulation for 39GHz for PC1, PC2 and PC5 for both urban macro and indoor deployment.

Table 3. System level simulation assumption reached in RAN4 #104bis-e.

| Parameters |  | Urban macro | Indoor |
| --- | --- | --- | --- |
| Network layout |  | hexagonal grid, 19 macro sites, 3 sectors per site with wrap around | 50m x 120m, 12BSs |
| Inter-site distance |  | 200m (baseline) 300m (optional) | 20m |
| BS antenna height |  | 25 m | 3 m |
| UE location | Outdoor/indoor | Outdoor and indoor | Indoor |
|  | Indoor UE ratio | 20% |  |
|  | Low/high Penetration loss ratio | 50% low loss, 50% high loss |  |
|  | LOS/NLOS | LOS and NLOS | LOS and NLOS |
|  | UE antenna height | Same as 3D-Uma in TR 36.873 | 1.5 m |
| UE distribution (horizontal) |  | Uniform |  |
| Minimum BS – UE distance (2D) |  | 35 m | 0 m |
| Shadowing correlation |  | Between cells: 1.0 Between sites: 0.5 |  |
| Pathloss |  | Uma LOS and NLOS in table 5.2.2.1-1 of 38.803 | InH – Office LOS and NLOS in table 5.2.2.1-1 of 38.803 |
| Carrier frequency |  | 29GHz, 39GHz |  |
| BS antenna configuration |  | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 8 dBi | (Mg, Ng, M, N, P) = (1, 1, 8, 16, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |
| UE antenna configuration |  | First priority: PC1/PC2/PC5: (Mg, Ng, M, N, P) = (1, 1, 4, 4, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi Second priority: PC3: (Mg, Ng, M, N, P) = (1, 1, 2, 2, 2) (dv, dh) = (0.5λ, 0.5λ) GE,max = 5 dBi |  |
| System bandwidth |  | 200MHz |  |
| UE Min peak EIRP |  | n257 PC1:40.0 dBm/PC2:29 dBm/PC3:22.4 dBm/PC5: 30dBm n260 PC1:38.0 dBm/PC3:20.6 dBm n259 PC2:25dBm, PC5:27.7dBm |  |
| Target SNR at BS side |  |  |  |
| UE max output power |  | PC1: 35 dBm/PC2: 23dBm/PC3: 23 dBm/PC5: 23 dBm |  |

Figure 4, CDF of SINR for PC1 under indoor with Fc = 39GHz

Figure 5, CDF of SINR for PC2 under indoor with Fc = 39GHz

Figure 6, CDF of SINR for PC5 under indoor with Fc = 39GHz

Figure 7, CDF of SINR for PC1 under Urban Macro with Fc = 39GHz

Figure 8, CDF of SINR for PC2 under Urban Macro with Fc = 39GHz

Figure 9, CDF of SINR for PC5 under Urban Macro with Fc = 39GHz

From figure 4, 5 and 6, we can find that more than 85% UE can achieve above 30dB SINR in indoor deployment for PC1, PC2 and PC5 @39GHz. With regard to in urban macro deployment, we can find from figure 7, 8 and 9 that more 45% can achieve above 30dB SINR for PC1 and about 10% UE can achieve 30dB SINR for both PC2 and PC5. Thus we observed PC1, PC2 and PC5 are feasible for UL 256QAM for 39GHz.

Observation 2:

for 39GHz_ Indoor：

- For PC1, PC2 and PC5 ~86% of UE can reach the operating SNR

for 39GHz_ Urban Macro：

- For PC1, ~55% of UE can reach the operating SNR

- For PC2, ~8% of UE can reach the operating SNR

- For PC5 , ~15% of UE can reach the operating SNR

Proposal 2: To confirm the feasibility of UL 256qam at 39GHz for PC1, PC2 and PC5

Figure 10 and figure 11 show the SLS results for 39GHz for PC3 for both urban macro and indoor deployment.

Figure 10, CDF of SINR for PC3 under Urban Macro with Fc = 39GHz

Figure 11, CDF of SINR for PC3 under indoor with Fc = 39GHz

From figure 10 we can find that only about 2% UE can achieve above 30dB SINR in urban macro deployment for PC3 @39GHz. With regard to in indoor deployment, we can find from figure 11 that more than 70% UE can achieve above 30dB SINR for PC3.

Observation 3:

for 39GHz_ Indoor：

- For PC3, ~75% of UE can reach the operating SNR

for 39GHz_ Urban Macro：

- For PC3, ~2% of UE can reach the operating SNR

- Conclusion

Based on the simulation results and derivation, the following observations and proposals are given:

Observation 1: PC2 and PC5 are feasible for UL 256QAM for 29GHz, and PC3 is also feasible if the operating SNR is not higher than 28dB.

Observation 2:

for 39GHz_ Indoor：

- For PC1, PC2 and PC5 ~86% of UE can reach the operating SNR

for 39GHz_ Urban Macro：

- For PC1, ~55% of UE can reach the operating SNR

- For PC2, ~8% of UE can reach the operating SNR

- For PC5 , ~15% of UE can reach the operating SNR

Observation 3:

for 39GHz_ Indoor：

- For PC3, ~75% of UE can reach the operating SNR

for 39GHz_ Urban Macro：

- For PC3, ~2% of UE can reach the operating SNR

Proposal 1. The UE EIRP for UL 256 QAM for PC1/2/5 UE are defined below:

| Parameter | Unit | Level (PC1) | Level (PC2) | Level (PC5) |
| --- | --- | --- | --- | --- |
| UE EIRP for UL 256 QAM | dBm | 18 | 1 | 8 |

Proposal 2: To confirm the feasibility of UL 256qam at 39GHz for PC1, PC2 and PC5

- Reference

RP-221862 revised WID NR RF requirements enhancement for frequency range 2 (FR2), Phase 3

R4-2220810 WF on FR2 UL 256QAM

R4-2218595 Discussion on FR2 UL 256QAM