3GPP TSG-RAN WG4 Meeting #106	R4-2301382

# Athens, GR, 27 Feb - 03 Mar, 2023

Source:	China Telecom, UESTC

Title:	Discussion on Self-Interference Model

Agenda item:	9.19.2.2.1

Document for:	Discussion

Introduction

This document provides RSIC analysis framework table based on the way forward[1] in last meeting.

Discussion

We provides values based on the following RSIC analysis framework table and the information on how the intermediate results are derived.

| FR1 (or FR2-1) |  |  |  | China Telecom |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class（Wide Area BS） |  |  |  | Case I | Case II |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm@FR2 | 30 dBm@FR2 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 30 dBc | 30 dBc |  |
|  |  | Frequency isolation techniques used |  | N\A | N\A |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 100 dBc | 80 dBc |  |
|  |  | Spatial isolation techniques used |  | spatial separation between TX/RX panel; cross polarization; shielding case; metal fences | spatial separation between TX/RX panel; |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 0dBc | 0dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | <1.5dB | <1.5dB |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | −100dBm | −80dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0dBc | 0dBc |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 0dBc |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | N\A | N\A |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | <1.5dB | <1.5dB |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBmPSI_TXSB (Note 1) |  |  | −70dBm | −50dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 20 dB | 20 dB |  |
|  |  | Frequency isolation techniques |  | Filtering | Filtering |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | 0~15dBm | 0~15dBm |  |
|  |  |  | Rx IM3 contribution (dBm) | <−140dBm | <−100dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Insignificant due to the large spatial isolation | Insignificant due to the large spatial isolation |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | −90dBm | −70dBm |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 0dBc | 0dBc |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | <1dB | <1dB |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 0 dBc | 30 dBc |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 120 dBc | 130 dBc |  |
| Noise floor ⑩dBm |  |  |  | −83dBm/100MHz | −83dBm/100MHz |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | −89 dBm | −89 dBm |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc | 119 dBc |  |
| SBFD configuration |  |  |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | 100MHz |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

Conclusion

| FR1 (or FR2-1) |  |  |  | China Telecom |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class（Wide Area BS） |  |  |  | Case I | Case II |  |
| BS TX Power dBPTX = ① dBm |  |  |  | 30 dBm@FR2 | 30 dBm@FR2 |  |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | 30 dBc | 30 dBc |  |
|  |  | Frequency isolation techniques used |  | N\A | N\A |  |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | 100 dBc | 80 dBc |  |
|  |  | Spatial isolation techniques used |  | spatial separation between TX/RX panel; cross polarization; shielding case; metal fences | spatial separation between TX/RX panel; |  |
|  | TX Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | 0dBc | 0dBc |  |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  | <1.5dB | <1.5dB |  |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  | −100dBm | −80dBm |  |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | 0dBc | 0dBc |  |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | 0dBc | 0dBc |  |
|  |  | RF IC techniques and other tech. (before LNA) |  | N\A | N\A |  |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | <1.5dB | <1.5dB |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBmPSI_TXSB (Note 1) |  |  | −70dBm | −50dBm |  |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | 20 dB | 20 dB |  |
|  |  | Frequency isolation techniques |  | Filtering | Filtering |  |
|  |  | RX IMD | Rx IIP3 capability (dBm) | 0~15dBm | 0~15dBm |  |
|  |  |  | Rx IM3 contribution (dBm) | <−140dBm | <−100dBm |  |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) | Insignificant due to the large spatial isolation | Insignificant due to the large spatial isolation |  |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | −90dBm | −70dBm |  |
|  | RX Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | 0dBc | 0dBc |  |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | <1dB | <1dB |  |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | 0 dBc | 30 dBc |  |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | 120 dBc | 130 dBc |  |
| Noise floor ⑩dBm |  |  |  | −83dBm/100MHz | −83dBm/100MHz |  |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | −89 dBm | −89 dBm |  |
| Required RSIC budget (①-⑪dBc) |  |  |  | 119 dBc | 119 dBc |  |
| SBFD configuration |  |  |  |  |  |  |
| Guardband assumption (if exist) |  |  |  |  |  |  |
| bandwidth over which suppression is achieved |  |  |  | 100MHz | 100MHz |  |
| Others |  |  |  |  |  |  |
| Note 1: Relevant metrics are derived from other parameters for checking purpose. Note 2: The relevant metric is gain-normalized, with reference point assumed to be at RX antenna. Note 3: The notations ①②③④⑤⑥⑦⑧⑨⑩⑪ are used to indicate the decimal values of the corresponding metrics. |  |  |  |  |  |  |

Reference

- R4-2220244, “WF for the feasibility from BS aspect”, Samsung.