3GPP TSG-RAN WG4 Meeting #106	R4-2301385

Greece, Athens, February 27th– March3th, 2023

Source:	ZTE Corporation

Title:	Discussion on Network A requirements

Agenda item:	9.24.2.3

Document for:	Approval

Introduction

At RAN 95 meeting the revised WI “Dual Transmission/Reception (Tx/Rx) Multi-SIM for NR” [1] was approved.  In the last meeting, we had the discussion on Network A requirements, and the outcomes were captured in [2]. Based on the outcomes, the following issues need to be further discussed.

- Parameters for L1/L3 measurement requirements

- In this paper we will continue to discuss the related issues and provide our views on the above issue.

Discussion

Issue 3-1-1: Principle on layer 1 and layer 3 measurement requirements after gap collision handling - Proposals - P1: Frameworks of gap cancellation in concurrent gaps design and LBT failure in NR-U design can be used as starting point when discussing NW A requirement impact. (Apple) - P2: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Navailable / Ntotal for network A requirements when MUSIM gaps are configured. (xiaomi vivo oppo Huawei Ericsson) - P3: For L3 measurement without gap, the legacy Kp and CSSF for concurrent gaps can be reused except the case when SMTC is fully overlapping with MUSIM gap. (oppo) - P4: Postpone the detail NW-A’s requirement discussion to phase 2 and recheck the status after RAN #99 meeting (Ericsson) - P5: Existing UE requirements applicable for Network A shall apply when UE is allocated MUSIM gaps (Nokia) - WF - Suggest the following options to be used in future discussion: - Option 1: Frameworks of gap cancellation in concurrent gaps design and LBT failure in NR-U design can be used as starting point when discussing NW A requirement impact. - Option 2: Reuse the principle used in Rel-17 concurrent gaps WI as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured, i.e., introduce a scaling factor like Kx = Navailable / Ntotal for network A requirements when MUSIM gaps are configured - Option 3: Existing UE requirements applicable for Network A shall apply when UE is allocated MUSIM gaps

In the last meeting, we didn’t discuss the Network A requirements because of the limited time, but as suggested by moderator, we shall continue to discuss it. What the reason for studying the NW A requirements is that the configured MUSIM gaps may collide with layer3 or layer 1 resources in NW A. The collision will impact RRM requirements in NW A. Most companies consider the principle on layer 1 and layer 3 measurement requirements after gap collision handling and the majority of them agreed that we should reuse the principle used in Rel-17 concurrent gaps as baseline to define NW A L1/L3 measurement requirements when MUSIM gaps are configured.

TS 38.133 defines the concrete factors on L3 measurement as below:

When UE supports concurrentMeasGap-r17 and is configured with concurrent measurement gaps, Kp is the scaling factor for an SSB frequency layer to be measured without measurement gaps. Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gap and/or per-FR measurement gap within the same FR as the SSB frequency layer, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with measurement gap occasions within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion within the window W, after accounting for measurement gap collisions by applying the measurement gap collision rule in section 9.1.8.3. Kp = 1 when Navailable = 0.

The scaling factor is defined by the Kp and window W, that is, is defined by the total number of measurement occasion (Ntotal ) in a window W and the available number of measurement occasion(Navailable) in the same window W.Based on these, we suggest that we shall reuse the R17 con-MG outcomes as baseline and only add the MUSIM gaps for MUSIM gaps. So option 2 is fine to us.

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal 2: On L3 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

- Intra-frequency (without gap):

- Kp=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion and  non-dropped MUSIM gap occasions within the window W.

- Kp = 1 when Navailable = 0.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

- Inter-frequency:

Kgap=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- c.    The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

TS 38.133 defines the concrete factors on L1 measurement as below:

When UE supports concurrent measurement gap and concurrent gaps are configured, - P value for SSB resource to be measured is defined as - Ntotal / Noutside_MG in FR1 - Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0 - Ntotal / Navailable in FR2 with Navailable > 0 - For a window W of duration max(TL1, MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion: - Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and - Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W - Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W - TL1 is periodicity of the target SSB.

- The same way in L1 measurement.

Proposal 3: On L1 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

P value for SSB resource to be measured is defined as

Ntotal / Noutside_MG in FR1

Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

Ntotal / Navailable in FR2 with Navailable > 0

where,

- Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions,MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor MUSIM gap occasion within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion, MUSIM gap nor any SMTC occasion within the window W

- TL1 is periodicity of the target SSB.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

Conclusion

In this paper we provided our views on NW A requirements related to MUSIM gaps.

Proposal 1: Reuse the principle used in Rel-17 concurrent gaps as the baseline to define network A L1/L3 measurement requirements when MUSIM gaps are configured.

Proposal 2: On L3 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

- Intra-frequency (without gap):

- Kp=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions within the window W, including those overlapped with MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are not overlapped with any non-dropped MG occasion and  non-dropped MUSIM gap occasions within the window W.

- Kp = 1 when Navailable = 0.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

- Inter-frequency:

Kgap=Ntotal/Navailable

- Ntotal is the total number of SMTC occasions that are covered by instances of the associated MG within the window W, including those overlapped with other MGs and MUSIM gaps within the window.

- Navailable is the number of SMTC occasions that are covered by instances of the non-dropped associated MG within the window W.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

Proposal 3: On L1 measurements: the scaling factors shall be re-defined for MUSIM gaps impact with reusing the R17 con-MG principle, as below:

P value for SSB resource to be measured is defined as

Ntotal / Noutside_MG in FR1

Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

Ntotal / Navailable in FR2 with Navailable > 0

where,

- Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions,MUSIM gaps or SMTC occasions within the window, and

- Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor MUSIM gap occasion within the window W

- Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion, MUSIM gap nor any SMTC occasion within the window W

- TL1  is periodicity of the target SSB.

- The duration of the window W equals max{SMTC period, MGRP_max, MUSIM gap period}.

- References

- RP-221018, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation

- R4-2220443, WF on NR Dual Tx/Rx Multi-SIM,Vivo

- R4-2219555, Discussion on NW-A requirements with MUSIM gaps,Huawei

- R4-2218805, On network A requirements for RRM requirements of MUSIM gaps, Vivo

- R4-2219923, Discussion on NW A RRM requirements for MUSIM, MTK