3GPP TSG-RAN WG4 Meeting #106	R4-2301393

Greece, Athens, February 27th– March3th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 1394 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TS 38.133: Revise the Measured quantity value(ZoA) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2 |  |  |  |  |  | Date: |  |  | 2023-02-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The measured quantity value of ZOA shall include 180 degree. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The range of measured quantity value of ZOA shall be 0 degree≤ZOA≤180 degree |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The 180 degree of ZOA will miss and there will be the impact on RAN3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE>

13.4	AoA/ZoA

13.4.1	Report mapping

The reporting range of UL Angle of Arrival, as defined in Clause 5.2.4 of TS 38.215 [4], is defined from -180 degree to +180 degree for azimuth angle (AoA). The reporting resolution is 0.1 degree.

The reporting range of UL Angle of Arrival, as defined in Clause 5.2.4 of TS 38.215 [4], is defined from 0 degree to +180 degree for vertical angle (ZoA). The reporting resolution is 0.1 degree.

The mapping of AoA measured quantity is defined in Table 13.4.1-1. The mapping of ZoA measured quantity is defined in Table 13.4.1-2.

Table 13.4.1-1: AoA measurement report mapping

| Reported value | Measured quantity value (AoA) | Unit |
| --- | --- | --- |
| AoA_0 | -180 ≤ AoA < -179.9 | degree |
| AoA_1 | -179.9 ≤ AoA < -179.8 | degree |
| AoA_2 | -179.8 ≤ AoA < -179.7 | degree |
| … | … | … |
| AoA_1798 | -0.2 ≤ AoA ≤ -0.1 | degree |
| AoA_1799 | -0.1 ≤ AoA < 0 | degree |
| AoA_1800 | 0 ≤ AoA < 0.1 | degree |
| AoA_1801 | 0.1 ≤ AoA < 0.2 | degree |
| AoA_1802 | 0.2 ≤ AoA < 0.3 | degree |
| … | … | … |
| AoA_3598 | 179.8 ≤ AoA < 179.9 | degree |
| AoA_3599 | 179.9≤ AoA < 180 | degree |

Table 13.4.1-2: ZoA measurement report mapping

| Reported value | Measured quantity value (ZoA) | Unit |
| --- | --- | --- |
| ZoA_0 | 0 ≤ ZoA < 0.1 | degree |
| ZoA _1 | 0.1 ≤ ZoA < 0.2 | degree |
| ZoA _2 | 0.2 ≤ ZoA < 0.3 | degree |
| … | … | … |
| ZoA _1798 | 179.8 ≤ ZoA < 179.9 | degree |
| ZoA _1799 | 179.8 ≤ ZoA ≤ 180 | degree |

< End OF CHANGE>