3GPP TSG-RAN WG4 Meeting # 106 	R4-2301426

Athens, Greece, February 27 – March 3, 2022

Source: 	Huawei, HiSilicon

Title: 	Discussion on the feasibility from BS aspect

Agenda Item:	9.19.2.2.1

Document for:	Discussion

- Introduction

SBFD feasibility study and RF impact from BS aspects were discussed in last meeting and WF for the feasibility from BS aspect was approved in [1].

In this contribution, we provide some discussion on the open issue of RSIC analysis.

- Discussion

The format of the RSIC analysis framework table was agreed and the following agreements were made in last meeting [1]:

- Companies are encouraged to provide values based on the RSIC analysis framework table.

- Companies are encouraged to provide the information on how the intermediate results are derived.

For FR1 our analysis and evaluation is sumarized in Table 2.1-1, we show two examples for Wide Area BS to consider different max TX power. One is 47 dBm max TX power and the other is 53 dBm. The major difference for WA example 2 is the adoption of analog filter to counteract the higher RX blocking from TX sub-band. The filter can be RF analog filter in the front-end or base-band analogue filter before the ADC. In the example we use RF analog filter which is put after LNA. The impacts to RX sensitivity due to insertion loss of the analog filter is negligible sine LNA can provide substantial gain on the wanted signal. Besides the filter, the evaluation of link buget is based on the capability of existing hardware. It should be noted that the bandwidth of TX sub-band and the RX sub-band is asymmetrical, the scalling factor is assumed in the budget calculation. From Table 2.1-1, we can find that RX IM3 is one of dominant contributions and it is not 1-to-1 relationship with the power. Therefore, maybe the linear selectivity is not suitable for the blocking estimation. Sum of the effects of IM3, phase noise, ADC dynamic would contribute on the overall selectivity. The performance would be different from implementations. The selectivity is a function of blocking power, and the lower the power, the better the selectivity. So it is safe to use the selectivity at max BS TX power for the evaluation. From table 2.1-1, the selelectivity at max BS TX power (47 dBm) for WA BS example 1 is ~ 62 dB, and ~73 dB for WA BS example 2.

Currently description on beam nulling /isolation may be ok to FR1 but it is not friendly to FR2. Hence we propose to change to ”Beam nulling /isolation in TX sub-band” and ”Beam nulling /isolation in RX sub-band” which are more suitble for FR1 DBF and FR2 ABF architectures.

Table 2.1-1 RSIC Analysis for FR1

| FR1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Wide Area BS example 1 | Wide Area BS example 2 |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm | 47 | 53 |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc | 45 | 45 |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX | DPD | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc | 80 | 80 |
|  |  | Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation | spatial separation between TX/RX panel | spatial separation between TX panel to single RX |
|  | Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc | 10 | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  | Less than 0.5 dB loss | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  | =-94 | -88 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc | N/A | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc | N/A | 10 |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) | N/A | Analog filter is put after LNA |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc | N/A |  |
|  | Self-Interference signal in gNB TX subband, measured at the input of LNA dBPSI_TXSB (Note 1) |  |  |  | -43 | -37 (Equivalent to -47 when 10 dB filtering is counted.) |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc | 80 | digital filtering |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX | digital filtering | digital filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  | -10 | -10 |
|  |  |  | Rx IM3 contribution (dBm) |  | -109 | -121 |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  | ADC noise: -109 reciprocal phase noise mixing:-112 | ADC noise: -113 reciprocal phase noise mixing:-116 |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm | =-105 | -111 |
|  | Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc | 10 | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc | Less than 0.5 dB loss | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc | 15 | 15 |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc | -103.6 | -102.3 |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW | -96 dBm/20 MHz | -96 dBm/20 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm | -102 dBm | -102 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc | 149 | 155 |
| SBFD configuration |  |  |  |  | [40, 20, 40] | [40, 20, 40] |
| Guardband assumption (if exist) |  |  |  |  | Existing SU | Existing SU+ additionally few RBs |
| bandwidth over which suppression is achieved |  |  |  |  |  |  |
| Others |  |  |  |  |  |  |

For FR2 our analysis and evaluation is sumarized in Table 2.1-2. The impact of RX link is differnet from that of FR1 case since different architectures are adopted. Normally FR2 uses ABF architecture and FR1 uses DBF arechitecture. Spatial isolation is measured in the beam domain rather than the isolation between the array andt the single RX branch as FR1. The From Talbe 2.1-2, it can be found that the blocking level to RX pannel is weak and should be even lower at each LNA input. Hence the IM3 is not a limited factors. Due to the same reason that the blocking level is relatively weak, the other RX impacts due to blocker in Tx sub-band can also be negligible. Hence we think that the evaluation on Self-interference leakage in gNB RX subband would be sufficient for FR2.

Table 2.1-2 RSIC Analysis for FR2

| FR2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| BS class |  |  |  | Wide Area BS | Wide Area BS |
| BS TX Power dBPTX = ① dBm |  |  |  | xxx dBm | 35 dBm/200MHz |
| Component capability and parameters | Frequency isolation at TX | Frequency isolation capability dBαSI-frequency, TX = ② dBc |  | xxx dBc | 28 |
|  |  | Frequency isolation techniques used |  | e.g., DPD, sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in TX | DPD |
|  | Spatial isolation | Spatial isolation capability dBαSI-spatial = ③ dBc |  | xxx dBc | 85~95 |
|  |  | Spatial isolation techniques used |  | e.g., spatial separation between TX/RX panel; cross polarization; circulator; shielding case; metal fences, etc. Note: List all relevant techniques used in the evaluation | spatial separation between TX/RX panel |
|  | Beam nulling /isolation in TX sub-band dBαSI-beam = ④ dBc |  |  | xxx dBc | 10 |
|  | DL EIRP impact due to beam nulling in TX sub-band |  |  |  | Less than 0.5 dB loss |
|  | Self-interference leakage in gNB RX subband due to non-ideal TX, measured at RX ant. dBPSI_leakage (Note 1) |  |  |  | -94~-104 |
|  | RF IC and other tech. (before LNA) | RF IC capability and other tech. in TX sub-band dBαSI-RFIC = ⑤ dBc |  | xxx dBc | N/A |
|  |  | RF IC capability and other tech. in RX sub-band dBαSI-RFIC = ⑧ dBc |  | xxx dBc | N/A |
|  |  | RF IC techniques and other tech. (before LNA) |  | e.g., RF IC, sub-band filtering etc. Note: List all relevant techniques used in RX (before LNA) | N/A |
|  |  | Impacts to RX sensitivity (due to e.g. insertion losses) due to RF IC or other techniques before LNA |  | xxx dBc | N/A |
|  | Self-Interference signal in gNB TX subband, measured at the input of dBPSI_TXSBRX array |  |  |  | -60 ~ -70 |
|  | Blocker Suppression at RX | Frequency isolation capability dBαSI-frequency, RX= ⑥ dBc |  | xxx dBc | 60 |
|  |  | Frequency isolation techniques |  | e.g., sub-band analog filtering, digital filtering, etc. Note: List all relevant techniques used in RX | digital filtering |
|  |  | RX IMD | Rx IIP3 capability (dBm) |  |  |
|  |  |  | Rx IM3 contribution (dBm) |  | negligible |
|  |  | Other RX | Any other RX impacts if significant (e.g. ADC noise, phase noise etc.) |  | negligible |
|  | Self-Interference signal in gNB RX subband caused by non-ideal RX selectivity, gain-normalized dBPSI_Selectivity(Note 1, 2) |  |  | xxx dBm | negligible |
|  | Beam nulling /isolation in RX sub-band dBαSI-beam = ⑨ dBc |  |  | xxx dBc | 10 |
|  | RX sensitivity degradation caused by RX beam nulling |  |  | xxx dBc | Less than 0.5 dB loss |
|  | Digital IC dBαSI-digtial = ⑦ dBc |  |  | xxx dBc | - |
| Overall RSIC capability dBRSIC (Note 1) |  |  |  | xxx dBc | -94 ~-104 |
| Noise floor ⑩dBm |  |  |  | xxx dBm/CBW | -88 dBm/40 MHz |
| Residual Interference budget with 1 dB desens target (⑪dBm=⑩dBm-6dB) |  |  |  | xxx dBm | -94 dBm |
| Required RSIC budget (①-⑪dBc) |  |  |  | xxx dBc | 129 |
| SBFD configuration |  |  |  |  | [80,40,80] |
| Guardband assumption (if exist) |  |  |  |  | Existing SU |
| bandwidth over which suppression is achieved |  |  |  |  |  |
| Others |  |  |  |  |  |

- Conclusion

In this contribution, we provide some consideration on RSIC analysis. Our analysis and evaluation is sumarized in Table 2.1-1 for FR1 and Table 2.1-2 for FR2.

- Reference

[1] R4-2220244, “WF for the feasibility from BS aspect”, Samsung

[2] R4-2216239, “Evolution of receiver blocking and AGC”, Huawei, HiSilicon