3GPP TSG-RAN4 Meeting #106	R4-2301465

# Athens, Greece, 27th Feb 2023 – 3rd March 2023

Source:				Ericsson

Title:					NTN enhancements – System parameters and bands definition

Agenda item:		9.25.1.1

Document for:		Approval

# Introduction

This document discusses the system parameters and bands definition in the scope of the NTN enhancements WI ([1]).  The Way Forward [2] agreed in last RAN4#105 meeting is used as input to this contribution.

# Discussion

## Bands definition

As explained in our other contribution on regulatory aspects ([3]), we propose to define the previously pre-agreed NTN Ka-bands based on the FCC and ECC Decisions.

Like RAN4 did for TN bands, the added notes clarify the applicability of each band and its corresponding regulations.

Proposal1: Define the pre-agreed NTN Ka-bands according to the following table:

Table 1: NTN Ka-bands definition

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.7 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.7 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.7 - 20.2 GHz |
| Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. Uncoordinated NTN ESIM-L operations are not allowed in 27.8285-28.4445 GHz (UL) and 28.9485-29.4525 GHz (UL). Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25. NTN ESIM operations with GSO satellite are not allowed in 17.7-17.8 GHz (DL), 19.4-19.6 GHz (DL) and 29.1-29.25 GHz (UL). Note 3: This band is applicable for NTN VSAT operations in the USA subject to FCC 47 CFR part 25. |  |  |

## Sub-carrier spacing

For TN FR2, RAN4 specified 2 sub-carrier spacing values, 60 and 120 kHz. It would make sense to consider those values as well for the NTN Ka-bands.

Nevertheless, as further elaborated in [4], there might be some timing budget issues using both sub-carrier spacing values. Before specifying sub-carrier spacing values for the NTN Ka-bands, RAN4 should better wait for the RRM conclusions on those potential issues.

Proposal2: Wait for RRM conclusion on the feasibility of using sub-carrier spacing values 60 and 120 kHz for NTN above 10 GHz.

# Conclusion

In this contribution, we discussed the NTN Ka-bands definition and sub-carrier spacing values to be considered for those bands. We made the following proposals:

Proposal1: Define the pre-agreed NTN Ka-bands according to the following table:

| NTN operating band | UL Earth-to-Space | DL Space-to-Earth |
| --- | --- | --- |
| n5121 | 27.5 - 30.0 GHz | 17.7 - 20.2 GHz |
| n5112 | 28.35 - 30.0 GHz | 17.7 - 20.2 GHz |
| n5103 | 27.5 - 28.35 GHz | 17.7 - 20.2 GHz |
| Note 1: This band is applicable in the countries subject to CEPT ECC Decision(05)01 and ECC Decision (13)01. Uncoordinated NTN ESIM-L operations are not allowed in 27.8285-28.4445 GHz (UL) and 28.9485-29.4525 GHz (UL). Note 2: This band is applicable in the USA subject to FCC 47 CFR part 25. NTN ESIM operations with GSO satellite are not allowed in 17.7-17.8 GHz (DL), 19.4-19.6 GHz (DL) and 29.1-29.25 GHz (UL). Note 3: This band is applicable for NTN VSAT operations in the USA subject to FCC 47 CFR part 25. |  |  |

Proposal2: Wait for RRM conclusion on the feasibility of using sub-carrier spacing values 60 and 120 kHz for NTN above 10 GHz.

# Reference

- RP-222564, NR NTN (Non-Terrestrial Networks) enhancements, Thales

- R4-2220239, WF for above 10GHz SAN RF requirements, Thales, ZT

- R4-2301464, NTN enhancements – Regulatory aspects, Ericsson

- R4-2302020, RRM requirements for NR NTN enh, Ericsson